| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A Compact DCD/UCD Message to Extend the Range of RS Nodes using the AAS Relay Zone |  |
| Date Submitted | 2007-07-05 |  |
| Source(s) | Dale Branlund, Michael Webb, Matt Volpe BRN Phoenix Inc. 2500 Augustine Drive Santa Clara, CA, USA 95054 John Norin, Robert Popoli The DIRECTV Group, Inc. 2250 East Imperial Hwy El Segundo, CA 90245 | Voice: +1-408-572-9703 Fax: +1-408-351-4911 dbranlund@brnphoenix.com Voice: +1-310-964-0717 Fax: +1-310-535-5422 john.norin@directv.com |
| Re: | Call for Technical Comments and Contributions regarding IEEE Project P802.16j; See 802.16j-07/007r2 |  |
| Abstract | This contribution describes an optional method to extend the range of the RS - MR-BS link by sending a compact DCD/UCD message on the access channel in the AAS Relay Zone. |  |
| Purpose | This document provides a proposal for a new DCD/UCD message to be considered for incorporation into the 802.16j amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

A Compact DCD/UCD Message to Extend the Range of RS Nodes using the AAS Relay Zone

# Dale Branlund, Mike Webb, Matt Volpe, BRN Phoenix, Santa Clara, CA, USA;
John Norin, Robert Popoli, The DIRECTV Group, Inc., El Segundo, CA, USA

This document describes an enhanced method, applicable to RS nodes, for receiving the DCD and UCD broadcast messages.  This method is only applicable to MR-BS nodes and RS nodes capable of communicating in the optional AAS Relay Zone.  This method provides range extension for edge-of-cell RS nodes that cannot receive standard broadcast messages without the additional system gain provided by adaptive array system (AAS) processing.

## Background

An MR-BS can communicate with RS nodes at a greater range, or higher MCS level or some combination of increased range and MCS if adaptive beamforming is used.  However, edge-of-cell RS nodes may not be able to receive the DCD and UCD messages reliably since they are sent without the benefits of beamforming gain. If the DCD and UCD messages are not received, initial ranging and network entry is not possible.

Section 8.4.4.7.2.3 of IEEE 802.16j-06/026r4 describes the overall strategy for edge-of-cell RS nodes and is repeated here:

“Most AAS-RS receive broadcast messages such as the DCD and UCD in the access zone using standard allocations pointed to by standard MAP IEs while edge-of-cell RS nodes receive DCD and UCD messages in the AAS Relay Zone Access Channel via allocations pointed to by IEs contained in the R-MAP received in the access channel”.

This contribution further describes the messaging and signaling used by edge-of-cell RS nodes in AAS Relay Zone Access Channel

## Proposed Solution

The proposed solution sends a “Compact AAS DCD/UCD” message in the AAS Access Channel which is just sufficient to enable initial ranging.  The message is sent with the benefit of repetitions and frequency diversity and possibly scatter-casting or cyclic delay diversity for reliable reception.  The entire message may be transmitted in one or two frames for most TDD relay frame structures using the well known modulation QPSK rate ½ with CC FEC encoding.  High levels of data repetition or the use of scatter-casting requires a compact message to increase the probability of network entry.

Successful receipt of the Compact AAS DCD/UCD message enables initial ranging over allocations in the Access Channel with the full benefit of MR-BS beamforming.  Thereafter, the full DCD and UCD messages may be received with the benefit of MR-BS beamforming gain.

## Detailed Solution

The RS and MR-BS shall perform the following actions:

The RS processes the FCH to determine the coding and repetitions.

The RS detects the preamble marking the start of the Compressed DL-MAP and Compressed UL.  This preamble is called the Forward Link Access (FLA) preamble.

The allocation for the Compact DCD/UCD immediately follows the Compressed DL-MAP and Compressed UL-MAP.

The RS receives and processes the Compact AAS DCD/UCD message checking the CRC32 for integrity. The modulation is the well known QPSK rate ½ with CC FEC.

The RS performs initial ranging in using the Range Request and Range Response messaging received in the Access Channel.  The initial ranging access codewords are codewords 2000 - 2015 defined in section 8.4.4.7.2.3 and are selected by the base color code.  Base color code 0 selects codewords 2000-2003, Base color code 1 selects codewords 2004-2007, Base color code 2 selects codewords 2008-2011,  and Base color code 3 selects codewords 2012-2015.  Within the set, the codeword is randomly selected by the RS.

The RS requests the full DCD and UCD messages with beamforming gain using the assign access codeword as training.  The DCD message contains the revisit rate (in frames) of the Compact AAS DCD/UCD message.

The RS monitors the change count in the Access Channel and if the change count is greater than the saved change count, it processes the message and requests the full DCD and UCD messages with beamforming gain using the assign access codeword as training.

The Compressed DL-MAP and Compressed UL-MAP contents are provided in Tables 1 and 2 below for reference.

Table 1	Compressed DL-MAP

| Compressed DL-MAP() { | bits | bytes |
| --- | --- | --- |
| Compressed MAP indicator | 3 bits | . |
| UL-MAP appended | 1 bit | . |
| Reserved | 1 bit | . |
| MAP message length | 11 bits | 2 |
| PHY Synchronization Field | 32 bits | 4 |
| DCD Count | 8 bits | 1 |
| Operator ID | 8 bits | 1 |
| Sector ID | 8 bits | 1 |
| No OFDMA Symbols | 8 bits | 1 |
| DL-MAP_IE | 0 | 0 |
| } |  |  |
| Minimum MAC PDU Size --------------> | 80 | 10 |

Table 2	Compressed UL-MAP

| Compressed UL-MAP() { | bits | bytes |
| --- | --- | --- |
| UCD Count | 8 bits | 1 |
| Allocation Start Time | 32 bits | 4 |
| No OFDMA Symbols | 8 bits | 1 |
| UL-MAP_IE | 0 | 0 |
| } |  |  |
| Minimum MAC PDU Size --------------> | 48 | 6 |

The Compact AAS DCD/UCD message with MAC encapsulation is shown in Table 3.  Note its compact length compared to the standard DCD and UCD as it has been optimized for initial ranging only.

Table 3	Compact AAS DCD/UCD PDU Construction

| Compact_AAS_DCD_UCD PDU | bits | bytes |
| --- | --- | --- |
| Generic MAC Header | 48 | 6 |
| Compact_AAS_DCD_UCD_Message_Format() { |  |  |
| Mac Management Type = 98 | 8 bits | 1 |
| DCD Configuration Count | 8 bits | 1 |
| UCD Configuration Count | 8 bits | 1 |
| BS EIRP | 16 bits | 2 |
| TTG in PSs | 16 bits | 2 |
| RTG in PSs | 8 bits | 1 |
| EIRxPIR,max | 16 bits | 2 |
| Base Station ID | 48 bits | 6 |
| MAC Version | 8 bits | 1 |
| Base Station Color Code | 8 bits | 1 |
| Initial Ranging Backoff Start | 8 bits | 1 |
| Initial Ranging Backoff End | 8 bits | 1 |
| Bandwidth Request Backoff Start | 8 bits | 1 |
| Bandwidth Request Backoff End | 8 bits | 1 |
| DCD TLV encoded information | variable | 0 |
| UCD TLV encoded information | variable | 0 |
| } |  |  |
| CRC | 32 | 4 |
| Minimum MAC PDU Size --------------> |  | 32 |

## Proposed Text Changes

Replace the last row of  Table 14(.16e)/Table 38(Rev2)with the follwing:

| Type | Message name | Message description | Connection |
| --- | --- | --- | --- |
| 98 | COMP_AAS_DCD-UCD | Compact AAS DCD/UCD | Basic |
| 99-255 |  | Reserved |  |

Insert new subclause 6.3.2.3.98

A Compact AAS DCD-UCD (Table xx1) shall be transmitted periodically in the AAS Relay Zone by the BS to define the characteristics of the physical channel in support of initial ranging.

Table xx1   Compact AAS DCD/UCD Message Format

| Syntax | Size | Notes |
| --- | --- | --- |
| Compact_AAS_DCD_UCD_Message_Format() { |  |  |
| Mac Management Type = 98 | 8 bits |  |
| DCD Configuration Count | 8 bits |  |
| UCD Configuration Count | 8 bits |  |
| BS EIRP | 16 bits | DCD parameters |
| TTG | 16 bits | ¦ |
| RTG | 8 bits | ¦ |
| EIRxPIR,max | 16 bits | ¦ |
| Base Station ID | 48 bits | ¦ |
| MAC Version | 8 bits | ¦ |
| Base Station Color Code | 8 bits | V |
| Initial Ranging Backoff Start | 8 bits | UCD parameters |
| Initial Ranging Backoff End | 8 bits | ¦ |
| Bandwidth Request Backoff Start | 8 bits | ¦ |
| Bandwidth Request Backoff End | 8 bits | V |
| DCD TLV encoded information | variable | optional |
| UCD TLV encoded information | variable | optional |
| } |  |  |

A BS shall generate Compact AAS DCD-UCDs in the format shown in Table xx1, including all of the following parameters:

DCD Configuration Change Count

Incremented by one (modulo 256) by the BS whenever any of the values of this channel descriptor

change, except for the Frame Number for the OFDM PHY. If the value of this count in a subsequent

DCD remains the same, the SS can quickly decide that the remaining fields have not changed and

may be able to disregard the remainder of the message.

UCD Configuration Change Count

Incremented by one (modulo 256) by the BS whenever any of the values of this channel descriptor

change, except for the Frame Number for the OFDM PHY. If the value of this count in a subsequent

UCD remains the same, the SS can quickly decide that the remaining fields have not changed and

may be able to disregard the remainder of the message.

BS EIRP

The BS equivalent isotropic received power signed in units of 1 dBm

TTG

TTG in PSs

RTG

RTG in PSs

EIRxPIR,max

Initial Ranging maximum equivalent isotropic received power at BS Signed in units of 1 dBm.

Base Station ID

Base Station ID

MAC Version

(See 11.1.3)

Base Station Color Code

Defines codeword range within a Base Station codeword set

Initial Ranging Backoff Start

Initial backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0-15 (the  highest order bits shall be unused and set to 0) This TLV shall be used in NBR-ADV message only to represent corresponding values that appear in UCD message fields

Initial Ranging Backoff End

Final backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0-15 (the highest order bits shall be unused and set to 0) This TLV shall be used in NBR-ADV message only to represent corresponding values that appear in UCD message fields.

Bandwidth Request Backoff Start

Initial backoff window size for contention BW requests, expressed as a power of 2. Values of n range 0-15 (the highest order bits shall be unused and set to 0). This TLV shall be used in NBR-ADV message only to represent corresponding values that appear in UCD message fields.

Bandwidth Request Backoff End

Final backoff window size for contention BW requests, expressed as a power of 2. Values of n range 0-15 (the highest order bits shall be unused and set to 0). This TLV shall be used in NBR-ADV message only to represent corresponding values that appear in UCD message fields.

DCD TLV encoded information

Further DCD TLVs may be optionally provided if desired

UCD TLV encoded information

Further UCD TLVs may be optionally provided if desired

## Initial Ranging for Edge-of-Cell RS Nodes

Add a subsection 8.4.4.7.2.3.1

Initial ranging for edge-of-cell RS Nodes begins with reception of the “Compact AAS DCD/UCD” message in the AAS Access Channel which is sufficient to enable initial ranging.  The message is sent with the benefit of repetitions and frequency diversity and possibly scatter-casting or cyclic delay diversity for reliable reception.  The entire message may be transmitted in one or two frames for most TDD relay frame structures using the well known modulation QPSK rate ½ with CC FEC encoding.  High levels of data repetition or the use of scatter-casting requires a compact message to increase the probability of network entry.  The RS procedure may be defined as follows:

Process the FCH to determine the coding and repetitions.

Detect the preamble marking the start of the Compressed DL-MAP and Compressed UL in the access channel.  This preamble is called the Forward Link Access (FLA) preamble.

The allocation for the Compact DCD/UCD immediately follows the Compressed DL-MAP and Compressed UL-MAP.

Receive and process the Compact AAS DCD/UCD message checking the CRC32 for integrity. The modulation is the well known QPSK rate ½ with CC FEC.

Perform initial ranging in using the Range Request and Range Response messaging received in the Access Channel.  The initial ranging access codewords are codewords 2000 - 2015 defined in section 8.4.4.7.2.3 and are selected by the base color code.  Base color code 0 selects codewords 2000-2003, Base color code 1 selects codewords 2004-2007, Base color code 2 selects codewords 2008-2011,  and Base color code 3 selects codewords 2012-2015.  Within the set, the codeword is randomly selected.

Request the full DCD and UCD messages with beamforming gain using the assign access codeword as training.  The DCD message contains the revisit rate (in frames) of the Compact AAS DCD/UCD message.

Monitor the change count in the Access Channel and if the change count is greater than the saved change count, process the message and request the full DCD and UCD messages with beamforming gain using the assign access codeword as training.