| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Aggregating the QoS Requirements within a Tunnel |  |
| Date Submitted | 2007-07-14 |  |
| Source(s) | Yuefeng Zhou, Mike Hart, Sunil Vadgama Fujitsu Laboratories Europe | Voice: +44-20-86064802 E-mail: Yuefeng.Zhou@uk.fujitsu.com |
| Re: | Call for Technical Comments Regarding IEEE Project (IEEE 802.16j-07/013r2) |  |
| Abstract | This document clarifies how to aggregate the QoS requirements for a tunnel |  |
| Purpose | To incorporate the proposed change into the P802.16j Baseline Document (IEEE 802.16j-06/026r4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarification on Aggregating the QoS Requirements within a Tunnel

Yuefeng Zhou, Mike Hart, Sunil Vadgama

Fujitsu Laboratories of Europe Ltd

# Description

The current baseline document C80216j-06_026r4 indicates that the QoS parameters of a tunnel are an aggregate of the QoS requirements of the individual service flows within a tunnel. However, how to aggregate the QoS requirements is not clear.  This contribution clarifies how to aggregate the QoS requirements of the individual service flows for a tunnel.

# Proposed Text

6.3.14.10 Tunnel Service Flows

[Modify the last paragraph and add a table at the end of this subclause]

When a new service flow is created, use of a tunnel is optional. If use of a tunnel is specified, then the MRBS determines whether the service flow should traverse a tunnel that exists between them (if such a tunnel has been established). If the service flow is to traverse the tunnel, the MR-BS or Access RS modifies the QoS parameters of the tunnel to include QoS requirements of the service flow. The QoS parameters of both the tunnel and service flow are sent as part of the connection setup messages (DSx messages). The Access RS and MR-BS use the QoS parameters of both the individual service flow and the tunnel in performing admission control and resource reservation. The QoS parameters of a tunnel are an aggregate of the QoS requirements of the individual service flows admitted into the tunnel (Annex XX gives an example of how to aggregate QoS parameters for a tunnel) Intermediate RSs only deal with supporting the aggregate QoS requirement of the tunnel. They do not have knowledge of the requirements for individual service flows, and therefore may ignore them.

[Add a new annex]

Annex XX – QoS Parameters for a Tunnel

XX.1 Example of How to Aggregate QoS Parameters for a Tunnel

Table xxx shows an example of how to aggregate QoS parameters for a tunnel.

Table xxx —  The value of aggregated QoS parameters admitted into a tunnel

| Name | Aggregated Value |
| --- | --- |
| Traffic Priority | The highest priority among all the service flows’ traffic priority admitted into a tunnel |
| Maximum Sustained Traffic Rate | Sum of all the service flows’ maximum sustained traffic rate admitted into a tunnel |
| Maximum Traffic Burst | Min { Sum of all the service flows’ maximum traffic burst admitted into a tunnel, Maximum allowed size of traffic burst } |
| Minimum Traffic Burst | Sum of all the service flows’ the minimum traffic burst admitted into a tunnel |
| Minimum Reserved Traffic Rate | Sum of all the service flows’ minimum reserved traffic rate admitted into a tunnel |
| Maximum Latency | Minimum value among all the service flows’ maximum latency values admitted into a tunnel |
| Tolerated Jitter | Minimum value among all the service flows’ tolerated jitter values admitted into a tunnel |
| Unsolicited Grant Interval | Highest common divisor of all the service flows’ unsolicited grant interval values admitted into a tunnel |
| Unsolicited Polling Interval | Highest common divisor of all the service flows’ unsolicited polling interval values admitted into a tunnel |