| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | # R-MAC Header Format Enabling Source QoS Control |  |
| Date Submitted | 2007-07-16 |  |
| Source(s) | Hang Zhang, Peiying Zhu, Mo-Han Fong, Wen Tong, David Steer, Gamini Senarath, G.Q. Wang, Derek Yu, Israfil Bahceci, Robert Sun and Mark Naden Nortel 3500 Carling Avenue Ottawa, Ontario K2H 8E9 Kanchei (Ken) Loa, Hua-Chiang Yin,Yi-Hsueh Tsai, Shiann Tsong Sheu,Yung-Ting Lee, Youn-Tai Lee, Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan, ROC | Voice: +613-763-1315 E-mail: wentong@nortel.com Voice: +613-765-8983 E-mail: pyzhu@nortel.com Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw |
| Re: | IEEE 802.16j-07/019: “Call for Technical Comments Regarding IEEE Project 802.16j” |  |
| Abstract | In this contribution, R-MAC PDU format enabling source QoS control is proposed |  |
| Purpose | To incorporate the proposed text into the P802.16j Baseline Document (IEEE 802.16j-06/026r4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# R-MAC Sub-Header Format Enabling Tunnel QoS Control

Hang Zhang, Peiying Zhu, Mo-Han Fong, Wen Tong, David Steer, Gamini Senarath, G.Q. Wang, Derek Yu, Israfil Bahceci, Robert Sun and Mark Naden

Nortel

Kanchei (Ken) Loa, Hua-Chiang Yin,Yi-Hsueh Tsai, Shiann Tsong Sheu,Yung-Ting Lee,

Youn-Tai Lee,

Institute for Information Industry

# 1. Introduction

In this contribution, R-MAC PDU sub-header format enabling tunnel QoS control is proposed.

# 2. Proposal

In contribution C80216j_07/424, single tunnel QoS control is proposed. For enabling tunnel QoS control,

the R-MAC header shall carry scheduling instruction: transmission deadline.

In this tunnel QoS approach, The deadline frame number may appear in the R-MAC sub-header.

The presence of deadline frame number in sub-header is indicated by Deadline Frame number sub-header (DLSH) indication bit. The deadline frame number is 8-bit number defined in 802.16-2005.

In DL data forwarding using single QoS control method, the MR-BS R-MAC layer creates an R-MAC PDU with the sub-header including scheduling instruction. In UL data forwarding using single tunnel QoS control method, an access RS R-MAC layer in R-link creates an R-MAC PDU with the sub-header including the scheduling instruction.

# 3. Proposed text change

++++++++++++++++++++++ Start Text ++++++++++++++++++++++++++++++++

[Add the following section 6.3.2.1.1.1.1]

6.3.2.1.1.1.1 R-MAC deadline frame number sub-header

The R-MAC frame number sub-header is used always with payload and can be used in both DL and UL. When DLSH is set to 1, it indicates deadline frame number indication are included in the R-MAC sub-header, and the

The R-MAC deadline frame number sub-header field encoding is show in Table xxx.

Table xxx.  R-MAC deadline frame number sub-header field encodings.

| Name | Length (bits) | Description |
| --- | --- | --- |
| HT | 1 | Header type. Should be set to 0 |
| DLSH | 1 | Deadline Frame number sub-header inclusion indication. Shall be set to 1 |
| RMI | 1 | R-MAC header indication. Shall be set to 1 |
| Reserved | 7 |  |
| Priority | 3 | Index of priority defined in 802.16 |
| Length | 11 | Length of MAC PDU |
| T-CID | 16 | T-CID of the destination access RS for DL or the source RS for UL |
| HCS | 8 | Header check sequence |

[Add the following section 6.3.2.1.1.1.2]

6.3.2.1.1.1.2 R-MAC subheaders

6.3.2.1.1.1.2.1 Deadline sub-header

This sub-header presents if the Deadline SH indication in tunnel QoS R-MAC header is set to 1. If this sub-header presents, it shall be the last sub-header.

| Syntax | Size | Notes |
| --- | --- | --- |
| Deadline | 8 bits | Indicates the absolute transmission time (8 LSB of frame number) where the payload of this R-MAC PDU shall start to be transmitted to MS by an access RS in DL direction or by a first-tier RS in UL direction |

+++++++++++++ End text ++++++++++++++++++++++++++++++++++++++++++++++++