| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MS connection management by a moving RS in moving BS mode |  |
| Date Submitted | 2007-07-15 |  |
| Source(s) | Hang Zhang, Peiying Zhu, Mo-Han Fong, Wen Tong, David Steer, Gamini Senarath, G.Q. Wang, Derek Yu, Israfil Bahceci, Robert Sun and Mark Naden Nortel 3500 Carling Avenue Ottawa, Ontario K2H 8E9 | Voice: +613-763-1315 E-mail: wentong@nortel.com Voice: +613-765-8983 E-mail: pyzhu@nortel.com |
| Re: | IEEE 802.16j-07/019: “Call for Technical Comments Regarding IEEE Project 802.16j” |  |
| Abstract | We propose the connections between MR-BS and the moving RS is managed by MR-BS. The connections of MSs associated with a RS in moving BS mode are managed by the RS and informed to the MR-BS. |  |
| Purpose | To incorporate the proposed text into the P802.16j Baseline Document (IEEE 802.16j-06/026r4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MS connection management by a moving RS Type-II

Hang Zhang, Peiying Zhu, Mo-Han Fong, Wen Tong, David Steer, Gamini Senarath, G.Q. Wang, Derek Yu, Israfil Bahceci, Robert Sun and Mark Naden

Nortel

# 1. Introduction

In current baseline document, a moving RS can work in moving RS mode (Type-I) or in a moving BS mode (Type-II)  (refer to 6.3.22.4.1.1). The corresponding data forwarding protocol is described in section 1.4.3. In this contribution, the address and connection management of MS and a moving RS Type-II in moving BS mode are addressed.

2. Proposal

The key benefit of this operation mode is to reduce the complexity caused by handover of a moving RS and all of associated MSs. In order to enable this operation, the connections between MR-BS and the moving RS is managed by MR-BS. The connections of MSs associated with a RS in moving BS mode are managed by the RS and informed to the MR-BS.

2.1 Connections of a moving RS Type-II in moving BS mode

At the initialization of a RS in this mode, the management connections are established in the same way as that of a MS. In addition to the management connections, a transport connection shall be established.  This connection is used for the RS and MR-BS to relay traffic of MS between MR-BS and an access RS. The corresponding identity is denoted as T-CID and is valid for both DL and UL. This T-CID shall be assigned during registration. The traffic flow on these transport connections are aggregated traffic of active service flows of MSs associated with this RS.

2.2 Connections of MSs served by a moving RS Type-II in moving BS mode

At MS initial network entry, the CID of management connections are assigned locally by the RS in moving BS mode and informed to the MR-BS so that a MR-BS has the knowledge of coupling between MAC address and its local basic CID of a MS accessed to a RS in moving BS mode.

The CID of a transport connection of a MS is assigned by the RS at the establishment of a service flow of the MS and is informed to MR-BS so that a MR-BS has the knowledge of coupling between a service flow and local CID assigned by the RS.

3 Relaying of Messages of MSs Served by moving RS Type-II

When a RS operates on this moving BS mode, the connections between MR-BS and the moving RS is managed by MR-BS. The connections of MSs associated with a moving RS Type-II in moving BS mode are managed by the RS and informed to the MR-BS. The authentication/authorization of a MS is managed by the network with the RS as an authenticator. The RS holds the MS’s security information such as AK, TEK and etc. The RS in this mode can process MS messages locally since the RS can perform the message authentication. Most of MS unicast messages carry contents that only involve local operation, such as ARQ related, channel measurement related, MAC state related and etc. The contents in some other MS unicast messages, however, need to be exchanged between MR-BS and MS, such as REG-REQ/RSP, PKM-REQ/RSP, DSX-REQ/RSP, MOB-XXHO-REQ/RSP and etc.

For relaying the contents of those messages, a new RS management message, called as RS_MSG_Relay-REQ/RSP is introduced to carry aggregated contents from one or more management messages of one or more MSs from the same type of management connections (basic or primary connection)

The RS shall create such message in the following way:

- For UL, after receives MS management message(s), RS shall authenticates the sender(s). If the contents need to be forwarded to MR-BS, the RS shall extract the content of available messages and may add some additional TLVs for control purpose and finally add CMAC TVL using the RS’s security material.

- For DL, MR-BS will do the similar thing. After an RS receives such a message, the RS shall extract the contents, create corresponding types of MAC management message with CMAC using MS’s security material.

This message can also be used by other type of access RS to aggregate multiple management messages before authentication performed.

The benefit of above is the low authentication overhead by aggregating the contents of multiple management messages (messages aggregation is possible in scenarios like handover of MSs from/to an RS) and using single CMAH of the RS.

3. Proposed text change

[Add the section 6.3.1.3.2 as followings]

+++++++++++++ Start text +++++++++++++++++++++++++++++++++++++++++++++++++

6.3.1.3.2 Connection management of moving RS Type-II in moving BS mode (MBS) and MS served by moving RS Type-II in MBS mode

The connections between MR-BS and a moving RS Type-II in moving BS mode  is managed by MR-BS. The connections of MSs associated with a RS in this mode are managed by the RS and informed to the MR-BS.

6.3.1.3. 2.1 Connections of a moving RS Type-II in moving BS mode

At the initialization of a RS in this mode, the management connections are established in the same way as that of a MS. In addition to the management connections, a transport connection shall be established.  This connection is used for the RS and MR-BS to relay traffic of MS between MR-BS and an access RS. The corresponding identity is denoted as FT-CID (Forwarding Tunnel CID) and is valid for both DL and UL. This FT-CID shall be assigned during registration. The traffic flow on these transport connections are aggregated traffic of active service flows of MSs associated with this RS.

6.3.1.3. 2.2 Connections of MSs served by a moving RS Type-II in moving BS mode

At MS initial network entry, the CID of management connections are assigned locally by the moving RS Type-II in moving BS mode and informed to the MR-BS so that a MR-BS has the knowledge of coupling between MAC address and its local basic CID of a MS accessed to a moving RS Type-II in moving BS mode.

The CID of a transport connection of a MS is assigned by the RS at the establishment of a service flow of the MS and is informed to MR-BS so that a MR-BS has the knowledge of coupling between a service flow and local CID assigned by the RS.

+++++++++++++ End text +++++++++++++++++++++++++++++++++++++++++++++++++

[Insert the following to the end of 6.3.2.3.8]

+++++++++++++ Start text +++++++++++++++++++++++++++++++++++++++++++++++++

The REG-RSP may include the following TLV:

DL/UL Forwarding Tunnel connection  CID

The DL Forwarding connection CID assigned by a MR-BS to a RS for forwarding of DL messages and service traffic of MSs serviced by a RS.

UL Forwarding Tunnel connection  CID

The UL Forwarding connection CID assigned by a MR-BS to a RS for forwarding of UL messages and service traffic of MSs serviced by a RS.

+++++++++++++ End text +++++++++++++++++++++++++++++++++++++++++++++++++

[Insert the following section 11.7.30 Forwarding tunnel connection CID]

+++++++++++++ Start text +++++++++++++++++++++++++++++++++++++++++++++++++

11.7.30 Forwarding Tunnel Connection CID

This TLV is used to indicate the assigned CID of forwarding tunnel connections by MR-BS to a RS.

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Forwarding Tunnel connection CIDs | TBD | 2 | Forwarding Tunnel connection CID | REG-RSP |

+++++++++++++ End text ++++++++++++++++++++++++++++++++++++++++++++++++

++++++++++++++++++++++ Start Text ++++++++++++++++++++++++++++++++

[Modify the last row in Table 38 as following]

| Type | Message name | Message description | Connection |
| --- | --- | --- | --- |
| 98-255 | RS_MSG_Relay-REQ | - | Basic/primary connection Depending on the type of relayed message |
| 99 | RS_MSG_Relay-RSP |  | Basic/primary connection Depending on the type of relayed message |
| 100-255 | reserved |  |  |

[Add the following section 6.3.2.3.91 as indicated]

6.3.2.3.91 MS message relay request (RS_MSG_Relay-REQ)

This message is used by MR-BS to forward contents of MS DL unicast management messages to an access moving RS Type-II in moving BS mode. This message can aggregate contents from multiple MS management messages to be sent on MSs’ basic connections or multiple MS management messages to be sent on MSs’ primary connections.

The message is also used by an access RS to relay contents of MS UL unicast message to MR-BS. This message can aggregate contents of multiple MS management messages received on MSs’ basic connections or multiple MS management messages received on MSs’ primary connections. The format of RS_MSG_Relay-REQ is shown in Table XXX.

Table XXX. RS_MSG_Relay request message format.

| Syntax | Size | Notes |
| --- | --- | --- |
| RS_MSG_Relay request format { |  |  |
| Management Message Type = 98 | 8 bits |  |
| Transaction ID | 8 bits | Transaction ID of this message |
| Number of relayed messages | 4 bits | Indicates the number of messages whose contents are includes |
| for ( i = 0; i < Number of relayed messages; i ++) { |  |  |
| L-CID | 16 bits | Basic CID of management connection of MS |
| Message type | 8 bits | Type of message |
| Length | 11 bits | Indicates the length of Message contents in bytes |
| Message contents | variable | The fields in the message relayed and TLVs (not including MS authentication TLV) |
| RS specific TLV | Variable | RS specific TLV |
| } |  |  |
| Authentication TLV | Variable | CMAC/HMAC TLV |
| } |  |  |

[Add the following section 6.3.2.3.92 as indicated]

6.3.2.3.91 MS message relay request (RS_MSG_Relay-RSP

This message is used by MR-BS to response RS_MSG_Relay-REQ message sent by an access in moving BS mode. This message includes contents of MS DL unicast management messages which is to be used for the access RS to create MS management message. The message is also used by an access RS as a response to RS_MSG_Relay-REQ sent by MR-BS. The format of RS_MSG_Relay-RSP is shown in Table XXX.

Table XXX. RS_MSG_Relay response message format.

| Syntax | Size | Notes |
| --- | --- | --- |
| RS_MSG_Relay response format { |  |  |
| Management Message Type = 99 |  |  |
| Transaction ID | 8 bits | Transaction ID in the corresponding request message |
| Number of relayed of messages | 4 bits | Indicates the number of messages whose contents are includes |
| for ( i = 0; i < Number of relayed messages; i ++) { |  |  |
| L-CID | 16 bits | Basic CID of management connection of MS |
| Message type | 8 bits | Type of message |
| Length | 11 bits | Indicates the length of Message contents in bytes |
| Message contents | variable | The fields in the message relayed and TLVs (not including MS authentication TLV) |
| RS specific TLV | Variable | RS specific TLV |
| } |  |  |
| Authentication TLV | Variable | CMAC/HMAC |
| } |  |  |

+++++++++++++ end text +++++++++++++++++++++++++++++++++++++++++++++++++