| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Transport connection establishment of a MS served by a RS in moving BS mode |  |
| Date Submitted | 2007-07-15 |  |
| Source(s) | Hang Zhang, Peiying Zhu, Mo-Han Fong, Wen Tong, David Steer, Gamini Senarath, G.Q. Wang, Derek Yu, Israfil Bahceci, Robert Sun and Mark Naden Nortel 3500 Carling Avenue Ottawa, Ontario K2H 8E9 | Voice: +613-763-1315 E-mail: wentong@nortel.com Voice: +613-765-8983 E-mail: pyzhu@nortel.com |
| Re: | IEEE 802.16j-07/019: “Call for Technical Comments Regarding IEEE Project 802.16j” |  |
| Abstract | For a RS operates in moving BS mode, the service flow of a MS is managed by MR-BS and service flow ID (SFID) is assigned by a MR-BS. However, the CID of corresponding transport connection is assigned by the MS’s serving RS. |  |
| Purpose | To incorporate the proposed text into the P802.16j Baseline Document (IEEE 802.16j-06/026r4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Transport connection establishment of a MS served by a moving RS Type-II

Hang Zhang, Peiying Zhu, Mo-Han Fong, Wen Tong, David Steer, Gamini Senarath, G.Q. Wang, Derek Yu, Israfil Bahceci, Robert Sun and Mark Naden

Nortel

# 1. Introduction

In current baseline document, a RS can work in moving RS mode or in a moving BS mode (Type-II) (refer to 6.3.22.4.1.1). The corresponding data forwarding protocol is described in section 1.4.3. In this contribution, the service flow setup and transport connection assignment is proposed.

2. Proposal

When a moving RS operates in Type-II moving BS mode, the service flow of a MS is managed by MR-BS and service flow ID (SFID) is assigned by a MR-BS. However, the CID of corresponding transport connection is assigned by the MS’s serving RS.

The procedure of service flow set up initiated by MR-BS is shown in Figure 1.

Figure1. Transport connection setup via a moving RS Type-II in MBS mode (MR-BS initiated)

For MR-BS initiated service flow setup, the MR-BS sends the contents of DSA-REQ message using RS_MSG_Relay-REQ message  over management connection of the RS( IEEE C802.16j-07/428r1 ). After the RS receives this message, the RS shall assign a local CID (L_CID) to the transport connection to be established for this service flow and creates binding between this SFID and this L_CID. The RS shall then send the DSA-REQ message to corresponding MS with the assigned local CID. After RS receives the DSA-RSP message from the MS, the RS shall relay the contents of this message as well as the assigned local CID TLV using RS_MSG_Relay-RSP message to MR-BS over its management connection.

After this MR-BS receives this RS_MSG_Relay-RSP message, the MR-BS creates a binding between SFID and local CID assigned by this RS.

For MS initiated service flow setup (refer to Figure 2), after a RS receives DSA-REQ from a MS, the RS shall assign a local CID to this potential UL service flow and relay the contents from the received DSA-REQ message as well as the local assigned CID to the MR-BS. The binding between the UL service flow and the local CID is established by MR-BS. The MR-BS shall use RS_MSG_Relay RSP message to carry the contents of DSA-RSP message and send this message the RS. The RS then creates the DSA-RSP message to the MS. The RS crates a binding between the SFID and the local CID.

Figure 2. Transport connection setup via a moving RS Type-II in moving BS mode (MS initiated service set up).

3. Proposed text change

[Add the section 6.3.14.9.6 as followings]

+++++++++++++ Start text ++++++++++++++++++++++++++++++++++++++++++++++++

6.3.14.9.6 Service flow management through a moving RS Type-II in moving BS mode

When a RS operates in moving BS mode, the service flow of a MS is managed by MR-BS and service flow ID (SFID) is assigned by a MR-BS. However, the CID of corresponding transport connection is assigned by the MS’s serving RS.

For MR-BS initiated service flow setup, the MR-BS sends the contents of DSA-REQ message using RS_MSG_Relay-REQ message  over management connection of the RS. After the RS receives this message, the RS shall assign a local CID (L_CID) to the transport connection to be established for this service flow and creates binding between this SFID and this L_CID. The RS shall then send the DSA-REQ message to corresponding MS with the assigned local CID. After RS receives the DSA-RSP message from the MS, the RS shall relay the contents of this message as well as the assigned local CID TLV using RS_MSG_Relay-RSP message to MR-BS over its management connection.

After this MR-BS receives this RS_MSG_Relay-RSP message, the MR-BS creates a binding between SFID and local CID assigned by this RS.

For MS initiated service flow setup (refer to Figure 2), after a RS receives DSA-REQ from a MS, the RS shall assign a local CID to this potential UL service flow and relay the contents from the received DSA-REQ message as well as the local assigned CID to the MR-BS. The binding between the UL service flow and the local CID is established by MR-BS. The MR-BS shall use RS_MSG_Relay RSP message to carry the contents of DSA-RSP message and send this message the RS. The RS then creates the DSA-RSP message to the MS. The RS crates a binding between the SFID and the local CID.

+++++++++++++ End text ++++++++++++++++++++++++++++++++++++++++++++++++

[Insert the following to the end of 11.27 RS_MSG_Relay-REQ/RSP message encodings]

+++++++++++++ End text ++++++++++++++++++++++++++++++++++++++++++++++++

11.27 RS_MSG_Relay-REQ/RSP message encodings

The RS-MSG_Relay-REQ/RSP message encodings are shown in Table XXX.

Table XXX. RS-MSG_Relay-REQ/RSP message encodings

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Locally assigned transport connection CIDs | 1 | 2 | Locally assigned transport connection CID | RS_MSG_Relay-REQ/RSP |

11.27.1 Locally assigned transport connection CID

This TLV is used for an access moving RS Type-II in moving BS mode  to inform MR-BS its locally assigned transport connection CID to a MS.

+++++++++++++ End text ++++++++++++++++++++++++++++++++++++++++++++++++