| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Tunnel Convergence sub-layer function of RS in moving BS mode |  |
| Date Submitted | 2007-07-15 |  |
| Source(s) | Hang Zhang, Peiying Zhu, Mo-Han Fong, Wen Tong, David Steer, Gamini Senarath, G.Q. Wang, Derek Yu, Israfil Bahceci, Robert Sun and Mark Naden Nortel 3500 Carling Avenue Ottawa, Ontario K2H 8E9 | Voice: +613-763-1315 E-mail: wentong@nortel.com Voice: +613-765-8983 E-mail: pyzhu@nortel.com |
| Re: | IEEE 802.16j-07/019: “Call for Technical Comments Regarding IEEE Project 802.16j” |  |
| Abstract | In this contribution, the required functions in an access RS in moving BS mode and the required functions in a MR-BS supporting such a RS are addressed. |  |
| Purpose | To incorporate the proposed text into the P802.16j Baseline Document (IEEE 802.16j-06/026r4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Tunnel Convergence sub-layer function of moving RS Type-II in moving BS mode

Hang Zhang, Peiying Zhu, Mo-Han Fong, Wen Tong,  David Steer, Gamini Senarath, G.Q. Wang, Derek Yu, Israfil Bahceci, Robert Sun and Mark Naden

Nortel

# 1 Introduction

In current baseline document, a moving RS can work in moving RS mode or in a moving BS mode (refer to 6.3.22.4.1.1). The corresponding data forwarding protocol is described in section 1.4.3, where the access RS needs to implement a simple convergence sub-layer function. In this contribution, the required functions in an access RS in moving BS mode and the required functions in a MR-BS supporting such a RS are addressed.

# 2 Proposal

The key benefits of this operation mode of moving RS are to reduce the complexity caused by handover of a moving RS and all of associated MSs, to reduce encryption overhead of MS traffic and message forwarding and to eliminate the unnecessary MS unicast messages forwarding (RS can locally process most of unicast message). In order to enable this operation, the connections between MR-BS and the moving RS is managed by MR-BS. The connections of MSs associated with a RS moving RS Type-II in BS mode are managed by the RS and informed to the MR-BS.

2.1 MR-BS CS sub-layer function

When a MR-BS works with an access moving RS Type-II in BS mode, at each service flow setup, MR-BS establishes a binding between the SFID, L-CID and forwarding connection identity FT-CID. The upper layer header suppression, if implemented, is performed on an end-to-end basis.

For DL traffic forwarding, for each of received upper layer packets, the classification in CS on the R-link results in an output {FT-CID, L-CID}. The tuple {SDU, FT-CID and L-CID} shall be then provided to CPS sub-layer through the SAP. For UL, when a short-version of MAC PDU is received by the CPS sub-layer in R-link of MR-BS, the CPS sub-layer shall send tuple {SDU, FT-CID, L-CID} to the CS sub-layer. The CS sub-layer uses FT-CID and L-CID to perform the un-suppression. The resulting packet then is delivered to the upper layer.

2.2 RS CS cub-layer function

For DL traffic forward, when a short-version MAC PDU is received by CPS sub-layer on R-link of an access RS in moving BS mode, the CPS sub-layer on R-link shall send the tuple {SDU, L-CID} to the CS sub-layer. CS sub-layer shall simply provide the tuple to the CS sub-layer on access like where the SDU is delivered to CPS sub-layer on access link of the RS with the CID identified by L-CID. The CPS sub-layer then creates MAC PDU to be delivered to MS.

For UL data forwarding, when a MAC PUD is received on access link of an access moving RS Type-II in BS mode, the MAC PDU is delivered to the CS sub-layer with the CID. The SDU and CID are simply sent to the CPS sub-layer on R-link of this RS. The CPS sub-layer then creates a short-version of MAC PDU and sends the short-version MPDU to R-MAC sub-layer for forwarding purpose.

# 3. Proposed text change

[Add the following section 5.3 CS functions supporting access RS in moving BS mode]

+++++++++++++ Start text +++++++++++++++++++++++++++++++++++++++++++++++++

5.2.3 CS function supporting access moving RS Type-II in BS mode

5.2.3.1. MR-BS CS functions supporting access moving RS Type-II in BS mode

When a MR-BS works with an access moving RS Type-II in BS mode, at each service flow setup, MR-BS establishes a binding between the SFID, L-CID and forwarding connection identity FT-CID. The upper layer header suppression, if implemented, is performed on an end-to-end basis.

For DL traffic forwarding, for each of received upper layer packets, the classification in CS on the R-link results in an output {FT-CID, L-CID}. The tuple {SDU, FT-CID and L-CID} shall be then provided to CPS sub-layer through the SAP. For UL, when a short-version of MAC PDU is received by the CPS sub-layer in R-link of MR-BS, the CPS sub-layer shall send tuple {SDU, FT-CID, L-CID} to the CS sub-layer. The CS sub-layer uses FT-CID and L-CID to perform the un-suppression. The resulting packet then is delivered to the upper layer.

5.2.3.2 CS functions of access moving RS Type-II in BS mode

For DL traffic forward, when a short-version MAC PDU is received by CPS sub-layer on R-link of an access moving RS Type-II in BS mode, the CPS sub-layer on R-link shall send the tuple {SDU, L-CID} to the CS sub-layer. CS sub-layer shall simply provide the tuple to the CS sub-layer on access like where the SDU is delivered to CPS sub-layer on access link of the RS with the CID identified by L-CID. The CPS sub-layer then creates MAC PDU to be delivered to MS.

For UL data forwarding, when a MAC PUD is received on access link of an access moving RS Type-II in BS mode, the MAC PDU is delivered to the CS sub-layer with the CID. The SDU and CID are simply sent to the CPS sub-layer on R-link of this RS. The CPS sub-layer then creates a short-version of MAC PDU and sends the short-version MPDU to R-MAC sub-layer for forwarding purpose.

+++++++++++++ end text +++++++++++++++++++++++++++++++++++++++++++++++++