| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes for Downlink Flow Control |  |
| Date Submitted | 2007-07-05 |  |
| Source(s) | Jaroslaw Sydir Intel Corporation RNB-4-135 2200 Mission College Blvd. Santa Clara, CA, 95054 | Voice: 408 765 2215 E-mail: jerry.sydir@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16j-07/019 Call for Technical Comments Regarding IEEE Project 802.16j |  |
| Abstract | Proposes the definition of a flow control protocol for downlink traffic. This protocol is used the the RS to indicate to the MR-BS whether it can accept DL traffic when the distributed scheduling model is employed. |  |
| Purpose | This is a supplement to a comment entered into commentary. The purpose if for inclusion into C802.16j-06/026 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text Changes for Removal of Moving BS Mode

Jaroslaw Sydir

Intel Corporation

# Proposal

C802.16j/026r4 does not define a flow control protocol. When a relay network operates using the distributed scheduling model, RSs need a mechanism to indicate to the MR-BS their ability to accept DL traffic. An RS receives DL data from the MR-BS. In the distributed scheduling model, the MR-BS schedules these transmissions without regard for channel conditions on the access link.  Temporary reductions in the capacity of the access link may cause the RS to accumulate DL packets destined for downstream RSs or MSs. In this situation the RS needs to indicate to the MR-BS that it is temporarily not able to accept DL traffic. When the backlog of packets has been cleared, the RS the indicates to the MR-BS that it is able to accept more packets.

The proposed remedy is to define a flow control protocol which allows the RS to signal to the MR-BS its ability to accept DL traffic. The proposed text suggested below is a placeholder for a detailed description of this protocol.

# Text Proposal

------------------- Proposed Text -----------------------------------------

[Insert new subclause 6.3.30 at the end of subclause 6.3]

6.3.30 RS Downlink Flow Control

When distributed scheduling is employed in an MR network, the RS shall signal the MR-BS its ability to accept downlink traffic on transport connections destined to downstream RSs or associated MSs. When the RS indicates that it cannot accept DL traffic, the MR-BS shall not schedule DL transmissions to that RS until the RS indicates that it can again accept DL traffic. The RS must always be able to receive DL data on management connections.

------------------- End Proposed Text -----------------------------------------