| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes for Downlink Flow Control |  |
| Date Submitted | 2007-07-18 |  |
| Source(s) | Jaroslaw Sydir Kerstin Johnsson Intel Corporation RNB-4-135 2200 Mission College Blvd. Santa Clara, CA, 95054 | Voice: 408 765 2215 E-mail: jerry.sydir@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16j-07/019 Call for Technical Comments Regarding IEEE Project 802.16j |  |
| Abstract | Proposes the definition of a flow control protocol for downlink traffic. This protocol is used the the RS to indicate to the MR-BS whether it can accept DL traffic when the distributed scheduling model is employed. |  |
| Purpose | This is a supplement to a comment entered into commentary. The purpose if for inclusion into C802.16j-06/026 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text Changes for Downlink Flow Control

Jaroslaw Sydir

Intel Corporation

# Proposal

C802.16j/026r4 does not define a flow control protocol. When a relay network operates using the distributed scheduling model, RSs need a mechanism to indicate to the MR-BS their ability to accept DL traffic. An RS receives DL data from the MR-BS. In the distributed scheduling model, the MR-BS schedules these transmissions without regard for channel conditions on links beyond its control.  Temporary reductions in the capacity of the access or relay links controlled by an RS may cause it to accumulate DL packets destined for downstream RSs or MSs. In this situation the RS needs to indicate to the MR-BS that it is temporarily not able to accept DL traffic. When the backlog of packets has been cleared, the RS then indicates to the MR-BS that it is able to accept more packets. It should be noted that this situation occurs on a faster timescale than connection setup/change. It occurs even after the RS has performed proper admission control and resource reservation, so the flow control protocol proposed here is not related to the connection signaling (DS*) messages.

The proposed remedy is to define a flow control protocol which allows the RS to signal to the MR-BS its ability to accept DL traffic. The proposed text suggested below is a placeholder for a detailed description of this protocol.

# Text Proposal

------------------- Proposed Text -----------------------------------------

[Change table 19a in subclause 6.3.2.1.2.2.2 as indicated]

| Extended Type field | MAC header type | Reference Figure | Reference Table |
| --- | --- | --- | --- |
| 0 | RS BR header | XX | XX |
| 1 | RS UL_DCH request header |  |  |
| 2 | Acknowledgement Header |  |  |
| 3 | HARQ RS error header |  |  |
| 4 | DL Flow Control Header |  |  |
| 45-7 | Reserved |  |  |

[Insert new subclause 6.3.2.1.2.2.2.5]

The DL Flow Control Header is used to perform DL flow control between an RS and its superordinate RS or MR-BS. This header is sent by an RS to its superordinate RS or MR-BS to report the number of bytes which the RS can receive in the DL direction. See subclause 6.3.30 for the usage of this message.

Figure 35f – DL Flow Control Header

Table 19f – Description of Fields in DL Flow Control Header

| Name | Length (bits) | Description |
| --- | --- | --- |
| Reserved | 2 | Set to 0 |
| Next Update | 8 | Indicates the number of frames into the future that the subordinate RS expects to elapse before it needs to send the next DL credit update. This field is set of 0 when the RS chooses not to make a prediction. |
| Credit | 16 | Indicates the number of bytes of DL traffic that the superordinate RS or MR-BS can send to the subordinate RS. |
| Reserved | 8 | Set to 0 |
| HCS | 8 | Header Check Sequence |

[Change contents of table 13c in subclause 6.3.2.2.7 as indicated]

| 5 | DL_Credit_REP Subheader | 2 bytes | See section 6.3.2.2.7.x |
| --- | --- | --- | --- |
| 6 | DL_Credit_REP_Poll Subheader | 3 bytes | See section 6.3.2.2.7.x |
| 57-127 | Reserved |  |  |

[Insert new subclause 6.3.2.2.7.x at the end of subclause 6.3.2.2.7]

6.3.2.2.7.x DL_Credit_REP Subheader

This subheader is sent by the subordinate RS to the superordinate RS or MR_BS to report the number of bytes which the RS can receive in the DL direction. See subclause 6.3.30 for the usage of this message.

Table xxx – DL_Credit_REP Subheader

| Name | Size | Description |
| --- | --- | --- |
| Credit | 16 | Indicates the number of bytes of DL traffic that the superordinate RS or MR-BS can send to the subordinate RS. |

[Insert new subclause 6.3.2.2.7.x at the end of subclause 6.3.2.2.7]

6.3.2.2.7.x DL_Credit_REP_Poll Subheader

This subheader is sent by the subordinate RS to the superordinate RS or MR_BS to report the number of bytes which the RS can receive in the DL direction and the number of frames into the future when it would like to send the next credit update. See subclause 6.3.30 for the usage of this message.

Table xxx – DL_Credit_REP Subheader

| Name | Size | Description |
| --- | --- | --- |
| Credit | 16 | Indicates the number of bytes of DL traffic that the superordinate RS or MR-BS can send to the subordinate RS. |
| Next Update | 8 | Indicates the number of frames into the future that the subordinate RS expects to elapse before it needs to send the next DL Credit update. |

[Change table 183f in subclause 6.3.3.67 as indicated]

| Syntax | Size | Note |
| --- | --- | --- |
| Configuration_para_type | 8 bits | b5=1: R-amble configuration is included b6: DL flow control 0 – DL flow control is disabled 1 – DL flow control is enabled b6 – b7: Reserved |
| If (b6 of Configuration_para_type == 1) { |  |  |
| Min_credit_update_interval | 8 bits | Minimum number of frames between credit updates. |
| Max_credit_update_interval | 8 bits | Maximum number of frames between credit updates. |
| } |  |  |

[Add the following text at the end of the list of parameter descriptions in subclause 6.3.3.67]

Min_credit_update_interval

This parameter specifies the minimum number of frames that must elapse between credit updates. The RS shall not send credit updates more often than specified by this parameter.

Max_credit_update_interval

This parameter specifies the maximum number of frames that may elapse between credit updates. The RS must send credit updates at least as often as specified by this parameter.

[Insert new subclause 6.3.6.7.1.3 at the end of subclause 6.3.6.7.1]

6.3.6.7.1.3  Downlink Flow Control In Distributed Mode With Non-Transparent RS

When distributed scheduling is employed in an MR network, the RS shall signal its superordinate station (MR-BS or RS) its ability to accept downlink traffic on transport connections destined to downstream RSs or MSs. When the RS indicates that it cannot accept DL traffic, the superordinate station shall not schedule DL transmissions to that RS until the RS indicates that it can again accept DL traffic. The RS shall always be able to receive DL data on management connections.

A credit based flow control scheme is used. RSs are directed by the MR-BS to participate in the DL flow control protocol. When configured to perform flow control, an RS shall periodically send credit updates to its superordinate station indicating the number of bytes that it is able to receive on the downlink. The minimum and maximum rate at which an RS shall send credit updates to its superordinate station is specified in the RS_Config-REQ message.

The RS sends credit updates to its superordinate station using the DL_Flow_Control Header, the DL_Credit_REP extended subheader, or the DL_Credit_REP_Poll extended subheader. When using the DL_Flow_Control header, the RS specifies the number of bytes that it is able to receive from the superordinate station in the Credit field of the DL_Flow Control header. The RS may also specify the number of frames into the future when it would like a UL allocation in order to send the next credit update. It does this by specifying the number in the Next Update field of the DL_Flow_Control header. If the RS does not wish to request a UL allocation for the next credit update, it shall enter a value of 0 in the Next Update field of the DL_Flow_Control header.

When using the DL_Credit_REP extended subheader to send a credit update, the RS specifies the number of bytes that it is able to receive from the superordinate station in the Credit field of the subheader. The RS uses this subheader when it does not need to request a UL allocation for the next credit update.

When using the DL_Credit_REP extended subheader to send a credit update, the RS specifies the number of bytes that it is able to receive from the superordinate station in the Credit field of the subheader. When using this subheader, the RS specifies the number of frames into the future when it would like an UL allocation for the next credit update.in the Next Update field of the DL_Credit_REP_Poll extended subheader.

------------------- End Proposed Text -----------------------------------------