| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Amendment to PM Confirmation Code in Explicit Path Management Messages |  |
| Date Submitted | 2007-07-18 |  |
| Source(s) | Haihong Zheng, Yousuf Saifullah, Shashikant Maheshwari Nokia Siemens Networks 6000 Connection Drive, Irving, TX 75019 USA | Voice: 972-894-5000 Email: Haihong.Zheng@nsn.com |
| Re: | IEEE 802.16j-06/027: “Call for Technical Proposals regarding IEEE Project P802.16j” |  |
| Abstract | This proposal clarifies the service flow management in MR with distributed RS. |  |
| Purpose | Discuss and adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein.. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Amendment to Explicit Path Management Messages

# 1. Introduction

A different message structure for DSA/DSC/DSD-RSP is defined for explicit path management because a PM confirmation code was introduced. However, the PM confirmation code can reuse confirmation code defined in .16d/e. This contribution proposes to replace PM confirmation code with confirmation code and remove the related text.

# 2. Specific Text Change

6.3.2.3.11  DSA-RSP message

[Change the text in 6.3.2.3.11 as following:]

In multihop relay network, a DSA-RSP is also sent by a RS to confirm the path management operation

requested in the correspondent DSA-REQ. The access RS on the last hop on a specific path should generate

the DSA-RSP in the form shown in Table 81. When a RS receives a DSA-RSP, it shall update the confir

mation code and generate a DSA-RSP in the form shown in Table 81 and sends it to the previous RS on the

path.

Table 81a – DSA-RSP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DSA-RSP() { |  |  |
| Management Message Type = 12 | 8 bits |  |
| Transaction ID | 16bits |  |
| PM Confirmation Code | 8 bits |  |
| TLV Encoded Information | Variable | TLV specific |
| } |  |  |

Parameters shall be as follows:

Transaction ID

Transaction ID from corresponding DSA-REQ

PM Confirmation Code (see 11.21.8)

The appropriate Path Management Confirmation Code for the entire correspondent DSA-REQ.

6.3.2.3.14 DSC-RSP message

[Change the text in 6.3.2.3.14 as following:]

In multihop relay network, a DSC-RSP is also sent by a RS to confirm the path management operation

requested in the correspondent DSC-REQ. The access RS on the last hop on a specific path should generate

the DSC-RSP in the form shown in Table 84. When a RS receives a DSC-RSP, it shall update the confirmation

code and generate a DSC-RSP in the form shown in Table 84 and sends it to the previous RS on the

path.

Table 84a – DSC-RSP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DSC-RSP() { |  |  |
| Management Message Type = 15 | 8 bits |  |
| Transaction ID | 16bits |  |
| PM Confirmation Code | 8 bits |  |
| TLV Encoded Information | Variable | TLV specific |
| } |  |  |

Parameters shall be as follows:

Transaction ID

Transaction ID from corresponding DSC-REQ

PM Confirmation Code (see 11.21.8)

The appropriate Path Management Confirmation Code for the entire correspondent DSC-REQ.

6.3.2.3.17 DSD-RSP message

[Change the text in 6.3.2.3.17 as following:]

In multihop relay network, a DSD-RSP is also sent by a RS to confirm the path management operation

requested in the correspondent DSD-REQ. The access RS on the last hop on a specific path should generate

the DSD-RSP in the form shown in Table 87. When a RS receives a DSD-RSP, it shall update the confirmation

code and generate a DSD-RSP in the form shown in Table 87 and sends it to the previous RS on the

path.

Table 87a – DSD-RSP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DSD-RSP() { |  |  |
| Management Message Type = 18 | 8 bits |  |
| Transaction ID | 16bits |  |
| PM Confirmation Code | 8 bits |  |
| TLV Encoded Information | Variable | TLV specific |
| } |  |  |

Parameters shall be as follows:

Transaction ID

Transaction ID from corresponding DSD-REQ

PM Confirmation Code (see 11.21.8)

The appropriate Path Management Confirmation Code for the entire correspondent DSD-REQ.

11.13 Service flow management encodings

[Add the following text before subclause 11.13.1:]

The CC = “Ok/success” and CC = “reject-other” can also be used as the confirmation code to confirm the path management operation as defined in subclause 6.3.2.3.11, 6.3.2.3.14 and 6.3.2.3.17.

[Remove subclause11.22.8]

11.22.8 PM-Confirmation-Code TLV

TBD.