| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Uplink HARQ for Transparent RS |  |
| Date Submitted | 2007-07-19 |  |
| Source(s) | Kanchei (Ken) Loa, Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan Gamini Senarath Nortel Maheshwari Shashikant NSN | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@nmi.iii.org.tw |
| Re: | IEEE 802.16j-07/019: “Call for Technical Comments Regarding IEEE Project 802.16j” |  |
| Abstract | This contribution proposes “Downlink HARQ for Transparent RS” |  |
| Purpose | Text proposal for 802.16j Baseline Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Uplink HARQ for Transparent RS

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee,

Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry (III)

Gamini Senarath

Nortel

Maheshwari Shashikant

NSN

Introduction

The contribution is response to comments 223, 231, 232, 239 in 80216j-07_020.cmt.

.

Text Proposal

[Change the text in sub-clause 6.3.17.4.3]

6.3.17.4.3 UL HARQ for transparent RS

When the MR-BS chooses to receive an HARQ sub-burst from the MS through the RS, it shall inform the RS and allocate UL transmission for the RS to relay the burst to the MR-BS. If an RS receives a HARQ sub-burst from an MS correctly, the RS saves it for any possible retransmission, and sends an ACK signal to the MR-BS using the ACK channel prepared by MR-BS. Then the MR-BS allocates bandwidth for the RS to relay the HARQ sub-burst. If the MR-BS receives ACK signal from the RS, it sends an ACK on HARQ ACK Bitmap IE to the MS directly. If the MR-BS cannot decode the sub-burst relayed by the RS correctly, the MR-BS sends a NAK to the RS and allocates bandwidth for the RS to retransmit the saved sub-burst. If an RS fails to receive the HARQ sub-burst from MS correctly, the RS sends a NAK signal to the MR-BS and the MR-BS sends a  NAK to the MS. Subsequently, the MR-BS may request the MS to retransmit the HARQ sub-burst.

It is also possible for the MR-BS to receive the first transmission from an MS directly. In such a case, the MR-BS informs the RS using the Compact_UL-MAP MONITOR IE that it needs to monitor the transmission. The RS, having the information on uplink resource allocations sent in the UL-MAP for the MS, monitors the HARQ sub-burst transmission sent by the MS to the MR-BS directly and attempts to decode it. When the RS receives the HARQ sub burst correctly, the RS saves it for a possible retransmission and sends an ACK to the MR-BS. On receiving the ACK from RS, MR-BS sends an ACK on HARQ ACK Bitmap IE to the MS directly. If the burst is received incorrectly at the RS the RS sends a NAK to MR BS. If MR-BS did not receive the HARQ sub-burst from the MS correctly and received a NAK from the RS, the MR-BS sends NAK on HARQ ACK Bitmap IE to the MS. Subsequently, the MR-BS may request the MS to retransmit the HARQ sub-burst.

If MR-BS receives the HARQ sub-burst from the MS correctly then regardless of the ACK/NAK received from the RS, the MR-BS sends ACK on HARQ ACK Bitmap IE to the MS.

Multiple transparent RSs can also be involved in the HARQ process. The schedule of source station transmitting a sub-burst to multiple transparent RSs can be signaled by using UL_COMPACT_MONITOR_IE Compact_UL-MAP MONITOR IE which points to the burst to be received by the RSs.If an RS fails to decode the burst correctly, it shall not reencode the erroneous packet to transmit to the next hop station. In case of hop-by-hop HARQ involving multiple RSs, HARQ data is scheduled and forwarded to the next hop when MR-BS receives an ACK from at least one of the RSs. In case of multiple RSs when  the resource is prescheduled for all the links, one of the RSs can be selected as designated RS, which is responsible for forwarding and reporting status to MR-BS in addition to the data forwarding.

If MS sends an ACK, the designated RS reports a C0 code; otherwise the designated RS replies by choosing C2 from Table xxx.

[Insert new sub-clause 6.3.17.4.3.1]

6.3.17.4.3.1 RS group assisted HARQ

Multiple transparent RSs can also be involved in the two-hop HARQ process. The schedule of source station transmitting a sub-burst to multiple transparent RSs may be signaled by using Compact UL-MAP MONITOR IE which points to the burst to be received by the RSs. RSs use shared ACK channel to report status to MR-BS. MR-BS replies an ACK to MS if it receives the ACK from RS; otherwsie, it replies NAK to MS. If the MR-BS does not receive the ACK from the RSs, the MR-BS shall arrange data retransmission for the access link. If the MR-BS receives the ACK from the RSs but fails to decode the sub-burst, the MR-BS shall arrange data retransmission for the relay link.

[Insert new sub-clause 6.3.17.4.3.1.1]

6.3.17.4.3.1.1 Hop-by-hop HARQ

In case of hop-by-hop HARQ involving multiple RSs, HARQ data is scheduled and forwarded to the MR-BS when MR-BS receives from the RSs the ACK on shared ACK channel. If an RS receives the HARQ sub-burst from the MS correctly, then the RS stores HARQ sub-burst and reports ACK to MR-BS. If an RS fails to decode the sub-burst correctly, it shall transmit nothing in the ACK channel. If MR-BS receives the ACK, it schedules RS(s) to forward HARQ sub-burst to MR-BS. For RSs who report the ACK to MR-BS, it shall forward stored HARQ sub-burst to MR-BS. For RS who does not report the ACK to MR-BS, it shall not transmit the erroneous packet to the MR-BS.

[Insert new sub-clause 6.3.17.4.3.1.2]

6.3.17.4.3.1.2 End-to-end HARQ

In case of multiple RSs and the resource is prescheduled for all links, the MR-BS allocates UL transmission for the RS to relay the received sub-burst from MS to the MR-BS and allocates one shared ACK channel for RSs to send an ACK signal to the MR-BS. If an RS receives the HARQ sub-burst from the MS correctly, then the RS forwards HARQ sub-burst to the MR-BS and reports an ACK to MR-BS. If an RS fails to decode the sub-burst correctly, it shall not transmit the erroneous packet to the MR-BS, and it shall transmit nothing in the ACK channel.If the MR-BS receives ACK report but fails to decode the data, it should perform retransmission only for the relay link. If it does not receive ACK, it can schedule the retransmission across all hops.