| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments to P802.16j Baseline Document: 6.3.22.1.4 Association procedure in an MR network |  |
| Date Submitted | 2007-7-19 |  |
| Source(s) | Yanling Lu Hisilicon Technologies Harbour Building, No.8, Dongbeiwang West Road, HaiDian District, Beijing, China Kerstin Johnsson, Jerry Sydir Intel Corporation 2200 Mission College Blvd. Santa Clara, CA | Voice: +86 10 82829010 luyanling@hisilicon.com Voice: +1 (408) 653 9651 kerstin.johnsson@intel.com |
| Re: | IEEE 802.16j-07/019, “Call for Technical Comments Regarding IEEE Project 802.16j.” |  |
| Abstract | This contributions provides comments to the P802.16j baseline document |  |
| Purpose | Improve the text of section “6.3.22.1.4 Association procedure in an MR network” in IEEE 802.16j-07/026r4. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments to P802.16j Baseline Document:

6.3.22.1.4 Association procedure in an MR network

Kerstin Johnsson

Intel Corporation

Yanling Lu

Hisilicon Technologies

# Suggested Remedy in Comment 249#

---------------------------------------------------------Start of the Text----------------------------------------------

[Adopt the following text modification to the P802.16j baseline document]

6.3.22.1.4 Association procedure in an MR network

In an centralized MR system with distributed scheduling, when the serving MR-BS decides to recommend that the MS to scan neighbor stations with association level 1 or 2, it should obtain the association parameters allocated by available from the MS’s neighbor stations before sending the MOB_SCN-RSP message to the MS.

If the a neighbor stations are is in a different MR-cells from the MS, the MS’ serving MR-BS shall forward a message via the network backbone to the neighbor station’s serving MR-BS requesting the neighbor station’s association parameters from the neighbor MR-BS via backbone network. If the neighbor station is an MR-BS, the association parameters are readily available. However, if the neighbor station is an RS, its serving MR-BS must forward an MR_ASC-REQ message to the RS requesting that it forward its association parameters via an MR_ASC-RSP message. Then the neighbor MR-BS can obtain the association parameters of its subordinate recommended RSs and respond to the serving MR-BS.

If the a neighbor station is an RSs are served by the serving MR-BS in the same cell as the MS, the serving MR-BS can directly shall request the association parameters directly from the neighbor RSs via an MR_ASC-REQ message.

The MR-BS may respectively send an association request (ASC-REQ) message to its subordinate recommended neighbor RS, requesting their association level. When an RS receives an MR_ASC-REQ message from its serving MR-BS, it The recommended neighbor RS shall responsed with an association response (MR_ASC-RSP) message to indicate the association level it can allocated to the MS. If the allocated association level is 1 or 2, the MR_ASC-RSP should include the association parameters (i.e. Rendezvous time, CDMA code, and Transmission opportunity offset) and further.

Upon receiving these association parameters, Tthe serving MR-BS may shall determine whether the responded association parameters are satisfied satisfy the MS’ association requirements or not. If not, the serving MR-BS may request the association parameters for more times. If they do, the serving MR-BS shall include those association parameters in the MOB_SCN-RSP message.

Insert new subclause 6.3.22.1.4.2:

6.3.22.1.4.2 Association level 0

When this association level is chosen, the MS may perform association with level 0.

After a the scanned RS neighbor station successfully receives a the ranging code from a scanning MS, it will shall provide an uplink allocation of adequate size for the MS to transmit an RNG-REQ message with the TLV parameters (Serving BS ID, MS MAC address) related to the association ranging. If the neighbor station is an RS, it shall forward tThe RNG-REQ message is sent by the MS and relayed to the its serving MR-BS, then the MR-BS which shall responds with an RNG-RSP message, which is the RS shall forward relayed to the MS.

Insert new subclause 6.3.22.1.4.3:

6.3.22.1.4.3 Association level 1

When this association level is chosen, the MS may perform association with level 1.

After a the scanned RS neighbor station successfully receives a the ranging code from a scanning MS, it will shall provide an uplink allocation of adequate size for the MS to transmit an RNG-REQ message with the TLV parameters (Serving BS ID, MS MAC address) related to the association ranging.  If the neighbor station is an RS, it shall forward tThe RNG-REQ message is sent by the MS and relayed to the its serving MR-BS, then the MR-BS which shall responds with an RNG-RSP message, which the RS shall forward to the MS.

Insert new subclause 6.3.22.1.4.4:

6.3.22.1.4.4 Association level 2

When an MS is scanning using During the scanning with association level 2, the MS is required to it shall transmit the a CDMA ranging code to the scanned neighbor station RS.  The MS does not have to wait for RNG-RSP from the scanned neighbor RS station. Instead of responding to the MS with an RNG-RSP message, the neighbor station RS shall send the RNG-RSP message to its serving MR-BS (the MS's neighbor MR-BS). If that MR-BS is not the MS’ serving MR-BS, it shall The neighbor MR-BS should send the ranging information to the MS’s serving MR-BS via the backbone network backbone. Then tThe MS's serving MR-BS shall then incorporate all the RNG-RSP information from all of the MS’neighbor stations MR-BSs into a single MOB_ASC_REP message, which it forwards to the MS. If the neighbor RS is served by the serving MR-BS, it will directly send the RNG-RSP to the serving MR-BS.

---------------------------------------------------------End of the Text-----------------------------------------------