| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | OFDMA AAS downlink IE for specifying AAS Direct Signaling mode of operation. |  |
| Date Submitted | 2007-09-20 |  |
| Source(s) | Dale Branlund, Matt Volpe, Will Sun BRN Phoenix Inc. 2500 Augustine Drive Santa Clara, CA, USA 95054 John Norin, Robert Popoli The DIRECTV Group, Inc. 2250 East Imperial Hwy El Segundo, CA 90245 | Voice: +1-408-572-9703 Fax: +1-408-351-4911 dbranlund@brnphoenix.com Voice: +1-310-964-0717 Fax: +1-310-535-5422 john.norin@directv.com |
| Re: | Working Group Letter Ballot #28, Technical Comments and Contributions regarding IEEE Project P802.16j; Draft Amendment P802.16/D1. |  |
| Abstract | This contribution describes an update to the OFDMA AAS downlink IE necessary to set up the AAS Relay Zone for Direct Signaling mode of operation. |  |
| Purpose | This document provides the necessary corrections to properly set up the AAS Relay Zone for Direct Signaling mode of operation. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# OFDMA AAS downlink IE for specifying AAS Direct Signaling mode of operation

# Dale Branlund, Will Sun, Matt Volpe, BRN Phoenix Inc., Santa Clara, CA, USA;
John Norin, Robert Popoli, The DIRECTV Group, Inc., El Segundo, CA, USA

This document describes IEs necessary to define the AAS Relay Zone.

## Background

The IEEE 802.16j D1 draft provides the OFDMA AAS_Relay _UL_IE that is used to set up the AAS Relay zone for direct signaling mode of operation. However, it does not describe the IE for the downlink.  The OFDMA AAS_Relay _DL_IE is required to properly set up the AAS Relay zone. The AAS_Relay _DL_IE needs to specify permutations, the zone configuration duration, and the AAS Relay zone type.

## Proposed Solution

The proposed solution is to provide the AAS_Relay _DL_IE to fully define the AAS Relay Zone.   Since both the AAS_Relay _DL_IE and AAS_Relay _UL_IE are signaled in the R-MAP, these IEs should appear in subclause 8.4.5.9 R-Map Messages.

## Detailed Solution

A number of AAS_Relay _DL_IE parameters must be supplied to support the configuration of Direct Signaling capability within the AAS Relay Zone.

The permutation parameter describes the permutations included in the AMC permutation of 2 bins x 3 symbols and 1 bin x 6 symbols.

The AAS Frame Repetition Field sets the number of frame repetitions for which the AAS zone configuration is valid. If the AAS Relay Zone Type = 1, specifying Direct Signaling, then the DL_PermBase/AAS Frame Repetition parameter field specifies a 3 bit value of N. The AAS Relay Zone configuration specified in the AAS_Relay _DL_IE is valid for a duration M = 2N frames before the IE must be resent.   This improves the efficiency of the R-MAP. The AAS Frame Repetition count, M, applies to both the DL and UL.

The AAS relay zone type is configurable by setting the AAS Relay Zone Type bit-flag.  This determines whether the zone supports the Diversity Map AAS method (as described in 8.4.4.6) or Direct Signaling AAS method (as described in 8.4.4.7.2.3).

## Proposed Text Changes

Insert  subclause 8.4.5.9.1.3

8.4.5.9.1.3 AAS Relay Downlink IE (AAS_Relay_DL_IE)

Table 496f  AAS_Relay_DL_IE

| Syntax | Size | Notes |
| --- | --- | --- |
| AAS_Relay_DL_IE() { |  |  |
| Type | 5 bits | AAS_Relay_DL_IE = 0x02 |
| Length | 4 bits | Length = 0x03 |
| OFDMA symbol offset | 8 bits | Denotes the start of the zone (counting from the frame preamble and starting from 0) |
| Permutation | 3 bits | 0b000 = PUSC permutation 0b001 = FUSC permutation 0b010 = Optional FUSC permutation 0b011 = AMC 0b100 = TUSC1 0b101 = TUSC2 0b110 = adjacent-subcarrier permutation, 2 bins x 3 symbols 0b111 = adjacent-subcarrier permutation, 1 bin x 6 symbols |
| DL_PermBase/AAS Frame Repetitions | 6 bits | if AAS Relay Zone Type = 1 Bit #0-2: Defines N, where the zone configuration is valid for M = 2N frames. Bit #3-5: Reserved Otherwise Bit #0-5: Permutation Base for the specified DL AAS Zone |
| Downlink_preamble_config | 2 bits | 0b00 = 0 symbols 0b01 = 1 symbol 0b10 = 2 symbols 0b11 = 3 symbols |
| Preamble type | 1 bit | 0 – Frequency shifted preamble is used in this DL AAS zone 1 – Time shifted preamble is used in this DL AAS zone |
| PRBS_ID | 2 bits | Values: 0..2. Refer to 8.4.9.4.1 |
| AAS Relay Zone Type | 1 bit | 0: Diversity Map Relay Zone 1: Direct Signaling Relay Zone |
| Reserved | 0 bit | Shall be set to zero |
| } |  |  |

OFDMA Symbol offset

The offset of the OFDMA symbol in which the AAS Zone starts, measured in OFDMA symbols

from beginning of the current downlink frame.

Permutation

Defines the permutation used within the DL AAS Zone.

DL_PermBase/AAS Frame Repetitions

If AAS Relay Zone Type = 0 (Diversity Map), then this defines the Permutation Base for the

specified DL AAS Zone.

If AAS Relay Zone Type = 1 (Direct Signaling), then this represents exponent N of the zone

configuration duration calculation, where the zone configuration is valid for a duration M = 2N frames.

The zone configuration duration applies to both the DL and the UL.

AAS Relay Zone Type

Defines which type of AAS is supported within the AAS Relay Zone.

Downlink_preamble_config

Defines the number of DL AAS preambles to be used before each DL burst in the AAS zone.

Remove Subclause 8.4.5.4.6

Insert Subclause 8.4.5.9.1.4

8.4.5.9.1.4 AAS Relay Uplink IE (AAS_Relay_UL_IE)

Table 496f   AAS_Relay_UL_IE

| Syntax | Size | Notes |
| --- | --- | --- |
| AAS_Relay_UL_IE() { |  |  |
| Type | 5 bits | AAS_Relay_UL_IE = 0x03 |
| Length | 4 bits | Length = 0x04 |
| Permutation | 2 bits | 0b00 = PUSC permutation 0b01 = Optional PUSC permutation 0b10 = adjacent-subcarrier permutation, 2 bins x 3 symbols 0b11 = adjacent-subcarrier permutation, 1 bins x 6 symbols |
| UL_PermBase | 7 bits |  |
| OFDMA symbol offset | 8 bits |  |
| AAS zone length | 8 bits | Number of OFDMA symbols in AAS zone |
| Uplink_preamble_config | 2 bits | 0b00 - 0 symbols 0b01 - 1 symbols 0b10 - 2 symbols, 4 symbols if AAS Relay Zone Type=1 0b11 - 3 symbols, 8 symbols if AAS Relay Zone Type=1 |
| Preamble type | 1 bit | 0 – Frequency shifted preamble is used in this UL AAS zone 1 – Time shifted preamble is used in this UL AAS zone 0 = Hadamard preamble, if AAS Relay Zone Type =1) 1 = Reserved |
| Number of AAS Relay Zone Access Channel subchannels | 1 bit | 0 = 1 subchannel pair 1 = 2 subchannel pairs |
| AAS Relay Zone Type | 1 bit | 0 = Diversity MAP Relay Zone 1 = Direct Signaling Relay Zone |
| Reserved | 1 bits | Shall be set to zero |
| } |  |  |

Change data in Type field Table 496c from 02 to1F to 05 to 1F.

Insert line in Table 496c below the row beginning with Type =01 with this data:

Type = 02, Usage = AAS_Relay_DL_IE()

Insert line in Table 496c below the line above with this data:

Type = 03, Usage = AAS_Relay_UL_IE()