| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Repeater Zone in RS |  |
| Date Submitted | 2007-09-07 |  |
| Source(s) | Junbae Ahn SOLiD Technologies 10th Fl., IT Venture Tower East Wing, 78 Garak-Dong, Songpa-Gu, Seoul, Korea | Voice: +82-2-2142-3861 Fax: +82-2-2142-3846 E-mail: jbahn@st.co.kr |
| Re: | Reply for IEEE 802.16 Working Group Letter Ballot #28 |  |
| Abstract | Enhancement of data throughput using Repeater Zone in RS |  |
| Purpose | Adoption of proposed changes into IEEE P802.16j/D1 (August 2007) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Repeater Zone in RS

Junbae Ahn

SOLiD Technologies

# 1. Introduction

Actually, RS has to extend the coverage of MR-BS and get a throughput enhancement at the same time. In many RS application, it is impossible to get two conditions at the same time. It needs extra bandwidth for relaying. Repeater Zone is defined to allow Relay amplify and forward data. MR-BS controls Repeater Zone for RS. It will be a good solution to improve data throughput where antenna isolation is not considered. We suggest that MR-BS and RS may have a Repeater Zone based on the relay function.

# 2. Proposed Method

To operate Repeater Zone in RS and MR-BS symbol, locations of Repeater Zone in RS shall be assigned and announced to the RS using MAC. It is simple structure. Assigned locations may be one or more symbols.

# 3. Proposed Text Changes

Add a section 3.xx.

3.xx DL repeater zone : A portion of the DL sub-frame in the RS frame used for MR-BS to RS/MS transmission

Add a section 3.yy.

3.yy UL repeater zone : A portion of  the UL sub-frame in the RS frame used for MS to MR-BS/RS transmission

Insert followings at the end of a section 6.3.2.3.23.

Repeater Zone support(11.8.3.7.xx)

Insert followings at the end of a section 6.3.2.3.24.

Repeater Zone support(11.8.3.7.xx)

Change the table 183n. in section 6.3.2.3.73 as indicated.

| if(DL indicator ==1) { |  |  |
| --- | --- | --- |
| Number of frame | 8 bits |  |
| for(i=0; i<Number of frame; i++) { |  |  |
| Number of relay zones | 2 bits |  |
| Number of repeater zone | 2 bits |  |
| reserved | 64 bits |  |
| for(j=0; j<Number of relay zone; j++) { |  |  |
| Transceiver mode | 2 bits | 00: Tx mode 01: Rx mode 11 : Idle mode |
| OFDMA Symbol Offset | 8 bits |  |
| Frame_Config_Duration | 6 bits |  |
| } |  |  |
| for(j=0; j<Number of repeater zone; j++) { |  |  |
| Transceiver mode | 2 bits | 10: TRx mode 11: Idle mode |
| OFDMA Symbol Offset | 8 bits |  |
| Frame_Config_Duration | 6 bits |  |
| } |  |  |
| } |  |  |
| } |  |  |

Change the table 183n. in section 6.3.2.3.73 as indicated

| if(UL indicator ==1) { |  |  |
| --- | --- | --- |
| Number of frame | 8 bits |  |
| for(i=0; i<Number of frame; i++) { |  |  |
| Number of relay zone | 2 bits |  |
| Number of repeater zone | 2 bits |  |
| Reserved | 64 bits |  |
| for(j=0; j<Number of relay zone; j++) { |  |  |
| Transceiver mode | 2 bits | 00: Tx mode 01: Rx mode 11 : Idle mode |
| OFDMA Symbol Offset | 8 bits |  |
| Frame_Config_Duration | 6 bits |  |
| } |  |  |
| for(j=0; j<Number of repeater zone; j++) { |  |  |
| Transceiver mode | 2 bits | 10: TRx mode 11: Idle mode |
| OFDMA Symbol Offset | 8 bits |  |
| Frame_Config_Duration | 6 bits |  |
| } |  |  |
| } |  |  |
| } |  |  |

Change subclause in 6.3.2.3.73 as indicated.

Transceiver mode

Transceiver mode in the relay zone is one of either Tx mode, Rx mode, or Idle mode. In the repeater zone, it is one of either TRx mode or Idle mode. When the transceiver mode is idle mode, it does not transmit nor receive.

Change subclause in 6.3.2.3.73 as indicated.

OFDMA Ssymbol Offset

The relay zone or repeater zone starts at the OFDMA Ssymbol Offset.

Change subclause in 6.3.2.3.73 as indicated.

Frame Configuration Duration

The relay zone or repeater zone ends after the duration starting from the OFDMA symbol offset. The unit of duration is OFDMA symbol.

Change subclause in 8.4.4.7.1.1 as indicated.

The DL sub-frame shall include at least one access zone and may include one transparent zone and may include repeater zone for RS to subordinate station transmissions. The MR-BS may also transmit in the transparent zone as well. The transparent zone shall be indicated by an STC_DL_ZONE_IE(), as defined in Table 279. The UL sub-frame may include a UL access zone, and a UL relay zone and repeater zone for superordinate RS or MR-BS.

Change subclause in 8.4.4.7.1.2 as indicated.

The DL sub-frame shall include on access zone for MR-BS to RS and MS transmissions and may include one transparent zone for RS to subordinate station transmissions and may include repeater zone for subordinate RS or MS. The UL sub-frame may include on access zone and may include one relay zone for RS to superordinate station transmissions and may include repeater zone for superordinate RS or MR-BS.

#### Add a section 11.8.3.7.xx

This field indicates the availability of RS for Repeater Zone.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| TBA | 1 | Bit #0: Repeater Zone support Bit #1-7 : reserved | SBC-REQ SBC-RSP |