| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications of RSID in RS Configuration Message |  |
| Date Submitted | 2007-09-09 |  |
| Source(s) | Fang-Ching Ren, Chie Ming Chou, Tzu-Ming Lin, Wern-Ho Sheen, I-Kang Fu ITRI/ National Chiao Tung University (NCTU) | frank_ren@itri.org.tw |
| Re: | Comments with Letter Ballot #28 “P802.16j/D1” |  |
| Abstract | This contribution describes the clarifications for RS configuration message to avoid the confliction on usage of multicast CIDs. |  |
| Purpose | To make IEEE Project 802.16j more maturity |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications for RSID in RS Configuration Message

- Problem Statement

In IEEE P802.16j/D1, subclause 6.3.2.3.67 specifies RS configuration request (RS_Config-REQ) message. In RS_Config-REQ message, an RS can be assigned a unicast RSID and/or a multicast RSID (both are 8 bits) which can be used to manage the RS individually or as a group. However, there is no specification to use the two RSIDs as connection identifications in control message, MAC header, or subheader. Therefore, the definition and usage of the two RSIDs should be clarified.

- Suggested Remedy

To support a RS can be managed individually or as a group, it is necessary to provide RSIDs to be unique within the associated MR-BS.

- For unicast function, each RS has been assigned a basic CID during initial ranging procedure. Therefore, the unicast RSID defined in Table 183f of P802.16j/D1 should be removed.

- For multicast function, a MR-BS can assign a multicast CID to associate with multiple RSs.

- For backward compatibility, both basic CID and multicast CID can be used in MAP IE to indicate the granted resource and in MAC control header for connection identifications.

- To reduce the overhead cost for connection identifications, reduced CID (RCID) can be adopted, e.g., R-MAP IE in P802.16/D1.

- Proposed Text Change

-----------------------------------------------------Start text proposal----------------------------------------------------

[Adopt the following modifications into the P802.16j/D1 document]

6.3.2.3.67 MR-BS configuration Request message

[Change the text in Table 183f as indicated:]

Table 183f-RS_Config-REQ message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RS_Config-REQ_Message_Format { |  |  |
| Management Message Type = 72 | 8 bits |  |
| Configuration_para_type | 8 bits | b0 = 1: preamble configuration is included; b1 = 1: remove multicast RSID CID to disassociate from the RS group; b2 = 1: Unicast RSID is included reserved ; b3 = 1: Multicast RSID CID is included; b4 = 0; Do not transmit preamble; 1: transmit the assigned preamble. b5 = 1: R-amble configuration is included b6 - b7: reserved |
| If (b0 of Configuration_para_type == 1 ) { |  |  |
| Preamble_index | 8 bits | Assign a preamble index value to the potential RS |
| } |  |  |
| If (b2 of Configuration_para_type == 1 ) { |  |  |
| Unicast RSID | 8 bits | Unicast RSID |
| } |  |  |
| If (b3 of Configuration_para_type == 1 ) { |  | Setting required operation parameters within a RS group. |
| Multicast RSID CID | 816 bits | Multicast RSIDCID as the RS Group ID |

[Change the description below Table 183f as indicated:]

Unicast RSID

This field is used to indicate the Unicast RSID.

Multicast RSIDCID

This field is used to indicate the Multicast RSIDCID for RS group operations.

6.3.9.16.3.1 RS grouping

[Change the second bullet text description as follows]

- Each RS is assigned an individual unicast RSID basic CID during initial ranging procedure and a multicast RSID CID as the RS group ID during network entry procedure. The multicast RSID CID is the same for all members in the group. With these two separate IDs, the RS can be managed individually or as a group. These IDs are unique within the associated MR-BS. The association of an RS to an RS group is configured using RS_Config_REQ message (see section 6.3.2.3.67). The multicast RSID CID is used for messages that are addressed to all the members of the RS group, and the unicast RSID basic CID is used for the individual members of the RS group. The MS network entry procedure follows the 6.3.9.16.3.1.1. The MS handover procedure follows 6.3.22.5.2. For an transparent (or non-transparent) RS group all the other procedures follow the procedures for the transparent (or non-transparent) RS. For example, if the RS group is non-transparent, the MAPs for the individual RSs will be received by the RSs according to the associated procedure defined for a nontransparent RS using the multicast RSID (see section 6.3.28.1).

-------------------------------------------------------End of text ---------------------------------------------------------