| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On MR_LOC-REQ/RSP messages |  |
| Date Submitted | 2007-09-09 |  |
| Source(s) | Adrian Boariu, Shashikant Maheshwari, Haihong Zheng, Yousuf Saifullah, Peter Wang Nokia Siemens Networks | E-mail: adrian.boariu@nsn.com |
| Re: | This is in response for call for comments P802.16j/D1 |  |
| Abstract | Some clarifications for the MR_LOC_REQ/RSP |  |
| Purpose | Review and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# On MR_LOC-REQ/RSP messages

Adrian Boariu, et al.

NSN

# Introduction

The MR_LOC-REQ/RSP messages in P802.16j/D1 document can be optimized while offering the same functionality.

As described in section 6.3.2.3.69.1 on p. 45, the MR_LOC-REQ message can be used both by the MR-BS and by the RS:

- When MR-BS sends this message to RS, the RS provides only its position, and this report can be set to be periodically or not;

- When the RS sends this message to MR-BS it can request position of other RSs that are identified in the message.

It is apparent that based on the originator of the message, the message content can be easily created as provided in the specification changes. This proposed message simplifies the current one.

A similar optimization can be applied to MR_LOC-RSP message.

# Specification changes

[Change the Table 183h in section 6.3.2.3.69.1 as following]

| Syntax | Size | Notes |
| --- | --- | --- |
| MR_LOC-REQ_ Message_Format() { | - | - |
| Management Message Type = 74 | 8 bits | - |
| if(MR-BS sends to RS) { | - | . Message send by the MR-BS |
| Report period | 12 bits | Report period in units of frame, corresponding to a range of 1 frame to 4095 frames. The value of 0 cancels the periodic reporting. |
| Reserved | 4 bits | Shall be set to zero |
| } | - | - |
| If ( RS sends to MR-BS) { | - | Message send by the RS |
| N_RS | 7 bits | Number of neighboring stations for which the RS wants to know the location information. |
| ID_Type | 1 bit | 0: R-amble index 1: BS ID |
| For (j=0;j<N_RS; j++) { | - | - |
| ID_Type_index | variable | The of the neighboring station whose location is requested. 8 bits if ID_Type is 0 or 1 48 bits if ID_Type is 2 |
| } | - | - |
| } | - | - |
| TLV Encoded Message | variable | TLV Encoded Message |
| } | - | - |

The following parameters shall be included in the MR_LOC_REQ message:

[Change the Table 183i in section 6.3.2.3.69.2 as following]

| Syntax | Size | Notes |
| --- | --- | --- |
| MR_LOC-RSP_ Message_Format() { | - | - |
| Management Message Type = 75 | 8 bits | - |
| if (RS responds to MR-BS) { | - | This message is transmitted by an RS to MR-BS. See MR_LOC-REQ. |
| LLA_IE() | 64 bits | Specifies the location of relay station in LLA format defined in section 6.3.2.3.62.3. |
| } | - |  |
| if (MR-BS responds to RS){ |  | This message is transmitted by MR-BS to RS. See MR_LOC-REQ. |
| N_RS | 7 bits | Number of stations whose location information is included in the current MR_LOC-RSP message. |
| Reserved | 1 bit | Set on zero |
| for (j=0;j<N_RS;j++) { | - | - |
| If (ID_Type==0){ |  | The RS requested using R-amble ID_Type |
| R-amble | 8 bits |  |
| } |  |  |
| BS ID | 48 bits | The of the neighboring station whose position was requested. |
| LLA_IE() | 64 bits | Specifies the location of neighbor access station in LLA format defined in section 6.3.2.3.62.3. |
| } | - | - |
| } | - | - |
| TLV Encoded Message | variable | TLV Encoded Message |
| } | - | - |

The following parameters shall be included in the MR_LOC_RSP message: