| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On RS_CD message |  |
| Date Submitted | 2007-09-09 |  |
| Source(s) | Adrian Boariu, Shashikant Maheshwari, Haihong Zheng, Yousuf Saifullah, Peter Wang Nokia Siemens Networks | E-mail: adrian.boariu@nsn.com |
|  | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee | Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan loa@iii.org.tw |
|  | Gamini Senarath Nortel | gamini@nortel.com |
| Re: | This is in response for call for comments P802.16j/D1 |  |
| Abstract | Some clarifications for RS_CD message |  |
| Purpose | Review and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# On RS_CD message

Adrian Boariu, et al.

NSN

# Introduction

The RS_CD message provides configuration parameters for an RS. The current message does not include certain parameters like EIRP, BSID, etc., so we have re-written this message.

Comments resolved:

356: 	superseded by 361 and by 624

357, 358: 	by 361

361: 	Do accept modify: Delete Unicast RSID row and change

Multicast RSID   |   168 bits  |  Multicast RSID as the RS Group ID RS Multicast CID

362:	superseded by 361

363, 364:	accept

383, 384, 393, 394, 395, 445, 1025:	solved

654:	accept

644, 796, 800, 803:	accept modify: instead of RS_Config-REQ/RCM use RD_CD

1011:	Accept the suggestion of Hang Zhang.

1021: 	Reject and replace all instances of RS-CD with RS_CD

661, 854, 907

# Specification changes

[Replace the section 6.3.2.3.73 as following]

This message is multicasted or is sent using the primary management CID to an RS. The message is used for initial network entry during neighborhood measurement or reconfiguration procedure, in order to inform the (re)configuration parameters that the RS shall apply. Upon receiving the message, the RS responds using MR_Generic-ACK (6.3.2.3.86) only if it evaluates that the configuration can be applied on time.

NOTE: The following messages are affected by this new format: REG-REQ/RSP, SBC-REQ/RSP, RS_Config-REQ

Table 183n—RS configuration description (RS_CD) message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RS-CD_Message_Format(){ |  |  |
| Management Message Type = 79 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Frame Number Action | 8 bits | The LSB 8 bits of the access station FN when the configuration shall be applied. |
| CPAT (Configuration Parameter and Action Type) bitmap | 16 bits | b0=1, Preamble index included b1=1, R-amble index included b2=1, RS EIRP included b3=1, RS Frame Offset included b4=1, BSID included b5=1, Used Subchannel included b6=1, DL subframe structure configuration included b7=1, UL subframe structure configuration included b8=1, R-amble transmission/monitoring parameters included b9-b15: reserved |
| If (b0 of CPAT == 1) { | - | - |
| Preamble index | 8 bits | The preamble index assigned. This is for non-transparent RS. |
| } | - | - |
| If (b1 of CPAT == 1) { | - | - |
| R-amble index | 8 bits | The R-amble index assigned. |
| } | - | - |
| If (b2 of CPAT == 1) { | - | - |
| RS EIRP | 8 bits | The EIRP value to be used by the RS on DL. |
| } | - | - |
| If (b3 of CPAT == 1) { | - | - |
| RS Frame Offset | 8 bits | LSB 8 bits, unsigned integer frame offset value between FN used by the RS’s superordinate station and FN that should be used by the RS Suggested to be used in order to generate different subcarrier randomization sequences (see 8.4.9.4.1.) |
| } | - | - |
| If (b4 of CPAT == 1) { | - | - |
| BSID | 48 bits | BSID assigned to RS. |
| } | - | - |
| If (b5 of CPAT == 1) { | - | - |
| Used Subchannel bitmap | 6 bits | bit #0, Subchannel group 0 bit #1, Subchannel group 1 bit #2, Subchannel group 2 bit #3, Subchannel group 3 bit #4, Subchannel group 4 bit #5, Subchannel group 5 |
| Reserved | 2 bits | shall be set to zero |
| } | - | - |
| if(b6 of CPAT == 1) { | - | - |
| Number of frames | 8 bits | This value shall be the same value as that used when UL configuration is present. |
| for(i=0; i<Number of frames; i++){ | - | - |
| Number of zones | 3 bits | First zone is always access zone for DL |
| reserved | 5 bits | Shall be zero |
| for(j = 0; j<Number of zones; j++){ | - | - |
| Transceiver mode | 2 bits | 00: Tx mode 01: Rx mode 11: Idle mode |
| OFDMA Symbol Offset | 7 bits |  |
| Frame_Config_Duration | 6 bits |  |
| Zone Configuration indicator | 1 bit | 1: Zone Configuration included |
| if (Zone Configuration indicator ==1) { | - | - |
| Zone Configuration IE _Format() | Variable | see Table 183o |
| } | - | - |
| } | - | - |
| } | - | - |
| } | - | - |
| if(b7 of CPAT ==1){ | - | - |
| Number of frames | 8 bits | This value shall be the same value as that used when DL configuration is present. |
| for(i =0; i<Number of frames; i++){ | - | - |
| Number of zones | 3 bits |  |
| reserved | 5 bits | Shall be zero |
| for(j = 0; j<Number of zones; j++){ | - | - |
| Transceiver mode | 2 bits | 00: Tx mode 01: Rx mode 11: Idle mode |
| OFDMA Symbol Offset | 7 bits |  |
| Frame_Config_Duration | 6 bits |  |
| Zone Configuration indicator | 1 bit | 1: Zone Configuration included |
| if (Zone Configuration indicator ==1) { | - | - |
| Zone Configuration IE _Format() | Variable | see Table 183o |
| } | - | - |
| } | - | - |
| } | - | - |
| } | - | - |
| If(b8 of CPAT == 1){ | - | - |
| Start Frame Number | 8 bits | 8 LSB bits of the frame number at MR-BS |
| Monitoring_Duration | 8 bits | Units are frame |
| Prefix | 2 bits | 00: The R-amble transmission and measurement is instructed by MR-BS. 01: The R-amble transmission and measurement shall be performed autonomously 10: The RSs shall report its neighbor measurement results 11: reserved |
| Report Request | 1 bit | 0:RSSI 1:CINR |
| Reserved | 1 bit | Shall be zero |
| if(Prefix == 00) { | - | - |
| Interleaving Interval | 8 bits | Units are frame |
| Iteration Number | 8 bits | Units are frame |
| N_stations | 6 bits | Number of stations received this message |
| For(i=0; i<Iteration; i++){ | - | - |
| N_Transmitter | 6 bits | Number of stations to transmit the R-amble |
| for(j=0;j<N_Transmitter;j++){ | - | - |
| Amble Index | 8 bits | The RS with the amble index in this list shall transmit the R-amble |
| } | - | - |
| for(j=0;j<N_stations - N_Transmitter; j++){ | - | - |
| Amble Index | 8 bits | The RS with the amble index in this list shall receive the R-amble |
| } | - | - |
| } | - | - |
| } | - | - |
| If(Prefix = 01){ | - | - |
| Config_type | 3 bits | Bit [0] = 1: R-amble for synchronization is present. Bit [0] = 0: R-amble for synchronization is not transmitted. Bit [1] = 1: R-amble for random monitoring is present; Bit [1] = 0: any current monitoring operation is to be stopped by all RSs. Bit [2] = 1: any RS which does not support subordinate RSs should transmit the R-amble for advertisement purpose Bit [2] = 0: any RS which does not support subordinate RSs should not transmit the R- amble |
| Reserved | 1 bit | Shall be zero |
| if(Config_type[0] == 1){ | - | - |
| Synchronization cycle | 8 bits | N, Units are frame (see subsection 8.4.6.1.1.3.1) |
| Synchronization frame offset | 4 bits | Ks, Units are frame (see subsection 8.4.6.1.1.3.1) |
| Reserved | 4 bits | Shall be zero |
| } | - | - |
| If(Config_type[1] == 1){ | - | - |
| Neighbor monitoring cycle | 4 bits | M, Units are frame (see subsection 8.4.6.1.1.3.2) |
| Neighbor monitoring frame offset | 4 bits | Km, Units are frame (see subsection 8.4.6.1.1.3.1) |
| Neighbor monitoring frame repetition | 8 bits | L, Units are frame (see subsection 8.4.6.1.1.3.1) |
| } | - | - |
| } | - | - |
| } | - | - |
| Encoded TLV | variable |  |
| } | - | - |

Frame Number Action

This is the frame number for configuration to take effect. The RS applies the configuration starting from this frame number of the access station and given in this parameter.

RS Frame Offset

RS frame offset indicates the offset value between frame number used by the RS’s superordinate station and frame number that should be used by the RS for whom this message is addressed. When the RS broadcast frame number in its frame, RS shall keep the offset relative to the frame number used by the superordinate station as indicated by this value. The value represents LSB 8 bits, unsigned integer for the frame offset.

RS EIRP

The MR-BS shall indicate to the RS the EIRP the RS can utilize on the access DL preamble and advertised in any DCD message transmitted by the RS on the access link. The EIRP parameter is reported in dBm and quantized in 1dB steps ranging from [TBD]dBm (encoded 0x00) to [TBD]dBm (encoded 0xFF). Values outside this range shall be assigned the closest extreme.

Transceiver mode

Transceiver mode in the relay zone is one of either Tx mode, Rx mode, or Idle mode. When the transceiver mode is idle mode, it does not transmit nor receive.

OFDMA symbol Offset

The relay zone starts at the OFDMA symbol Offset counted after the preamble of the corresponding frame.

Frame Configuration Duration

The relay zone ends after the duration starting from the OFDMA symbol offset. The unit of duration is OFDMA symbol.

Number of frames

This field indicates the number of frames in a multi-frame. The value zero is not valid. If the value is one, this indicates single-frame structure.

Start Frame Number

The RS shall start transmitting/receiving the R-amble from this designated frame number

Monitoring_Duration

Duration (in units of frames) of the measurement/monitoring/transmission process. If the Monitoring_Duration value is set to 0x00 and prefix is 0b01 monitoring is to be continued until further notice

Interleaving Interval

The period (in units of frames) which is interleaved between the consecutive R-amble transmission/reception opportunity

Iteration

The requested number of iterating intervals

N_Transmitter

Number of stations instructed to transmit R-amble, the station may be RS or MR-BS.

N_Receiver_RS

Number of RSs instructed to receive R-amble

Amble index

The index of the R-amble (see 8.4.6.1.1.3)

Synchronization Cycle Length, N

This field is used to indicate the synchronization R-amble period if present

Synchronization Frame Offset, Ks

The offset of the second R-amble in the synchronization cycle

Neighbor Monitoring Frame Repetition Rate, L

This field is used to indicate the neighbor monitoring R-amble period if present

Neighbor Monitoring Frame Offset, Km

The offset of the R-amble in the neighbor monitoring cycle

Neighbor Monitoring Cycle Length, M

This defines the number of neighbor monitoring R-amble frames in an R-amble monitoring cycle

The RS-CD message may include the following TLVs:

UL allocation start time

This TLV indicates the effective start time of the uplink allocation defined by the R-MAP on R-link. If the effective start time is defined as 0, the uplink allocation defined by the R-MAP is effective in the current frame; if the value of this TLV is set to N, the uplink allocation defined by the R-MAP in frame i is effective in frame i + N.

When the Prefix is set as "00", the RS shall follow the pattern instructed by MR-BS to transmit/receive the R-amble. The pattern is composed by the amble index, and the RS shall transmit/receive the R-amble according to the field where its amble index is. Start Frame Number is the 8 LSB bits of frame number index used to indicate the starting point of subsequent R-amble transmission/reception opportunities. In order to coordinate the R-amble transmission/reception in different MR-cell, a coordinator in backhaul network is needed to ensure the Start Frame Number parameters sent in different MR-cell will align to the same time.

If the Prefix is set "01", the RS will autonomously transmit/receive the R-amble without periodic instruction from MR-BS by defining R-amble repetition patterns and monitoring patterns. The deactivation or active tion of the functionalities of individual RSs can be done by sending (unicast) this message during initial entry of an RS. In the case of conflict, broadcast message parameters shall supersede the unicast message parameters except for the case of the parameter M which shall be set only by the unicast message. The detail design of the associated parameters is stated in 8.4.6.1.1.4. When the RS is instructed to transmit/receive the R-amble transmission autonomously, the RS shall send the measurements using standard measurement reporting mechanisms already defined in this document. Alternatively, MR-BS can instruct the RS to report its measurement results by this message with the prefix set as "10".

RS paging group (see 11.23.2)

Preamble indexes reserved for moving relay station (see. 11.23.1)

Preamble reselection thresholds (see. 11.24)

HMAC/CMAC Tuple (see 11.1.2)

Table 183o Zone Configuration IE

| Zone Configuration IE _Format() { | - | - |
| --- | --- | --- |
| Zone Configuration bitmap | 8 bits | b0=1, permutation based included b1=1, range of subchannels included b2=1, STC mode b3=1, Cooperative diversity mode b4=1, AMC mode b5=1, Safety/PAR reduction mode b6-b7: reserved |
| if (b0 of Zone Configuration bitmap ==1) { |  |  |
| Permutation based | 6 bits | DL_PermBase to be used in this zone |
| Reserved | 2 bits |  |
| } | - | - |
| if (b1 of Zone Configuration bitmap ==1) { | - | - |
| Min Subchannel index | 8 bits | The index of subchannel from which the allocation starts. |
| Max Subchannel index | 8 bits | The index of subchannel at which the allocation ends. |
| } | - | - |
| if (b2 of Zone Configuration bitmap ==1) { | - | - |
| STC mode | TBD |  |
| } | - | - |
| if (b3 of Zone Configuration bitmap ==1) { | - | - |
| Cooperative diversity mode | TBD |  |
| } | - | - |
| if (b4 of Zone Configuration bitmap ==1) { | - | - |
| AMC mode | TBD |  |
| } | - | - |
| if (b5 of Zone Configuration bitmap ==1) { | - | - |
| Safety/PAR reduction mode | TBD |  |
| } | - | - |
| } | - | - |

More changes need to be done…

[Change the section 6.3.2.3 as following]

Change the line 42 the “Connection” column for RS_CD from “Basic” to “Primary”

Delete the section 11.7.27 RS frame offset.

Delete on p. 22 lines 11 and 22 the “RS frame offset (11.7.27)”. Also delete the lines 27 and 28 with the sentence that refers to this TLV.

{Modify the text in subclause 6.3.9.16.3.3 as follows (comment #661:]

6.3.9.16.3.3 Neighbor station measurement report

During the neighborhood measurement process at the network entry or re-entry, an RS may be required, as indicated in the RS network entry optimization TLV in the RNG-RSP message, to report the signal strength and preamble/R-amble index of neighbor stations. If required, the RS shall send the RS_NBR-MEAS-REP message (6.3.2.3.68) to the MR-BS to report the neighbor station measurements after completion of registration. MR-BS may transmit RS-CD message to the RS to indicate the R-amble monitoring pattern and R-amble location and MR-BS may also request a complete neighborhood discovery as described in 6.3.26.

{Modify the following text as indicated below (comment #854}

6.3.26 Relay station neighborhood discovery

The transmission and monitoring frames for the R-amble are specified by the MR-BS sending the RS Configuration Description (RS-CD) message (6.3.2.3.75). Each RS finds out the R-amble sequences from the parameters provided in the RS-CD message. More details are provided in Section 8.4.6.1.1.4. After measurement are done tThe measurement results can may be sent to the MR-BS using the message RS_NBR-MEAS-REP (6.3.2.3.668) or using any other appropriate measurement report messages, periodically or as requested by the MR-BS.

Because not every RS will transmit its own preamble and the existing RSs in MR network need to perform  measurement over the RS, MR-BS may instruct the RSs to perform complete neighborhood discovery/measurement at initial network entry this stage:

The neighborhood discovery/measurement can be used in different stages of its operation, during neighborhood measurement process in the initial network entry (before path selection stage), during periodic intervals or whenever an MR-BS requires this information. There are two methods to carry out this neighborhood measurements: (1) Repeatable R-amble transmission and monitoring  scheme; (2) Pre-planned R-amble transmission and monitoring scheme.

{Modify the following text as indicated below (comment# 907) }

{In Section 8.4.4.7.2.2, page 167, line 16}

The MR-BS or RS shall transmit the RS-CD message in the access zone for the subordinate RSs to configure the multihop relay frame structure through during initial network entry or shall transmit the RS-CD message in the DL relay zone, when the frame configuration is changed through normal relay operation.