| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On RS_Member_List_Update message |  |
| Date Submitted | 2007-09-09 |  |
| Source(s) | Adrian Boariu, Shashikant Maheshwari, Haihong Zheng, Yousuf Saifullah, Peter Wang Nokia Siemens Networks | E-mail: adrian.boariu@nsn.com |
| Re: | This is in response for call for comments P802.16j/D1 |  |
| Abstract | Some clarifications for RS_Member_List_Update |  |
| Purpose | Review and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# On RS_Member_List_Update message

Adrian Boariu, et al.

NSN

# Introduction

The RS_Member_List_Update message provides the list of CIDs to be monitored by the members of the RS group. We are providing an optimization of the message while keeping the same functionality.

The following comments are superseded by this contribution: #438, #810, #811.

# Specification changes

[Change the section 6.3.2.3.84 as following]

The superordinate station of the RS group may transmit RS_Member_List_Update message using the RS basic CID or the multicast CID of the RS group, to update the group members with the details of the traffic burst they shall forward. This message may be transmitted whenever there is a change in the connection list of the RS group members. The RSs receiving the message may respond with MR_Generic-ACK or Acknowledgement Header for confirmation.

Table 183z—RS_Member_List_Update message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RS_Member_List_Update_Message_Form at { |  |  |
| Management Message Type = 91 | 8 bits |  |
| Frame Action Number | 4 bits | The 4-bits LSB of the frame number at the superordinate station when the configuration shall be applied by all RS group members |
| Configured_para_type | 4 bits | b0 = 1: RS sends acknowledgement if the message is received b0 = 0: RS does not send acknowledgment if the message is received b1 = 1: data forwarding on a per CID basis b1 = 0: RSs in the group shall forward all data b2 – b3: reserved |
| If (b0 of Configured_para_type == 1 ) { |  |  |
| Transaction ID | 16 bits |  |
| } |  |  |
| If (b1 of Configured_para_type == 1 ) { |  |  |
| Number_RS | 5 bits | Number of RS group members involved with the CID list update |
| Reserved | 3 bits |  |
| for (i=0;i<Number_RS;i++) { |  |  |
| CID | 16 bits | Basic CID of the RS group member |
| N_CID_Add_designated | 8 bits | Number of connections for which the RS is the designated RS. If Mode 2 is used this value is zero. |
| N_CID_Add_non_designated | 8 bits | Number of connections for which the RS is a non-designated RS. |
| for (j=0;j<N_CID_Add_designated;j++) { |  |  |
| CID | 16 bits | For this basic CID of MS the RS is designated. Irrespective of the current RS assignment for a given CID, the indicated RS would be a designated RS for this CID and shall forward data and measurements. |
| } |  |  |
| for (j=0;j<N_CID_Add_non_designated;j++) { |  |  |
| CID | 16 bits | For this basic CID of the MS the RS is non-designated. Irrespective of the current RS assignment for a given CID, the indicated RS would be a non-designated RS for this CID and shall forward only traffic data. |
| } |  |  |
| N_CID_Remove | 8 bits | Number of CIDs to be removed from the forwarding list |
| for (j=0;j<N_CID_Remove;j++) { |  |  |
| CID | 16 bits | MS basic CID to be removed. |
| } |  |  |
| } |  |  |
| TLV Encoded Information | variable | TLV specific |
| } |  |  |

The RS_Member_List_Update message may include the following parameter encoded as TLV-tuples:

HMAC/CMAC Tuple (see 11.1.2)

[Change the section 6.3.22.5.2 as following.]

6.3.22.5.2 MS movement among access stations which share same preamble/FCH/MAP

In this case, MS is not aware of the HO. Therefore,  the stations that share the same preamble/FCH/MAP shall perform measurement of MS signal quality to assist MS movement among stations .

The stations  shall measure the signal quality (RSSI, CINR),  Timing Adjustment (TA), power and Frequency Adjustment (FA) for each active MS served by these stations to support MS mobility among these stations. All RSs shall use RS_MOBMEASR-REP to provide MR-BS with the selected report metrics (RSSI and/or CINR, TA, FA and power adjustments) for each active MS when needed.

Two modes of operation are described in the subclauses 6.3.22.5.2.1 and 6.3.22.5.2.2. The mode of operation and related reporting parameters are configured in RS_Config-CMD in subclause 6.3.2.3.67.

MR-BS may select a new designated RS based on the measurement results and use RNG-RSP to adjust the timing and the power level of the MS, in order to fulfill the handover procedure. MR-BS may use RS_Member_List_Update message defined in subclause 6.3.2.3.84 to notify the RSs of the changes regarding data forwarding status for the specified MSs.

6.3.22.5.2.1 Mode 1

For this mode of operation, only those RSs that are marked as designated RS shall automatically report the measurement results to MR-BS in an event-triggered or periodic way, for the basic CID of MSs provided in the MR_Member_List_Update.

For event-triggered reporting, the designated RS shall report its measurement results if at least one of the power, CINR, frequency or timing requirement for the specific MS is not satisfied.

For periodic reporting, the designated RS shall send RS_MOBMEASR-REP every REP_INT which is specified in RS_Config-CMD message and the MR-BS shall periodically allocate uplink resource for the designated RS to report the latest measurement result for each active MS.

In Mode 1, non-designated RSs shall report their measurement results only if MOB_RSSCN-RSP message is received. The MR-BS shall send MOB_RSSCN-RSP message to request all or part of RSs in the same RS group to report their measurement results for a specific MS. The MR-BS shall allocate uplink resource for the selected non-designated RSs to send their RS_MOBMEASR-REPs at the frame specified in MOB_RSSCN-RSP.

6.3.22.5.2.2 Mode 2

For this mode of operation an RS may report the measurement results using the RS_MOBMEASR-REP to MR-BS in an event-triggered way as indicated below.

If the MR-BS provides the thresholds values for triggering, the RS shall send the measurement report to MR-BS in the following instances:

- When the measured RSSI/CINR  crosses above RSSI/CINR_T_ADD[i] (i=0,...,max);

- When the measured RSSI/CINR crosses below the RSSI/CINR_T_DEL[i] (i=0,...,max) ;

- When the current measured TA exceeds TA_DIFF and the RS has the CID of the MS included in the RS_Member_List_Update message. 
If RS_Member_List_Update message is not provided, then an RS may send the measurement report whenever the current measured TA exceeds TA_DIFF.

If the MR-BS does not provide the thresholds values for adding/removing an MS, the RS uses its own threshold values to decide when to report the measurement to MR-BS.