| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | UL HARQ for centralized scheduling using data tunneling |  |
| Date Submitted | 2007-09-09 |  |
| Source(s) | Adrian Boariu, Shashikant Maheshwari, Haihong Zheng, Yousuf Saifullah, Peter Wang NSN Aik Chindapol, Yishen Sun, Jimmy Chui Siemens Corporate Research | Voice: 972 894 5000 E-mail: adrian.boariu@nsn.com Voice: +1 609 734 3364 Fax: +1 609 734 6565 aik.chindapol@siemens.com |
|  | Gamini Senarath, Israfil Bahceci Nortel | gamini@nortel.com |
| Re: | This is in response for call for comments P802.16j/D1 |  |
| Abstract | This contribution proposes a procedure for handling UL HARQ in a relay system that operates under centralized scheduling with tunneling. |  |
| Purpose | Review and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# UL HARQ for centralized scheduling using data tunneling

# Introduction

The current specifications of the baseline document for RSs does not provide solutions for UL HARQ reporting under the centralized scheduling with data being tunneled from the MR-BS to MSs via RSs.

# Specific text changes

[Insert new sub clause 6.3.17.5.3]

6.3.17.5.3 UL HARQ for tunnels under centralized scheduling mode

An UL tunnel has its beginning at the access RS for the corresponding MSs that are part of the tunnel, while the end of the tunnel is at the MR-BS. The access RS shall pack into the tunnel only those MPDUs that have been successfully received from the corresponding MSs. The UL transmission across the hops follows the instructions provided in the section 6.3.17.5.1 with exception that the MR-BS may not allocate bandwidth for individual ACK/NAK channels for the subbursts from the MSs whose packets will be forwaded over the tunnel connection. Additionally, MR-BS shall allocate ACK/NAK channels for the tunnel packets between the superordinate station of the access RS and MR-BS. Upon receiving correctly the tunneled MPDU, the MR-BS determines those MPDUs that have been included and those MPDUs that failed on the access link. Subsequently, MR-BS may schedule the retransmission of those MPDUs that have failed on access link.

Optionally, MR-BS may configure additional HARQ ACK/NAK channels from access RS all the way back to MR-BS to report the receiving status of MS’s MPDU via Aggregated-HARQ ACK region allocation IE. This option allows the MR-BS to reduce the latency in scheduling the retransmission of corresponding HARQ data sub-burst on the access link, by providing the MR-BS with faster reports of failed access links in the case that the tunnel fails on any intermediate RS. The convention used for aggregation procedure in this situation is the same as that described in section 6.3.17.4.4; in the aggregated report, access RS sets for an MS the corresponding bit on zero if the MPDU has been received successful and on one if the MPDU was received in error.

[Insert in the Table DDD (contribution C80216j-07/403) on section 6.3.17.4.4 before the Padding the following lines:]

| N_CID_UL | 8 bits | Number of UL T-CIDs that are served by this region. |
| --- | --- | --- |
| For (i = 0; i < N_CID_UL; i++) { |  |  |
| RCID_IE() | Variable | Tunnel CID |
| N_ACK_channels | 8 bits | No. of aggregated HARQ ACK channels that are allocated to RS to transmit the reception status report of MS’s MPDUs. |
| } | - | - |