| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | CQICH Reporting in Centralized Scheduling |  |
| Date Submitted | 2008-01-14 |  |
| Source(s) | Shashikant Maheshwari, Adrian Boariu, Yousuf Saifullah, Haihong Zheng, Peter Wang, Aik Chindapol NSN | E-mail: shashi.maheshwari@nsn.com |
| Re: | This is in response for call for comments on P80216j/D2 |  |
| Abstract | Efficient CQICH reporting for optimum scheduling and reducing overhead in MR system |  |
| Purpose | Review and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# CQICH reporting in Centralized scheduling

Shashikant Maheshwari, et al.

NSN

# Introduction

In centralized scheduling, MR-BS controls and schedules the resources on relay and access link. In order to make efficient scheduling, MR-BS requires knowledge of channel qualities of relay links and access links. In IEEE p802.16j/D2, no mechanism is defined for CQI parameter reporting therefore, MR-BS cannot implement centralized scheduling efficiently. Therefore need some mechanism to fix this issue.

# Proposed Remedy

In IEEE 802.16-2004 and 802.16e-2005, CQICH channel is defined to provide channel quality feedback. In general, CQI feedback requires 1 UL OFDM Slot for reporting. Extending same scheme over multi-hop will introduce significant overhead. This contribution proposes bandwidth efficient CQI parameter reporting, which is based on full and differential CQI reporting. RS uses feedback header as defined in section 6.3.2.1.2.2.1 of IEEE 802.16Rev2/D2 for CQI reporting. Differential CQI parameter reporting will require fewer resources therefore multiple MS can be mapped on a feedback header.

# Specification changes

[Insert new sub clause 8.4.5.4.10.16 ]

8.4.5.4.10.16 Fast feedback reporting in MR system

In multi-hop relay system with centralized scheduling, MR-BS may configure RS to report on the relay link, either full or differential fast feedback reporting using feedback header defined in section 6.3.2.1.2.2.1. Differential fast feedback reporting uses fewer bits; therefore RS can map multiple MS’s fast feedback information on a feedback header defined by Figure 33.

Differential feedback reporting can be used to report either physical CINR feedback or effective CINR feedback. If differential feedback reporting is performed on effective CINR, then RS will perform the differential operation based on the “label” of Table 448.

If access RS receives the special codeword (e.g. indication flag feedback, MIMO mode feedback or AMC mode transition) on the CQICH channel or Feedback Header from an MS, then access RS shall not perform differential operation to the special codeword and send special codeword indication to the MR-BS. Access RS stores the received special codeword and forwards the latest received special codeword to MR-BS when MR-BS requests the full feedback reporting. When CINR codeword is followed by special codeword, differential operation should be resumed. When MR-BS receives special codeword indication, MR-BS may request full feedback reporting from an RS. MR-BS may also request full feedback reporting periodically.

MR-BS may allocate MR-Fast-Feedback region on the relay link for forwarding MS CQI parameters reports using MR-Fast-Feedback region allocation IE as defined in Table xxa.

Access RS performs the differential feedback reporting on behalf of MS. Access RS extracts the MSs fast feedback channel information from CQICH_Allocation_IE or CQICH_enhanced_allocation_IE transmitted in UL-MAP. When access RS performs the differential feedback reporting, it uses N_agg field of MR-Fast-Feedback region allocation IE for aggregating differential feedback reporting.

The 32-bits payload of “feedback header without CID field” defined in Figure 24 is constructed by aggregating the payload bits from Table xxb or Table xxc, starting from the most significant bits to least significant bits. The payloads b10 and b100, respectively, are used to indicate that the access RS received special codewords from the MS. The access RS aggregates the payloads of the corresponding MSs in the 32-bits feedback header in the UL relay zone, starting with the MSB and ending with LSB. The aggregation is performed in the order of the feedback information received in the UL access zone by the RS. Dummy payload bits corresponding to dummy MS are set to zeros, and these bits may be used to fill the payload of the feedback header.

Table xxb: 3 bit Differential Fast feedback encoding per differential CQI report with 8 steps of quantization

| 3 bit differential CQI parameter reporting |  |
| --- | --- |
| Quantization step | Payload bits |
| +3 | 011 |
| +2 | 010 |
| +1 | 001 |
| 0 | 000 |
| -1 | 101 |
| -2 | 110 |
| -3 | 111 |
| Special codeword indication | 100 |

Table xxc : 2 bit differential feedback encoding per differential CQI report with 4 steps of quantization

| 2 bit differential CQI parameter reporting |  |
| --- | --- |
| Quantization step | Payload bits |
| +1 | 01 |
| 0 | 00 |
| -1 | 11 |
| Special codeword indication | 10 |

Access RS stores the last received full feedback information of the MSs and performs the differential feedback reporting based on received feedback information and last stored full feedback information. Access RS quantize the difference into either 8 or 4 steps according to Tables xxb or xxc, respectively. If the difference of the two full feedback information is outside the range of supported number of quantization steps, then RS reports the closest quantization step for differential feedback reporting.

When access RS is requested to transmit differential feedback reporting, it transmits the differential feedback information to super-ordinate RS according to the same order in which it receives the fast feedback reporting from the MSs in the fast feedback region of the UL access zone.

Intermediate RSs simply forward to super-ordinate station, the fast feedback information received on the relay zone.  When RS forwards the fast feedback information to super-ordinate station, it puts the differential fast feedback information corresponding to its access zone before the fast feedback information received on the relay zone.

[Insert new sub clause 8.4.5.10.1.10 at line 50 p.220]

8.4.5.10.1.10 MR-Fast-Feedback region allocation IE

MR-BS may allocate MR-Fast-Feedback region on the relay link for forwarding MS CQI parameters reports using MR-Fast-Feedback region allocation IE as defined in Table xxa.

Table xxa- MR-Fast-Feedback region allocation IE format

| Syntax | Size (in bits) | Notes |
| --- | --- | --- |
| MR-Fast-Feedback region allocation IE { |  |  |
| Type | 5 | 02 |
| Length | 4 | Length in bytes |
| OFDMA Symbol offset | 8 | - |
| Subchannel offset | 7 | - |
| No. OFDMA symbols | 7 | - |
| No. subchannels | 7 | - |
| N_CID | 4 | Number of RS CID |
| For(i=0; i < N_CID; i ++) { | - | - |
| Full_differential_feedback_reporting | 1 | 0 = full feedback reporting 1 = differential feedback reporting |
| If(full_differential_feedback_reporting = 1) { | - | - |
| N_agg | 1 | 0 = 3 bits differential feedback 1 = 2 bits differential feedback |
| } |  |  |
| RCID_IE() | Variable | Reduced CID of RS |
| Offset | 8 | Starting position in the fast feedback region where RS reports the MS’s fast feedback information |
| } | - | - |
| } | - | - |

[insert following line before reserved field of table 496c p. 209]

| 09 | MR-Fast-feedback region allocation IE |
| --- | --- |

[modify Table 17 on page 65 line 59 of IEEE 802.16REV2/D2 as following]

| 1110 | RS aggregates CQI parameters of MSs on the relay link. Full CQICH reporting – N*5 bits Differential CQICH reporting – if N_agg = 0 the M*3 bits if N_agg = 1 then P*2 bits N_agg is defined in MR-Fast-Feedback region allocation IE | MSs CQICH codewords relayed by RS. |
| --- | --- | --- |
| 1110-1111 | Reserved |  |