| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | UL Access Zone CQI Reporting in Centralized Scheduling |  |
| Date Submitted | 2007-09-09 |  |
| Source(s) | Yuefeng Zhou, Nader Zein Sampath Rangarajan; Karthiks Sundaresan, Seung-jun Kim | E-mail: Yuefeng.zhou@ttd.neceur.com |
| Re: | This is in response for IEEE 802.16 Working Group Letter Ballot #28: Announcement, 80216j-07_043.pdf |  |
| Abstract | UL access zone CQI reporting is not specified for centralized scheduling. |  |
| Purpose | Review and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# UL Access Zone CINR Reporting in Centralized Scheduling

Yuefeng Zhou, Nader Zein

Sampath Rangarajan; Karthiks Sundaresan, Seung-jun Kim

# Introduction

In legacy IEEE802.16e, BS can directly measure the uplink CQI for each MS to facilitate the radio resource control and scheduling.  However, in multi-hop relay networks, the MR-BS could not measure the CQI in UL access zone for the MS connecting to RS in centralized scheduling. Some mechanisms are needed to fix this issue.

# Proposed Remedy

MR-BS sends a UL Access Zone CQI reporting request MMSG, RS_UL_Access_REP_REQ, to subordinate RS.  The RS will report the CQI for the MSs listed in the request message by reply RS_UL_Access_REP_RSP MMSG.

# Specification changes

[Modify Table 38 as bellow]

Table 38—MAC management messages

Table 38 MAC management message

| Type | Message name | Message description Connection | Connection |
| --- | --- | --- | --- |
| 97-255 | Reserved |  |  |
| 97 | RS_UL_Access_REP-REQ | UL Access Zone Channel Measurement Request | TBD |
| 98 | RS_UL_Access_REP-RSP | UL Access Zone Channel Measurement Response | TBD |
| 99-255 | Reserved |  |  |

[Insert new sub clause after 6.3.2.3.89]

6.3.2.3.90 UL Access Zone Channel Measurement Request (RS_UL_Access_REP-REQ)

In multi-hop relay system with centralized scheduling, MR-BS may send RS_UL_Access_REP-REQ message with RS basic CID to configure RS to report the channel measurement over UL Access Zone for one or more MSs.  The RS_UL_Access_REP-REQ message is defined in Table xxa.

Table xxa- RS_UL_Access_REP-REQ message format

| Syntax | Size (in bits) | notes |
| --- | --- | --- |
| RS_UL_Access_REP-REQ format { |  |  |
| Management Message Type | 8 |  |
| Transaction ID | 16 |  |
| Typp of CQI Measurement | 8 | Bit 0:1 = Physical CINR Bit 1:1 = Effective CINR Bit 2-5: = Average Alpha in multiples of 1/32 (range [1/32, 16/32]) Bit 6-7: reserved |
| N_MS | 8 | Number of MSs |
| For (i=0; i<N_MS; i++) { |  |  |
| RCID_IE() | variable | Reduced MS CID |
| } |  |  |

Transaction ID:  Unique identifier set by the sender for identifying this transaction.

[Insert new sub clause after 6.3.2.3.90]

6.3.2.3.91 UL Access Zone Channel Measurement Response (RS_UL_Access_REP-RSP)

In multi-hop relay system with centralized scheduling, RS shall response the RS_UL_Access_REP-REQ message to by sending a RS_UL_Access_REP-RSP message, which is defined in Table xxb.

Table xxb- RS_UL_Access_REP-RSP message format

| Syntax | Size (in bits) | notes |
| --- | --- | --- |
| RS_UL_Access_REP-RSP format { |  |  |
| Management Message Type | 8 |  |
| Transaction ID | 16 |  |
| For (i=0; i<N_MS; i++) { |  |  |
| CINR Report | varable |  |
| } |  |  |

CINR Report:  The value of this field is specified in the table xxc:

Table xxc- Value of CINR report field in RS_UL_Access_REP-RSP message

| Typp of CQI Measurement in RS_UL_Access_REP-REQ message | Size (in bits) | notes |
| --- | --- | --- |
| Bit 0:1 = Physical CINR | 16 | 1 byte average CINR value 1 byte standard deviation |
| Bit 0:2 = Effective CINR | 24 | 1 byte: a 1 byte: b 1 byte: c |

Transaction ID:  Unique identifier set by the sender for identifying this transaction.

[Insert new sub clause after 8.4.11.4]

8.4.11.5 UL Access Zone CINR Reporting

In multi-hop relay system with centralized scheduling, MR-BS may request RS to report the CINR value of the MSs connecting to the RS by sending  RS_UL_Access_REP-REQ message on RS’ basic CID.  RS  shall send RS_UL_Access_REP-RSP as the response.