| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | UL Access Zone CQI Reporting in Centralized Scheduling |  |
| Date Submitted | 2007-09-17 |  |
| Source(s) | Yuefeng Zhou, Nader Zein Sampath Rangarajan; Karthiks Sundaresan, Seung-jun Kim Chie Ming Chou, Tzu-Ming Lin, Fang-Ching Ren, Wern-Ho Sheen, I-Kang Fu ITRI/ NCTU | E-mail: Yuefeng.zhou@ttd.neceur.com chieming@itri.org.tw |
| Re: | This is in response for IEEE 802.16 Working Group Letter Ballot #28: Announcement, 80216j-07_043.pdf |  |
| Abstract | UL access zone CQI reporting is not specified for centralized scheduling. |  |
| Purpose | Review and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# UL Access Zone CINR Reporting in Centralized Scheduling

Yuefeng Zhou, Nader Zein

Sampath Rangarajan; Karthiks Sundaresan, Seung-jun Kim

# Introduction

In legacy IEEE802.16e, BS can directly measure the uplink CQI for each MS to facilitate the radio resource control and scheduling.  However, in multi-hop relay networks, the MR-BS could not measure the CQI in UL access zone for the MS connecting to RS in centralized scheduling. Some mechanisms are needed to fix this issue.

# Proposed Remedy

MR-BS sends a UL Access Zone CQI reporting request MMSG, RS_UL_Access_REP_REQ, to subordinate RS.  The RS will report the CQI for the MSs listed in the request message by reply RS_UL_Access_REP_RSP MMSG.   MR-BS may also allocate CQICH to the access RS to report the access UL CINR for each MS.

# Specification changes

[Modify Table 38 as bellow]

Table 38—MAC management messages

Table 38 MAC management message

| Type | Message name | Message description Connection | Connection |
| --- | --- | --- | --- |
| 97-255 | Reserved |  |  |
| 97 | RS_UL_Access_REP-REQ | UL Access Zone Channel Measurement Request | TBD |
| 98 | RS_UL_Access_REP-RSP | UL Access Zone Channel Measurement Response | TBD |
| 99-255 | Reserved |  |  |

[Insert new sub clause after 6.3.2.3.89]

6.3.2.3.90 UL Access Zone Channel Measurement Request (RS_UL_Access_REP-REQ)

In multi-hop relay system with centralized scheduling, MR-BS may send RS_UL_Access_REP-REQ message with RS basic CID to configure RS to report the channel measurement over UL Access Zone for one or more MSs.  The RS_UL_Access_REP-REQ message is defined in Table xxa.

Table xxa- RS_UL_Access_REP-REQ message format

| Syntax | Size (in bits) | notes |
| --- | --- | --- |
| RS_UL_Access_REP-REQ format { |  |  |
| Management Message Type | 8 |  |
| Transaction ID | 16 |  |
| Type of CQI Measurement | 8 | Bit 0:1 = Physical CINR Bit 1:1 = Effective CINR Bit 2-5: = Average Alpha in multiples of 1/32 (range [1/32, 16/32]) Bit 6-7: reserved |
| N_MS | 8 | Number of MSs |
| For (i=0; i<N_MS; i++) { |  |  |
| RCID_IE() | variable | Reduced MS CID |
| } |  |  |

Transaction ID:  Unique identifier set by the sender for identifying this transaction.

[Insert new sub clause after 6.3.2.3.90]

6.3.2.3.91 UL Access Zone Channel Measurement Response (RS_UL_Access_REP-RSP)

In multi-hop relay system with centralized scheduling, RS shall response the RS_UL_Access_REP-REQ message to by sending a RS_UL_Access_REP-RSP message, which is defined in Table xxb.

Table xxb- RS_UL_Access_REP-RSP message format

| Syntax | Size (in bits) | notes |
| --- | --- | --- |
| RS_UL_Access_REP-RSP format { |  |  |
| Management Message Type | 8 |  |
| Transaction ID | 16 |  |
| For (i=0; i<N_MS; i++) { |  |  |
| CINR Report | varable |  |
| } |  |  |

CINR Report:  The value of this field is specified in the table xxc:

Table xxc- Value of CINR report field in RS_UL_Access_REP-RSP message

| Typp of CQI Measurement in RS_UL_Access_REP-REQ message | Size (in bits) | notes |
| --- | --- | --- |
| Bit 0:1 = Physical CINR | 16 | 1 byte average CINR value 1 byte standard deviation |
| Bit 1:1 = Effective CINR | 8 | 1 byte effective CINR value |

Transaction ID:  Unique identifier set by the sender for identifying this transaction.

[Insert new sub clause 8.4.5.4.10.17]

8.4.5.4.10.17 Fast Access Uplink feedback reporting in MR system

In multi-hop relay system with centralized scheduling, MR-BS may configure access RS/RSs to report Access UL CINR on pilot subcarrier for each MS on the relay link.

MR-Access-UL-CQICH Alloc IE(), as defined in Table xxd, is introduced to dynamically allocate or deallocate a CQICH to one or multiple access RSs. Once allocated, the RS transmits channel quality information on the assigned CQICH on subsequent frames until the RS receives a MR-Access-UL-CQICH Alloc IE () to deallocate the assigned CQICH .

Table xxd- MR-Access-UL-Fast-Feedback region allocation IE format

| Syntax | Size (in bits) | notes |
| --- | --- | --- |
| MR-Access-UL-CQICH Alloc IE { |  |  |
| Type | 5 | 02 |
| Length | 4 | Length in bytes |
| OFDMA Symbol offset | 8 | - |
| Subchannel offset | 7 | - |
| No. OFDMA symbols | 7 | - |
| No. subchannels | 7 | - |
| N_RS_CID | 8 | Number of RS CIDs |
| For(i=0; i < N_RS_CID; i ++) { | - | - |
| RS_RCID_IE() | Variable | Reduced CID of RS |
| Offset | 8 | Starting position in the fast feedback region where RS reports the CINR values |
| N_MS | 8 | Number of MSs, whose CINR values will be reported by RS |
| For (j=0; j<N_MS; j++) { |  |  |
| RCID_IE() | variable | Reduced MS CID, which is used to indicate the MSs’ UL CINR to be reported by the Access RS. |
| } |  |  |
| Period (p) | 2 | A CQI feedback is transmitted on the CQICH every 2p frames. |
| Duration(d) | 3 | A CQI feedback is transmitted on the CQI channels indexed by the CQICH_ID for 10 x 2d frames. If d == 0, the CQICH is deallocated. If d == 0b111, the RS should report until the MR-BS command for the RS to stop. |
| Feedback Type | 2 | 0b00: Physical CINR feedback 0b01: Effective CINR feedback 0b10–0b11: Reserved |
| If (Feedback Type = 0b00) { |  | Physical CINR feedback |
| Averaging parameter included | 1 | 1: included; 0: not included |
| If (Averaging parameter included == 1) { |  |  |
| Averaging parameter | 4 | Averaging parameter alpha used for deriving physical CINR estimates reported through CQICH. This value is given in multiples of 1/16 in the range of [1/16..16/16] in increasing order. |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Number of bits required to align to byte length, shall be set to zero. |
| } |  |  |

MR-BS uses RS_RCID_IE() field to identify the CQICH allocations for each access RS.

[Insert new sub clause after 8.4.11.4]

8.4.11.5 UL Access Zone CINR Reporting

In multi-hop relay system with centralized scheduling, MR-BS may request RS to report the CINR value of the MSs connecting to the RS by sending  RS_UL_Access_REP-REQ message on RS’ basic CID.  RS  shall send RS_UL_Access_REP-RSP as the response. The CINR value of Access Uplink will be used for link adaptation.  The RS shall derive the mean CINR based on the equation 203 (8.4.11.3) from pilot subcarriers.

The MR-BS may also allocate CQICH by sending MR-Access-UL-CQICH Alloc IE() to conduct one or multiple access RSs to report CINR values of the MSs connecting to them.  Once received MR-Access-UL-CQICH Alloc IE(), the access RS shall report MS’s UL mean CINR based on the equation 203(8.4.11.3) from pilot subcarriers.

[insert following line before reserved field of table 496c]

| 02 | MR-Access-UL-CQICH Alloc IE |
| --- | --- |