| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAP and Data Association in RS for Centralized Scheduling |  |
| Date Submitted | 2008-01-08 |  |
| Source(s) | Mary Chion, Jerry Chow ZTE USA Inc Hongyun Qu, Yuqin Chen ZTE Corp | E-mail: mchion@zteusa.com |
| Re: | IEEE 802.16-07/059: IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement |  |
| Abstract | This contribution proposes methods to provide mapping between bandwidth allocation from MAP messages to MAC PDUs at RS when centralized scheduling is used. |  |
| Purpose | Discuss and adopt proposed text in TG16j |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAP and Data Association in RS for Centralized Scheduling

Mary Chion, Jerry Chow, Hongyun Qu, Yuqin Chen

ZTE

# Introduction

As defined in current draft 16j/D2, when centralized bandwidth allocation is used, the MR-BS shall determine the bandwidth allocations for all links (access and relay) in its MR-cell. For non-transparent RS, the MR-BS performs bandwidth allocations for all DL and UL links and sends scheduling result in MAP message to each RS via RS’ basic connection. Data packets are relayed through the network using the bandwidth allocation over transport connections or tunnel connection. Hence, MAP messages and data packets using the bandwidth allocations are received by the RS through different connections asynchronously. A mapping (reference) is needed between bandwidth allocation and MAC PDUs to allow the RS transmitting the intended MAC PDUs using a specific allocation.

This contribution proposes methods to provide the mapping between bandwidth allocation and MAC PDUs to be used by RS to determine the intended MAC PDUs for each allocation.

# Problem Statement

As defined in the current draft document, MAP messages and data packets using the bandwidth allocations are received by the RS through different connections asynchronously. Except when tunnel packet mode is used, relay MAC PDUs are relayed through the relay links instead of data burst. In addition, each data burst allocation can contain multiple MAC PDUs of different connections. To further explain the operation of MAC PDU relaying, an example is provided using CID based data forwarding Mode. Figure 1 shows an example of 3-hops relay.

BS

RS1

RS2

MS4

MS1

MS2

MS3

Figure 1 Example of 3 hops Relay

Figure 2 shows an example of downlink MAC PDU relaying through the relay network illustrated in Figure 1. The color boxes represent the MAC PDUs designated to different MSs with respective color. The grey box represents a data burst allocation. In this example, it is shown that when transmitted over another relay link or access link, MAC PDUs from different frames can be re-assembled into one data burst, or MAC PDUs concatenated within one data burst can be re-bundled with other MAC PDUs. Figure 2 also shows that one data burst can be partitioned into several data bursts when forwarded. Additonally, data burst can be simply transferred without any reassembling or partitioning.

Figure 2. Example of downlink MAC PDU relay

As described in the example above, the following parameters need to be identified to allow RS determine the intended MAC PDU for each allocation:

- The frame index when the data burst is received;

- The index of MAP IE associated with the received burst;

- The CID of MAC PDUs in the received burst;

According to different data forwarding modes, some parameters may be not required.

# Proposed Solutions

There are multiple transmission options included in the draft document, namely, tunnel packet mode, tunnel burst mode, burst mode and station CID based mode. There exists an allocation subheader used for downlink relay link transmission using tunnel packet mode. In this contribution, we propose the solution to address the transmission using station CID mode, and uplink transmission using tunnel packet mode. The methods will be used to provide MAP and data referencing.

Station CID mode, Uplink Tunnel Packet Mode

This method is defined for transmission without Allocation Subheader in relay MAC header. The method is defined as following:

- DL Allocation Reference IE is defined in the format of DL-MAP extended-2 IE

  - DL Allocation Reference IE is used to include a reference to MAC PDUs for each allocation and should follow its associated DL-MAP-IE immediately. The associated DL-MAP-IE includes data burst allocation for which DL Allocation Reference IE is providing a reference to the MAC PDUs concatenated in the data burst.

  - DL Allocation Reference IE contains the following parameters per connections included in data burst:

- Num_Received Frames: The number of frames where RS receives MAC PDUs, this field enables RS to assemble MAC PDUs from different frames into one single burst.

- Received Frame Index: LSBs of Frame number where the MAC PDUs should be received by the RS.

- Num_MAP IEs: The number of bursts where RS receives MAC PDU in one received frame. This field enables RS to assemble MAC PDUs from different bursts into one single burst.

- MAP IE index: indicates the received burst.

- All_Connections: Identify whether all MAC PDUs from the data burst would be assembled into one single burst.

- Num_CIDs: The number of connections included in the allocation.

- CID: Identity the connection that is transmitting. It can be MS CID, RS management CID, T-CID and MT-CID.

- Total Data Allocated: Total data allocated for this connection, in unit of bytes.

  - RS should store the received frame index and the MAP IE index for each MAC PDU and use the stored information to match what’s included in DL Allocation Reference IE.

  - If RS cannot find any MAC PDU that matches the DL Allocation Reference IE, it should ignore the DL allocation.

  - DL Allocation Reference IE should be included in both R-MAP and DL-MAP used by RS.

  - RS removes DL Allocation Reference IEs from MAP message before broadcasting in access zone or relay zone.

- UL Allocation Reference IE is defined in the format of UL-MAP extended-2 IE

  - UL Allocation Reference IE is used to include a reference to MAC PDUs for each allocation and should follow its associated UL-MAP-IE immediately. The associated UL-MAP-IE includes data burst allocation for which UL Allocation Reference IE is providing a reference to the preferred MAC PDUs to be relayed in uplink data burst

  - UL Allocation Reference IE contains the following parameters:

- Num_Received Frames: The number of frames where RS receives MAC PDUs, this field enables RS to assemble MAC PDUs from different frames into one single burst.

- Received Frame Index: LSBs of Frame number where the MAC PDUs should be received by the RS.

- Num_MAP IEs: The number of bursts where RS receives MAC PDU in the received frame. This field enables RS to assemble MAC PDUs from different bursts into one single burst.

- MAP IE index: indicates the received burst.

- All_Connections: Identify whether all MAC PDUs from the data burst would be assembled into one single burst.

- Num_CIDs: The number of connections included in the allocation.

- CID: Identity the connection that is transmitting. It can be MS CID, RS management CID, T-CID and MT-CID.

- Total Data Allocated: Total data allocated for this connection, in unit of bytes.

  - Utilizing UL Allocation Reference IE, RS is able to assemble uplink bursts from subordinate stations, which reduces the overhead of transmitting a large number of UL MAP IEs.

  - RS should store the received frame number and the MAP IE index for each MAC PDU and use the stored information to match what’s included in UL Allocation Reference IE. However, if MAC PDUs that match the UL Allocation Reference IE specification are not sufficient to fill the burst allocation, the RS may fill the remainder of the burst with other MAC PDUs.

  - UL Allocation IE should only be included in R-MAP used by RS.

According to different data forwarding mode, DL/UL Allocation Reference IE can be used for different cases, the application scenario is described as follows.

In CID based data forwarding mode, DL/UL Allocation Reference IE is used in any cases. Such as:

- Forwarding MAC PDUs from different frames in one burst;

- Forwarding MAC PDUs from different bursts in one burst;

- Partitioning big burst into several bursts;

- Assembling several small bursts into one big burst in uplink, which reduces the overhead caused by a large number of MAP IEs;

- Simply forwarding burst without partitioning or assembling.

In uplink transmission of tunnel packet mode, UL Allocation Reference IE is used for:

- Assembling several small bursts into one big burst, which reduces the overhead caused by a large number of MAP IEs;

- Forwarding MAC PDUs from different frames in one burst;

- Forwarding MAC PDUs from different bursts in one burst.

- Simply forwarding burst without partitioning or assembling.

# Proposed Text

[To modify section 6.3.3.8.1 on Page 77 and section 6.3.3.8.2 on Page 78.]

6.3.3.8.1   Transmission using tunnels

[To insert the following sentences at the end of the second paragraph]

When tunnel packet mode is used with centralized scheduling, UL Allocation Reference IE shall be included in R-MAP message sent to RS via broadcast connection and basic connection. RS shall use UL Allocation Reference IE to match the MAC PDUs with corresponding allocation in the MAP message for uplink transmission.

6.3.3.8.2 Transmission using station CID

[To insert the following sentences at the end of this section ]

When CID based mode is used with centralized scheduling, forwarding of MPDU is performed based on the station CID contained in MAC PDU. DL Allocation Reference IE and UL Allocation Reference IE shall be used to match the MAC PDU with corresponding allocation in the MAP message for downlink and uplink transmission.

[Modify section 8.4.5.3.2.2, table 385 on Page 153]

| Extended-2 DIUC (hexadecimal) | Usage |
| --- | --- |
| … | … |
| 0E | AAS_SDMA_DL_IE |
| 0F | Reserve DL Allocation Reference IE |

[Insert section 8.4.5.3.30, table 421c on Page 181]

8.4.5.3.30 DL Allocation Reference IE format

MR-BS may transmit DL_Allocation_Reference_IE when sending DL-MAP and R-MAP messages to RS to

inform RS the bandwidth allocation of RS’ access zone and relay zone.  Each DL Allocation Reference IE is associated with a DL MAP IE with DIUC 0-12.  When included, the DL_Allocation_Reference_IE shall follow the associated DL-MAP-IE immediately. The associated DL-MAP IE includes data burst allocation for which DL Allocation Reference IE is providing a reference to.

DL_Allocation_Reference_IE shall be removed from the MAP messages by RS before broadcasting.

Table 421c – DL Allocation Reference IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| DL Allocation Reference IE () { | - | - |
| Extended-2 DIUC | 4bits | DL Allocation Reference IE == 0xF |
| Length | 8bits | - |
| Num_Received Frames | 4bits | The number of received frames associated to the allocation. |
| For(i=0; i<Num_Received Frames; i++){ | - | - |
| Received Frame Index | 4bits | LSB of frame number where the MAC PDUs are received by the RS. |
| Num_MAP IEs | 4bits | The number of received bursts associated to the allocation. |
| For(j=0; j<Num_MAP IEs; j++){ | - | - |
| MAP IE index | 10bits | The MAP IE index of received burst in received frame |
| All_Connections | 1bit | 0b1:All connections are associated to this allocation. 0b0:Part of connections are associated to this allocation. |
| If(All_Connections == 0){ |  |  |
| Num_CIDs | 4bits | The number of connections in received burst is included in the allocation. |
| For(k=0; k<Num_CIDs; k++){ | - | - |
| CID | 16bits | Connection Identifier |
| Num_MAC PDUs | 9bits | Total data allocated for this connection, in unit of bytes. |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

[Modify table 290c, insert after table 433 on Page 183 in Section 8.4.5.4.4.2]

| Extended-2 DIUC (hexadecimal) | Usage |
| --- | --- |
| 00 | CQICH_Enhanced Allocation_IE |
| 01 | HO_Anchor_Active_UL-MAP_IE |
| 02 | HO_Active Anchor UL MAP |
| 03 | Anchor_BS_switch_IE |
| 04 | UL_sounding_command_IE |
| 05 | Reserved |
| 06 | MIMO UL Enhanced IE |
| 07 | 07 HARQ UL MAP IE |
| 08 | HARQ ACKCH Region Allocation IE |
| 09 | HARQ ACKCH Region for Relay Data IE |
| 0A | MR UL-MAP Monitor IE |
| 0B | MIMO UL Extended IE |
| 0C | UL Allocation reference IE |
| 0D | Reserved |
| 0E | AAS_SDMA_UL_IE |
| 0F | Feedback_polling_IE |

[Insert section 8.4.5.4.32]

8.4.5.4.32 UL Allocation Reference IE format

MR-BS may transmit UL_Allocation_Reference_IE when sending R-MAP messages to RS. Each UL_Allocation_Reference_IE is associated with a UL-MAP-IE with UIUC 0-10. When included, the

UL_Allocation_Reference_IE shall follow the associated UL-MAP-IE immediately. The associated UL-MAP IE

specifies data burst allocation for which UL Allocation Reference IE provides additional information as to

which MAC PDUs should be given preferential use of the allocation.

Table 486e – UL Allocation reference IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| UL Allocation Reference IE () { | - | - |
| Extended-2 UIUC | 4bits | UL Allocation Reference IE == 0xC |
| Length | 8bits | - |
| Num_Received Frames | 4bits | The number of received frames associated to the allocation. |
| For(i=0; i<Num_Received Frames; i++){ | - | - |
| Received Frame Index | 4bits | LSB of frame number where the MAC PDUs are received by the RS. |
| Num_MAP IEs | 4bits | The number of received bursts associated to the allocation. |
| For(j=0; j<Num_MAP IEs; j++){ | - | - |
| MAP IE index | 10bits | The MAP IE index of received burst in received frame |
| All_Connections | 1bit | 0b1:All connections are associated to this allocation. 0b0:Part of connections are associated to this allocation. |
| If(All_Connections == 0){ |  |  |
| Num_CIDs | 4bits | The number of connections in received burst is included in the allocation. |
| For(k=0; k<Num_CIDs; k++){ |  |  |
| CID | 16bits | Connection Identifier |
| Num_MAC PDUs | 9bits | Total data allocated for this connection, in unit of bytes. |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

# Reference

- “Air Interface for Fixed and Mobile Broadband Wireless Access Systems - Multihop Relay Specification”, IEEE 802.16j-06/D2, December 2007

.