| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Optional initial path selection procedure |  |
| Date Submitted | 2007-09-10 |  |
| Source(s) | Yan peng Zheng Shang Zhengzheng Li, Tony Zhang, Shirley Xiao | Voice : +86-21-50993162 Email: peng.yan@huawei.com Voice: +86-21-50993165 Email: shang_zheng@huawei.com Voice: 86-755-28970222 E-mail: lizhengzheng@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | [Cite the specific document number of the appropriate Call for Contributions, the ballot number, etc. Contributions that are not responsive to this section of the template, may be refused or assigned a low priority for review.] |  |
| Abstract | This contribution provides a proposal for speeding up the RS to re-entry the selected station by contention-free. |  |
| Purpose | Adopt the text proposal in this document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Introduction

During the path selection procedure performed prior to “RS operation parameter configuration”, the MR-BS shall determine the path and the RS shall perform network re-entry provided the preamble index assigned in the RS_Path-REQ message is different from that of the current access station.

However, according to the current specification, when the RS performs network re-entry, the delay is long since it needs to perform contention-based initial ranging to the new access station. During this RS network re-entry procedure, MSes which will access the network via this RS also suffer long delay in entry procedure.

This contribution provides a proposal for speeding up the RS network re-entry procedure by ranging to the new access station in non-contention-based manner. The proposal is: in the path selection procedure, the MR-BS may inform the RS of the dedicated ranging parameters of the new access station, so that the RS can use the dedicated ranging parameters for faster ranging in non-contention-based manner.

# Proposed text changes

## 6.3.9.16.3.5 Initial path selection procedure

During this operation, the MR-BS shall determine the path (i.e. access station) of this RS based on the reported neighbour station measurements and other information such as path loading. The MR-BS may obtain parameters (i.e. Rendezvous time, CDMA code, and Transmission opportunity offset ) from its recommended new access station. The MR-BS shall send the RS_Path-REQ message to the RS to indicate the preamble index of the selected access station, and optional parameters (i.e. Rendezvous time, CDMA code, and Transmission opportunity offset ).The RS shall respond with the MR_Generic-ACK message. If the access station indicated in the RS_Path-REQ message is not the access station the RS currently attaches to, the RS shall perform network re-entry as described in 6.3.9.

## 6.3.2.3.82 RS path selection request(RS_Path-REQ) message

| Syntax | Size | notes |
| --- | --- | --- |
| RS_Path-REQ_Message_Format{ |  |  |
| Type=89 | 8 bits |  |
| Configuration_para_type | 8 bits | b0=1: CDMA code b1~b7: reserved |
| Preamble Index | 8 bits | Preamble Index of the access station the RS shall attach to. |
| Transaction ID | 16 bits |  |
| If(b0 of Configuration_para_type ==1){ |  |  |
| Rendezvous time | 8 bits | Units are frames |
| CDMA code | 16 bits | From initial ranging codeset |
| Transmission opportunity offset | 8 bits | Units are transmission opportunity |
| } |  |  |
| } |  |  |