| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A clarification on RS assisted DL HARQ |  |
| Date Submitted | 2007-09-10 |  |
| Source(s) | Zheng Shang Yan peng Xia Yang | Voice: +86-21-50993165 Email: shang_zheng@huawei.com Voice : +86-21-50993162 Email: peng.yan@huawei.com Voice : +86-755-28970209 Email : yangxia@huawei.com |
| Re: | IEEE 802.16j-07/xxx, “Call for Technical Comments Regarding IEEE Project 802.16j.” |  |
| Abstract | This contribution proposes to make it clear that no matter in direct ACK/NACK mode or encoded ACK/NACK mode, RS needs to listen ACK/NACK from MS. |  |
| Purpose | Adopt proposed spec changes in P802.16j Baseline Document (P80216j_D1). |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

RS assisted DL HARQ optimization in direct ack/nack mode

# Introduction

There are two modes in RS assisted DL HARQ, which are direct ACK/NACK mode and encoded ACK/NACK mode. MR-BS can configure the RS which mode to use via Compact DL-MAP MONITOR IE. According to the current specification, it can be deduced that just in encoded ACK/NACK mode, the RS needs to listen ACK/NACK from the MS, and then the RS shall clear the HARQ sub burst depending upon the ACK/NACK information. But in direct ACK/NACK mode, it can be deduced that the RS does not listen ACK/NACK from the MS, so the result is that when the RS clears the sub burst is unclear.

This issue can be shown in figure 1 below.

Figure 1. RS does not listen ACK/NACK from MS in direct ACK/NACK mode

In figure 1, it can be found that, without listening the ACK/NACK from MS, the RS in direct ACK/NACK mode can not clear the burst depending upon the ACK/NACK information.

Here we suggest that in RS assisted HARQ, it shall be made clear that RS shall listen the ACK/NACK from the MS in both direct and encoded ACK/NACK mode. So the RS in direct ACK/NACK mode can clear the saved sub burst depending upon the ACK/NACK information from the MS.

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------

[Adopt the following text modification to the P802.16j baseline document]

6.3.17.4.2.2 RS assisted HARQ

In a case where the MR-BS sends a HARQ sub-burst to the MS directly, the MR-BS informs the RS that it needs to monitor that particular transmission and listen the ACK/NACK from the MS by Compact DL-MAP MONITOR IE and also allocate HARQ ACK region allocation IE on the relay link for sending ACK/NACK from RS. The RS, having information on the downlink resource allocations sent in the DL MAP for the MS and Compact DL-MAP MONITOR IE, monitors the HARQ sub burst transmission sent to MS by MR-BS directly and attempts to decode it. When the RS receives the HARQ sub burst correctly, the RS saves it for a possible retransmission.

When MR-BS receives ACK/NACK from MS directly, MR-BS informs RS to reply ACK/NACK signal after RS receives the HARQ sub-burst. In this case, MR-BS receives ACK/NACK from RS and MS separately. When MR-BS receives NACK from both RS and MS, MR-BS retransmits the HARQ sub-burst. If MR-BS receives ACK from RS and NACK from MS, MS-BS makes the RS retransmits the HARQ sub-burst. If RS monitored the ACK from MS, the RS shall clear the HARQ sub-burst saved.

MR-BS may also configure RS to use encoded ACK/NACK using Compact DL-MAP MONITOR IE. When receiving such IE, the RS listens the ACK/NACK from the MS as well as monitors the DL HARQ sub burst. After the RS receives ACK/NACK from the MS, the RS replies using an encoded ACK/NACK defined in Table xxx through ACK channel prepared by MR-BS. RS shall clear the HARQ sub-burst depending upon the ACK/NACK information received from MS. If the RS received the HARQ sub-burst correctly and receives a NACK from MS, the RS replies the C2 to MR-BS. In this case, the MR-BS requests the RS to retransmit the HARQ sub-burst saved at the RS. When the RS fails to receive the HARQ sub-burst and receives a NACK from the MS, the RS sends a NACK to the MR-BS. Then the MR-BS retransmits the burst by itself. When the RS receives an ACK from MS then irrespective of whether RS receives the HARQ sub-burst correctly or not, the RS replies ACK to the MR-BS. RS will send the encoded ACK/NACK in the UL ACKCH according to the order of CID in the compact DL-MAP MONITOR IE.

---------------------------------------------------------End of the Text-----------------------------------------------