| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments to P802.16j Baseline Document: 6.3.2.3.70 MS scanning inform (MS_SCN-INF) message |  |
| Date Submitted | 2007-09-10 |  |
| Source(s) | Yan peng Zheng Shang Jianjun Yang Ke Zeng Lina Liu | Voice : +86-21-50993162 Email: peng.yan@huawei.com Voice: +86-21-50993165 Email: shang_zheng@huawei.com Email: yougth@huawei.com Email: Zengk@huawei.com Email: liulina@huawei.com |
| Re: | IEEE 802.16j-07/xxx, “Call for Technical Comments Regarding IEEE Project 802.16j.” |  |
| Abstract | This contributions provides comments to the P802.16j-D1 document |  |
| Purpose | Improve the text of section “6.3.2.3.70 MS scanning inform (MS_SCN-INF) message” in IEEE 802.16j-D1. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments to P802.16j Baseline Document: 6.3.2.3.70 MS scanning inform (MS_SCN-INF) message

# Introduction

In “6.3.2.3.49 MOB_SCN-RSP (scanning interval allocation response) message” of P80216Rev2_D0d, the latest 802.16e specification, the Start Frame field has been changed to 8bits. In order to facilitate the coherence of the specification, we propose to modify the Start Frame field in MOB_SCN-RSP message in “6.3.2.3.70 MS scanning inform (MS_SCN-INF) message” of P80216j_D1 to 8bits.

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------

[Adopt the following text modification to the P802.16j baseline document]

6.3.2.3.70 MS scanning inform (MS_SCN-INF) message

A MS_SCN-INF message may be transmitted by an MR-BS to inform an access RS of MS scanning operation. An MR-BS includes the information of scanning intervals of MS(s) in a MS_SCN-INF message. An MR-BS shall generate MS_SCN-INF message in the format shown in Table 183k. The MS_SCN-INF message shall be transmitted on the RS's basic CID.

Table 183k—MS_SCN-INF message format

| Syntax | Size | Notes |
| --- | --- | --- |
| MS_SCN-INF_Message_Format(){ |  |  |
| Management Message Type = 76 |  |  |
| Transaction ID | 16 bits |  |
| N_MS | 8 bits | Number of MSs |
| for (i=0, i< N_MS, i++){ |  |  |
| MS_CID | 16 bits | Basic CID of MS |
| Start frame | 8bits | Start frame number from which the MS start the first scanning interval |
| Scan duration | 8 bits | Duration (in units of frames) where the MSmay perform scanning |
| Interleaving interval | 8 bits | Duration in frames. The period interleaved between scanning intervals when MS shall perform normal operation |
| Scan iteration | 8 bits | The number of iterating scanning interval |
| } |  |  |
| } |  |  |

Insert new subclause 6.3.2.3.71:

---------------------------------------------------------End of the Text-----------------------------------------------