| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Hop by Hop Vs End to End HARQ |  |
| Date Submitted | [The date this revision of the document is uploaded, in the format 2007-06-19. Tell the truth. Automatic numbering is prohibited.] |  |
| Source(s) | Shang Zheng Yan peng Chen Bin | Voice: +86-21-50993165 E-mail: shang_zheng@huawei.com Voice : +86-21-50993162 Email: peng.yan@huawei.com Voice: +86-755-28973349 Email: binchen@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Technical Proposals regarding IEEE Project P802.16j (IEEE 802.16j-07/007r2) |  |
| Abstract | This contribution compare hop by hop HARQ and end to end HARQ in transparent mode RS, and then propose a text changing to adopt end to end HARQ |  |
| Purpose | Add proposed spec changes in P802.16j Baseline Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Hop by Hop vs End to End HARQ

# 1.  Introduction

In DL HARQ for transparent RS with hop-by-hop relaying (6.3.17.4.2.1 of [1]), the retransmission procedure is described as “If the RS does not receive the HARQ sub burst successfully, the RS shall send a NACK signal to the MR-BS. Upon receiving the NACK from the RS, the MRBS shall retransmit the HARQ sub burst to the RS. When HARQ sub-burst is successfully received at RS, MS-BS request RS to transmit HARQ sub-burst.” .

It could be shown as figure 1.

Figure 1 Retransmission in the first hop

It could be found that the MR-BS needs to allocate DL MAP hop by hop. Just after receiving the ACK from RS, the BS allocates the resource for next hop, which will cause extra transmission delay.

If the BS allocates the DL MAP end-to-end (allocate bandwidth over all the links in the path at the same time), when RS receives the burst correctly, it can forward it immediately, which will reduce transmission delay significantly. It is shown in figure 2 below.

Figure 2, end-to-end allocation, no retransmission

The problem of end to end allocation is that when RS receives the burst incorrectly, it has no burst to forward to the MS. But the MS does not know it and will still receive the empty burst, and then it will combine the empty burst with the burst received latterly, which will cause gain degradation in burst decoding. It can be show as figure 2 below.

Figure 3. end to end allocation, retransmission

# 2. Proposed Text Changes

---------------------------------------------------------Start of the Text----------------------------------------------

[Adopt the following text modification to the P802.16j baseline document]

6.3.17.4.2.1 RS hop-by-hop HARQ

When MR-BS or RS sends a HARQ sub burst to MS through RS, the RS shall receive the HARQ sub burst from the MR-BS for relaying the burst to the MS. If the RS receives the HARQ sub burst correctly, then the RS sends an ACK signal to the MR-BS and saves it for the event that there may be a retransmission to MS. Subsequently, the RS forwards the sub burst to the MS. If the RS does not receive the HARQ sub burst successfully, the RS shall send a NACK signal to the MR-BS. Upon receiving the NACK from the RS, the MRBS shall retransmit the HARQ sub burst to the RS. When HARQ sub-burst is successfully received at RS, RS forwards the sub burst to the MS. When the MR-BS receives a NACK from the MS, the MRBS notifies the RS to retransmit the HARQ sub burst to the MS, and the RS shall retransmit the stored correct HARQ sub burst to the MS.

---------------------------------------------------------End of the Text-----------------------------------------------

# 4. Reference

[1]	P802.16j/D1, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems Multihop Relay Specification”