| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Signaling to assign the resource in the UL access zone |  |
| Date Submitted | 2007-11-14 |  |
| Source(s) | Changyoon Oh, Hyunjeong Kang, Jungje Son Samsung Electronics, Suwon, Korea Rakesh Taori Samsung Advanced Institutes of Technology (SAIT) Eugene Visotsky Motorola Labs Shashikant Maheshwari Nokia-Siemens Kerstin Johnsson Intel | E-mail: changyoon.oh@samsung.com |
| Re: | IEEE802.16-07/043”IEEE 802.16 Working Group Letter Ballot #28: Announcement” |  |
| Abstract | This contribution proposes a signaling method for the resource request of access zone. |  |
| Purpose | Discuss and adopt proposed text by TG16j |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Signaling to assign the resource in the UL access zone

Changyoon Oh, Hyunjeong Kang, Jungje Son, Rakesh Taori*

Samsung Electronics, Samsung Advanced Institutes of Technology (SAIT)*

# 1. Introduction

This contribution proposes a signaling method of assigning the resource in the uplink access zone in response to the resource request of uplink access zone from RS, i.e., ‘RS UL size request header’ in subclause 6.3.2.1.2.2.2.6 [1]. ‘RS UL size request header’ proposed by [2] has been accepted in July meeting. However, signaling to assign the resource in response to ‘RS UL size request header’ has not been defined yet. To complete the aforementioned ‘RS UL size request header’, the signaling method of assigning the resource in the UL access zone is proposed in this contribution.

# 2. Proposed solution

To complete the aforementioned operation, i.e., ‘RS UL size request header’, signaling method of resource allocation, i.e., RS UL access zone IE, is proposed.

# 3. Proposed text change

[Insert new subclause 8.4.5.9.1.3:]

8.4.5.9.1.3 RS UL access region zone IE

In the distributed scheduling, MR-BS may transmit RS UL access region zone IE to its subordinate RSs upon receiving RS UL size request header (see 6.3.2.1.2.2.2.6) from one of the subordinate RSs. RS UL access region zone IE provides the allocation for a region of the UL access region zone. The RS whose RS CID appears in RS UL access region zone IE shall allocate that region to its subordinate MSs , whereas MR-BS and other RSs that receive the RS UL access region zone IE shall not allocate that region for their subordinate MSs.

Table xxx – RS_UL-access region zone IE format

| Name | Length | Description |
| --- | --- | --- |
| RS_UL-access region AZONE IE { |  |  |
| Type | 5bits | RS_UL- access region AZONE IE=0x02 |
| Length | 4bits | Variable |
| CID | 16bits | RS basic CID |
| Indicator | 1bit | 1: indicates allocation of resource 0: indicates release of resource Notes: allocation/release is effective in the next frame |
| OFDMA symbol offset | 8bits |  |
| No. OFDMA symbols | 7bits |  |
| Subchannel Offset | 7bits |  |
| No. of subchannels | 7bits |  |
| } |  |  |

Indicator: RS UL access region zone IE with indicator field=1 allocates a region of an UL access region zone to the RS whose CID appears in the RS_UL-access region zone IE. In this case, the RS shall allocate that resource to its subordinate MSs. MR-BS and other RSs that have received RS_UL-access region zone IE shall not allocate that same resource to its subordinate MSs. RS_UL-access region zone IE with indicator field=0 releases the allocated region of an UL access region zone from the RS whose CID appears in the RS_UL-access region zone IE. In this case, MR-BS and its subordinate RSs may allocate the released resource to their MSs autonomously.

[Change Table 496c in page 192 as follows:]

Table 496c – R-link specific IE types

| Type (hexadecimal) | Usage |
| --- | --- |
| 00 | RS_UL_DCH assignment IE |
| 01 | RS_BW-ALLOC IE |
| 02 | RS_UL-access region IE |
| 03-1F | Reserved |

Modify the first sentence in subclause 6.3.2.1.2.2.2.6 as follows::

Under distributed scheduling, an RS may request size for its region of the access uplink zone from the superordinate

RS/MR-BS by sending the RS UL Size Request Header

Insert the following text at the end of subclause 6.3.2.1.2.2.2.6::

The MR-BS may allocate a region of the uplink access region zone in the RS UL access region zone IE (Section 8.4.5.9.1.3).

Insert new subclause 6.3.6.7.1.2.3:

6.3.6.7.1.2.3 UL access region request and allocation in distributed mode with non-transparent RS

A non-transparent RS may request a dedicated access uplink region for its access uplink traffic by sending an RS_UL_Size_Request header to its MR-BS (see  6.3.2.1.2.2.2.6).  Upon assigning a dedicated uplink access region, the MR-BS shall notify the RS by inserting an RS UL_access_region zone_IE in the R-UL-MAP (see 8.4.5.9.1.3).  The MR-BS may allocate to the RS a dedicated access uplink region without receiving a request from the RS.

Reference:

[1] IEEE P802.16j/D1 August 2007

[2] IEEE C802.16j-07/307r1, “UL Access Region Size Request for Semi-Distributed RS”.