| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Acknowledgement of RS Dedicated Uplink Channel (RS_UL_DCH) allocation |  |
| Date Submitted | 2007-09-07 |  |
| Source(s) | Sungcheol Chang Sunggeun Jin Kwangjae Lim Chulsik Yoon ETRI 161, Gajeong-dong, Yuseong-Gu, Daejeon, 305-350, Korea | scchang@etri.re.kr sgjin@etri.re.kr kjlim@etri.re.kr csyoon@etri.re.kr |
| Re: | This is a response to Letter Ballot 16j on P802.16j/D1 |  |
| Abstract | The baseline document defines the Dedicated Uplink Channel (RS_UL_DCH) for RS and its operation of resource allocation for the channel using RS_UL_DCH assignment IE. Due to the the continuous resource allocation characteristics for the RS_UL_DCH, the corruption of the RS_UL_DCH assignment IE in an RS causes continuous losses of dedicated uplink resources. In order to cope with the problem, we provide a proposal to confirm the resource allocation for the dedicated uplink channel. In our proposal, the RS_UL_DCH header is updated from the RS_UL_DCH request header and sent by the RS in the response to the RS_UL_DCH assignment IE. |  |
| Purpose | To incorporate the proposed text into P802.16j Baseline Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Acknowledgement of RS Dedicated Uplink Channel (RS_UL_DCH) allocation

Sungcheol Chang, Sunggeun Jin, Kwangjae Lim, Chulsik Yoon

ETRI

# 1. Introduction

The IEEE 802.16j draft 1.0 contains the definitions of the Dedicated Uplink Channel (RS_UL_DCH) for RS and its operation of resource allocation for the channel using UL_DCH assignment IE. The resources assigned by an RS_UL_DCH keep valid until they are updated by a new RS_UL_DCH. Dedicated Uplink Channel enables an RS to transmit its data with short latency to its superordinate MR-BS/RS, and additionally, it is helpful to save MAP overhead as much as the amount of normal MAP IEs, which could be transmitted in every frame without RS_UL_DCH assignment IE.

However, an MR-BS/RS can broadcast RS_UL_DCH assignment IE to its subordinate RS incorrectly due to a channel error without acknowledgement, thus incurring information mismatch as to the resource allocation between the RS and the MR-BS/RS. In this case, the MR-BS/RS tries to receive data by utilizing the newly updated resources while the RS transmits its data by utilizing the outdated resources. This kind of information mismatch always causes the MR-BS/RS to fail to receive the data transmitted by the RS through the Dedicated Uplink Channel. Moreover, it is impossible for MR-BS/RS to recognize whether the reception failure occurs due to either the information mismatch or channel errors. This problem regarding uplink dedicated channel deteriorates the effectiveness of Dedicated Uplink Channel, and hence results in wasting the involved resources. Figure 1 shows that the case that MR-BS/RS receives corrupt continuous UL data which RS1 relays through the resources assigned by the RS_UL_DCH when RS1 fails to receive the RS_UL_DCH assignment IE updating the dedicated uplink resources because of channel error.

Figure 1 Resource assignment by UL_DCH assignment IE and its acknowldgement.

The solution of the information mismatch problem is described in the following text.

# 2. Proposed solution

The acknowledgement corresponding to the UL_DCH assignment IE is proposed in this contribution. For this purpose, the RS_UL_DCH request header defined in the draft is modified to the RS_UL_DCH header by appending DCH Assignment Acknowledgement (ACK) in the TYPE field of the RS_UL_DCH header.

As discussed earlier, in order to cope with the information mismatch between an superordinate MR-BS/RS and an RS, RS_UL_DCH header with DCH Assignment ACK is proposed to be sent by the RS in response to the RS_UL_DCH assignment IE. For this reason, after receiving the corresponding RS_UL_DCH header with DCH Assignment ACK, the MR-BS completes the resource assignment procedure for the dedicated uplink channel. The allocated resources to the RS are considered to be dedicated even before the RS/MR-BS receives the DCH Assignment ACK. The RS/MR-BS confirms the dedicated resources are successfully assigned to the desired RS when it receives the DCH Assignment ACK. If the RS/MR-BS does neither receive the DCH Assignment ACK nor successfully receive any PDU from the designated RS the acknowledgement response time, it retransmit the RS_UL-DCH assignment IE. Figure 2 shows the procedure that MR-BS broadcasts RS_UL_DCH assignment IEs, which RS1 relays them to RS2. In this figure, RS2 replies with RS_UL_DCH headers with DCH Assignment ACK to the MR-BS via RS1.

Figure 2 Resource assignment by UL_DCH assignment IE and its acknowldgement.

If an RS fails to receive the RS_UL_DCH assignment IE sent by a superordinate MR-BS/RS, the superordinate MR-BS/RS cannot receive the RS_UL_DCH header with DCH Assignment ACK, and hence, it detects incompletion of the procedure to update dedicated resources for the RS_UL_DCH. In this case, it repeats the procedure by sending the RS_UL_DCH assignment IE until successful completion of the procedure. Figure 3 shows the recovery procedure that the RS_UL_DCH assignment IE is sent repeatedly in case that the first RS_UL_DCH assignment IE is lost. RS_UL_DCH assignment IE, the proposed procedure is applied to every Assignment type defined in RS_UL_DCH assignment IE including the case of removal.

Figure 3 Recovery procedure of assignment mismatch by sending RS_UL_DCH assignment IE again.

An RS can recognize the duplicated assignment of the same resource by RS_UL_DCH assignment IEs since those received RS_UL_DCH assignment IEs are identical. The RS shall send RS_UL_DCH header with DCH Assignment ACK when receiving RS_UL_DCH assignment IEs.

Figure 4 The loss of the first PHY burst and recovery.

# 3. Text Proposals

[Change the text at the table 19a in page 10]

Table 19a-Extended Type field encodings for extended MAC signaling header type II

| Extended Type field | MAC header type | Reference figure | Reference table |
| --- | --- | --- | --- |
| 0 | RS BS header | Figure 35b | Table 19b |
| 1 | RS_UL_DCH request header | Figure 35c | Table 19c |
| 2 | Acknowledgement header | Figure 35d | Table 19d |
| 3 | HARQ RS error report header | Figure 35e | Table 19e |
| 4 | MR_Code-REP header | - | Table 19f |
| 5 | RS UL size request header | Figure 35f | Table 19g |
| 6-7 | Reserved |  |  |

[Change the text at the section 6.3.2.1.2.2.2.2 in page 11]

6.3.2.1.2.2.2 RS UL DCH request header (RS_UL_DCH)

The RS requests a dedicated uplink resource through the RS_UL_DCH request header with TYPE=0000, DCH Request. The RS confirms the successful receiving the RS_UL_DCH assignment IE through the RS_UL_DCH header with TYPE=0001, DCH Assignment Acknowledgement (ACK). The format of this header is illustrated in Figure 35c and described in Table 19c.

Figure 35c-RS_UL_DCH request header format

Table 19c-Description of fields in R UL_DCH request header

| Syntax | Size | Notes |
| --- | --- | --- |
| MAC Header() { |  |  |
| HT | 1 bit | Shall be set to 1 |
| EC | 1 bit | Shall be set to 1 |
| Type | 1 bit | Shall be set to 1 |
| Extended TYPE | 3 bits | Shall be set to 001 for RS_UL_DCH request header |
| TYPE | 4 bits | 0000 = DCH Request 0001 = DCH Assignment Acknowledgement (ACK) 00012 – 1111 = Reserved |
| if (TYPE==0000) { |  | DCH Request |
| DCHTYPE | 2 bits | 00 = DCH Request Incremental 01 = DCH Request Aggregate 10 = DCH Request Rate Based 11 = Reserved |
| If (DCHTYPE == 00) { |  | DCH Request Incremental |
| Bandwidth request | 16 bits | Number of bytes requested by the RS. Zero in this field indicated DCH release request. |
| N | 4 bits | Allocation repeats once every N frames. |
| } else if (DCHTYPE == 01) { |  | DCH Request Aggregate |
| Bandwidth request | 16 bits | Number of bytes requested by the RS. Zero in this field indicated DCH release request. |
| N | 4 bits | Allocation repeats once every N frames. |
| } else if (DCHTYPE == 10) { |  | DCH Request Rate Based |
| Average rate | 20 bits | Average data rate in units of bytes per second 18 MSB bits: magnitude 2 LSB bits: base-10 exponent |
| } |  |  |
| RS CID | 8 bits | Reduced Basic CID of RS |
| } else if (TYPE == 0001) { |  | DCH Assignment Acknowledgement (ACK) |
| Frame Number | 8 bits | 8-bit LSBs of the frame in which the RS_UL_DCH assignment IE is received. |
| Update type | 2 bits | This field shall be the same value in the corresponding RS_UL_DCH assignment IE. 00 = Normal 01 = Service flow based 10-11 = Reserved |
| if (Update type == 01) { |  | Service flow based |
| Access RSCID | 8 bits | Reduced basic CID of the access RS of the MS that completed the service flow event. This field shall be the same value in the corresponding RS_UL_DCH assignment IE. |
| } else { |  |  |
| Reserved | 8 bits | Shall be set to 0. |
| } |  |  |
| RS CID | 8 bits | Reduced Basic CID of RS |
| Reserved | 2 bits | Shall be set to 0. |
| } |  |  |
| HCS | 8 bits | Header check squence |
| } |  |  |

[Change the text at the section 6.3.6 in page 79]

Similar bandwidth request methods are available for the RS. These are detailed in subclause 6.3.6.7. In addition, an RS may request a dedicated uplink resource with the RS_UL_DCH request header with TYPE=0000, DCH Request. The RS_UL_DCH header with TYPE=0001, DCH Assignment ACK shall be sent by an RS in response to the RS_UL_DCH assignment IE.

[Insert the following text at the end of the section 6.3.6.7.1.2.2 in page 84]

When successfully receiving an RS_UL_DCH assignment IE in the R-MAP, an RS shall send an RS_UL_DCH header with TYPE=0001, DCH Assignment ACK, in response to the RS_UL_DCH assignment IE to its superordiante MR-BS/RS. The RS_UL_DCH header shall appear in the first position within the PHY burst and the first frame that the dedicated uplink resource updated by the RS_UL_DCH assignment IE is activated. The frame number in the RS_UL_DCH header with DCH Assignment ACK is that of the frame in which the RS_UL_DCH assignment IE is received. The superordinate MR-BS/RS finish updating resources for the dedicated uplink channel by receiving the RS_UL_DCH header with DCH Assignment ACK successfully. If it fails to receive the RS_UL_DCH header with DCH Assignment ACK, the same RS_UL_DCH assignment IE as what it previously transmitted shall be sent again.

In case that an MR-BS removes dedicated uplink channel assigned to a particular RS by sending RS_UL_DCH assignment IE with Assignment type field set to either Remove or Remove all, the RS may send the RS_UL_DCH header with DCH Assignment ACK by utilizing the other resources allocated to it without dedicated uplink channel.

[Insert the following text at the end of the section 6.3.6.7.2.2.2 in page 87]

When successfully receiving an RS_UL_DCH assignment IE, an RS shall send an RS_UL_DCH header with TYPE=0001, DCH Assignment ACK, in response to the RS_UL_DCH assignment IE. The RS_UL_DCH header shall appear in the first position within the PHY burst and the first frame that the dedicated uplink resource updated by the RS_UL_DCH assignment IE is activated. The frame number in the RS_UL_DCH header with DCH Assignment ACK is that of the frame in which the RS_UL_DCH assignment IE is received.

After sending an RSRS_UL_DCH assignment IE to an RS, a MR-BS shall receive the RS_UL_DCH header with DCH Assignment ACK and finish updating resources for the dedicated uplink channel. If there is no response, that is failure to receiving RS_UL_DCH header with DCH Assignment ACK, the RS_UL_DCH assignment IE with same assignment shall be sent again.

If a dedicated uplink resource updated by an RSRS_UL_DCH assignment IE, including the cases that Assignment type is Remove or Remove all, is less than the resource that an RSRS_UL_DCH header can be sent, an RS may send the RS_UL_DCH header with DCH Assignment ACK in other uplink resources except the RS_UL_DCH.

[Change the table at the section 11.7.25 in page 211]

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 43 | 3 | Bit #0: Bandwidth request and UL Tx Power Report header support Bit #1: Bandwidth request and CINR report header support Bit #2: CQICH Allocation Request header support Bit #3: PHY channel report header support Bit #4: Bandwidth request and uplink sleep control header support Bit #5: SN report header support Bit #6: Feedback header support Bit #7-10: SDU_SN extended subheader support and parameter Bit #7: SDU_SN extended subheader support Bit #8-10 (=p): period of SDU_SN transmission for connection with ARQ disabled = once every 2p MAC PDUs Bit #11: DL sleep control extended subheader Bit #12: Feedback request extended subheader Bit #13: MIMO mode feedback extended subheader Bit #14: UL Tx Power Report extended subheader Bit #15: Mini-feedback extended subheader Bit #16: SN request extended subheader Bit #17: PDU SN(short) extended subheader Bit #18: PDU SN(long) extended subheader Bit #19: ACK header Bit#20: RS BR header Bit#21: RS_UL_DCH request header Bit#22: HARQ RS error report header Bit#23: Relay MAC header | REG-REQ, REG-RSP |