| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HARQ Operation on RS_UL_DCH |  |
| Date Submitted | 2007-09-07 |  |
| Source(s) | Sungcheol Chang Sunggeun Jin Kwangjae Lim Chulsik Yoon ETRI 161, Gajeong-dong, Yuseong-Gu, Daejeon, 305-350, Korea | scchang@etri.re.kr sgjin@etri.re.kr kjlim@etri.re.kr csyoon@etri.re.kr |
| Re: | This is a response to Letter Ballot 16j on P802.16j/D1 |  |
| Abstract | The baseline document defines the Dedicated Uplink Channel (RS_UL_DCH) for RS and its operation of resource allocation for the channel using UL_DCH assignment IE. There are no definitions for HARQ operations on the RS_UL_DCH. An HARQ operation of the PHY burst is proposed. |  |
| Purpose | To incorporate the proposed text into P802.16j Baseline Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# HARQ Operation on RS_UL_DCH

Sungcheol Chang, Sunggeun Jin, Kwangjae Lim, Chulsik Yoon

ETRI

# 1. Introduction

The baseline document defines the Dedicated Uplink Channel (RS_UL_DCH) for RS and its operation of resource allocation for the channel using UL_DCH assignment IE. However HARQ retransmission of the PHY burst on RS_UL_DCH is not defined in the baseline document. If the PHY burst on the RS_UL_DCH is corrupted, the HARQ operation is not applied to recover this corruption on the RS_UL_DCH.

The solution of HARQ operation on the RS_UL_DCH is described in the following text.

# 2. Proposed solution

The RS_UL_DCH is used to assign periodic and dedicated resources in uplink channel. If a PHY burst on the RS_UL_DCH is not successfully decoded, the superordinate RS/MR-BS sends the RS_UL_DCH HARQ Retx IE to the RS. This IE contains corrupted PHY burst-related frame number and RS_UL_DCH identifier. Figure 1 shows the HARQ retransmission by RS_UL_DCH HARQ Retx IE and in-band retransmission for the RS_UL_DCH. In this figure, the PHY burst sent by the RS on the dedicated uplink resource in frame N is assumed to be corrupted, and thereafter, the same PHY burst is retransmitted on the dedicated uplink resource in frame N+2. The dedicated uplink resources are preempted and the new PHY burst is delayed to the next dedicated uplink resource. The dedicated resource is already reserved and fixed so that additional resources are not allocated for HARQ retransmission.

Figure 1 HARQ on RS_UL_DCH and in-band retransmission.

If the RS_UL_DCH HARQ Retx IE has the other resources for PHY burst retransmission, the out-band retransmission is done over the other resource allocated by the IE and except the dedicated uplink resources. Figure 2 shows that the retransmission of the PHY burst is tried on resources except the dedicated uplink resources. This out-band retransmission does not preempt new PHY burst transmission on the dedicated uplink resources for the RS_UL_DCH.

Figure 2 HARQ on RS_UL_DCH and out-band retransmission.

The RS shall maintain all the PHY burst in buffer during maximum frame number of retransmission. This requires the RS to have more buffers for successful retransmission. However, the maximum number of buffers for retransmission in RS is limited by the field of “Maximum frame number of retransmission”.

# 3. Text Proposals

[Change the table at the section 8.4.5.4.4.2 in page 182]

| Extended-2 Type (hexadecimal) | Usage |
| --- | --- |
| 00 | CQICH Enhanced Allocation IE |
| 01 | HO Anchor Active UL-MAP IE |
| 02 | HO Active Anchor UL-MAP IE |
| 03 | Anchor BS Switch IE |
| 04 | UL Sounding Command IE |
| 05 | Reserved |
| 06 | MIMO UL Enhanced IE |
| 07 | HARQ UL MAP IE |
| 08 | HARQ ACKCH Region Allocation IE |
| 09 | HARQ ACKCH Region for Relay Data IE |
| 0A | MR UL-MAP Monitor IE |
| 0B | MIMO UL Extended IE |
| 0C | RS_UL_DCH assignment IE |
| 09C ... 0D | Reserved RS_UL_DCH HARQ Retx IE |
| 0E | AAS SDMA UL IE |
| 0F | Feedback Polling IE |

[Change the Table 496c at the section 8.4.5.9.1 in page 191]

Table 496c-R-link specific IE types

| Type (hexadecimal) | Usage |
| --- | --- |
| 00 | RS_UL_DCH assignment IE |
| 01 | RS_BW-ALLOC IE |
| 02 | RS_UL_DCH HARQ Retx IE |
| 023-1F | Reserved |

[Change the Table 496d at the section 8.4.5.9.1.1 in page 192]

Table 496d-RS_UL_DCH assignment IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| RS_UL_DCH assignment IE() { |  |  |
| Type | 5 bits | RS UL DCH HARQ Retx IE = 0x00 |
| Length | 4 bits | Variable |
| RSCID | 8 bits | Reduced basic CID of the RS |
| Update type | 2 bits | 00 = Normal 01 = Service flow based 10-11 = Reserved |
| If (Update type == 01) { |  | If service flow based update |
| Throughput size | 24 bits | Amount of throughput update in byte/s |
| Access RSCID | 8 bits | Reduced basic CID of the access RS of the MS that completed the service flow event |
| } |  |  |
| Assignment type | 2 bits | 00 = Incremental 10 = Aggregate 10 = Remove 11 Remove all |
| OFDMA symbol offset | 8 bits |  |
| UIUC | 4 bits |  |
| Repetition coding indication | 2 bits | 0b00 – no repetition coding 0b01 – repetition coding of 2 0b10 – repetition coding of 4 0b11 – repetition coding of 6 |
| Subchannel offset | 8 bits |  |
| Duration | 10 bits | Resources allocated to DCH (in OFDMA slots) |
| Frequency (N) | 4 bits | Allocation repeats once every N frames |
| HARQ type | 2 bits | 0b00 – Not applided 0b01 – HARQ Chase 0b10 – HARQ CTC IR 0b11 – HARQ CC IR |
| If (HARQ type == 0b01) { |  |  |
| ACID | 4 bits | HARQ Channel ID |
| } |  |  |
| If (HARQ type == 0b10) { |  |  |
| NEP | 4 bits |  |
| NSCH |  |  |
| SPID |  |  |
| ACID | 4 bits | HARQ Channel ID |
| } |  |  |
| If (HARQ type == 0b11) { |  |  |
| SPID |  |  |
| ACID | 4 bits | HARQ Channel ID |
| } |  |  |
| } |  |  |

[Insert the following text at the end of the section 8.4.5.9.1.2 in page 193]

8.4.5.9.1.3 RS_UL_DCH HARQ Retx IE

This RS_UL_DCH HARQ Retx is transmitted to a RS to request the retransmission of the PHY burst indicated by the RS_UL_DCH HARQ Retx IE. The detail description on the RS_UL_DCH HARQ Retx IE is illustrated in Table 296f.

Table 296f -Description of fields in RS_UL_DCH HARQ Retx IE

| Syntax | Size | Notes |
| --- | --- | --- |
| RS_UL_DCH HARQ Retx IE() { |  |  |
| Type | 5 bits | RS UL DCH HARQ Retx IE = 0x02 |
| Length | 4 bits | Variable |
| Number of HARQ Retx sub-bursts | 3 or 4 bits | 3 bits in UL-MAP of access region. 4 bits in relay region. |
| for (i=0;i<Number of HARQ Retx sub-bursts; i++) { |  |  |
| RSCID | 8 bits | Reduced basic CID of the RS |
| In-band or out-band Retx | 1 bit | 0: in-band HARQ retransmission 1: out-band HARQ retransmission |
| if (In-band or out-band Retx == 1) { |  | Out-band HARQ retransmission |
| Subchannel offset | 8 bits |  |
| Duration | 10 bits | Resources allocated to DCH (in OFDMA slots) |
| } |  |  |
| if (HARQ Chase) { |  |  |
| ACID | 4 bits | HARQ Channel ID |
| } |  |  |
| if (HARQ CTC IR or CC IR) { |  |  |
| ACID | 4 bits | HARQ Channel ID |
| SPID | 2 bits |  |
| } |  |  |
| } |  |  |
| } |  |  |