| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HARQ Operations on RS_UL_DCH |  |
| Date Submitted | 2007-11-13 |  |
| Source(s) | Sungcheol Chang, Sunggeun Jin, Kwangjae Lim, and Chulsik Yoon ETRI 161, Gajeong-dong, Yuseong-Gu, Daejeon, 305-350, Korea Derek Yu, hang Zhang, Peiying Zhu, Wen Tong, David Steer, Gamini Senarath, Mark Naden, G.Q. Wang Nortel 3500 Carling Avenue Ottawa, Ontario K2H 8E9 | scchang@etri.re.kr sgjin@etri.re.kr kjlim@etri.re.kr csyoon@etri.re.kr wentong@nortel.com pyzhu@nortel.com |
| Re: | IEEE P802.16j/D1: IEEE 802.16j working group letter ballot #28. |  |
| Abstract | The baseline document defines the Dedicated Uplink Channel (RS_UL_DCH) for RS and its operation of resource allocation for the channel using UL_DCH assignment IE. There is no definition for HARQ operations on the RS_UL_DCH. An HARQ operation of the PHY burst is proposed. |  |
| Purpose | To incorporate the proposed text into P802.16j Baseline Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# HARQ Operations on RS_UL_DCH

Sungcheol Chang, Sunggeun Jin, Kwangjae Lim, Chulsik Yoon

ETRI

Derek Yu, hang Zhang, Peiying Zhu, Wen Tong, David Steer, Gamini Senarath, Mark Naden, G.Q. Wang

Nortel

# 1. Introduction

The RS UL_DCH supports signaling and data traffic associated with service flows, and hence HARQ on UL_DCH needs to be supported.  As the UL_DCH is a persistence allocation and the resources are available periodically without any further signaling until it needs to be updated, such as, to a different size.  As such, it is not possible to follow the current HARQ process of using embedded sub-burst IE inside HARQ UL MAP IE during HARQ BW allocation.

The current ACK/NAK for the UL HARQ transmission bursts from the MSs/RSs are sent using DL HARQ ACK IE.  It contains an ACK/NAK bitmap and the order of bits follows the same order of the HARQ sub-burst allocations in the broadcasted HARQ UL MAP IE.  As such, it also cannot be used for HARQ in UL_DCH.

This contribution supplies text to enable HARQ operation on the RS UL_DCH.

# 2. Proposed solution

The proposed HARQ operation on the RS UL_DCH is essentially the implementation of the high level UL HARQ design described in the following paragraph in Subclause 6.3.17 in 802.16e Standard document (P802.16Rev2/D1, October 2007). Minor differences have been introduced to take advantage of the nature of RS UL_DCH.

“The HARQ scheme is a stop-and-wait protocol. The ACK is sent by the SS after a fixed delay (synchronous ACK) defined by HARQ DL ACK delay for DL burst, which is specified in UCD message. Timing of retransmission, however, is flexible and corresponds to the asynchronous part of the HARQ. The ACK/NAK is sent implicitly by toggling the AI_SN bit or together with the HARQ ACK IE by the BS. The ACK/NAK is sent by an SS using the UL ACK subchannel. Transmission of HARQ Bitmap IE by BS is optional.”

The high level design of the HARQ operations on RS UL_DCH is NAK based and retransmission can either be done at the dedicated resource or additional resource depending on the needs. HARQ on RS UL_DCH is a hop-by-hop HARQ. Transmission of synchronous HARQ NAK bitmap by the superordinate station is optional. The additional resources for retransmission also provides the implicit NAK to the HARQ transmission error.

For HARQ on the UL_DCH, each RS cannot determine how many HARQ bursts are being transmitted by all the other RSs through their UL_DCH and their order of transmission.  Hence, the NAK for different RSs cannot be combined into a broadcasted bitmap to acknowledge HARQ bursts from all subordinate RSs on their dedicated channels.  A unicast NAK bitmap is used for each RS and it is sent using a DL MAC control header.

For multihop centralized scheduling, it is not possible to dynamically allocated NAK resource for superordinate RSs that are two or more hops away from the MR-BS as the minimum round trip delay will be larger than the maximum of 3 frames delay for the transmission of synchronous NAK signaling.  To effectively enable NAK signaling for the UL HARQ transmission on the dedicated channel, the MR-BS needs to allocate dedicated signaling channel in the downlink for the RS using RS DL DCH assignment IE.

For distributed scheduling, in place of frequent/periodic ACK/NAK allocations, the superordinate station may allocate dedicated signaling channel in the downlink for its RSs when significant number of HARQ enabled service flows have been established.

With synchronous NAK, there is much less buffer demand on the sender side. Successfully received HARQ packets by the receiver can be quickly removed at the sender side. Moreover, the synchronous NAK bitmap is only send when there are one or more bursts that are received in error.

Retransmission can be required at the dedicated resource on RS_UL_DCH or at the additional resource. Dedicated resource allocation for retransmission can work with both centralized and distributed allocation.  Additional resource allocation for retransmission can work with distributed allocation but may incur extra delay and signaling under multi-hop centralized allocation as an additional resource request to the MR-BS is needed. The additional retransmission resource is requested by indicating HARQ retransmission request in the RS UL_DCH header and it is sent to the MR-BS. The retransmission resource is allocated using RS_UL_DCH HARQ RETX IE. The transmission of RS_UL_DCH HARQ RETX IE to allocate retransmission resource provides the implicit NAK to the RSs.

The proposed HARQ scheme is also apply to transparent RS as long as the transparent RS is responsible for both receiving the HARQ bursts and sending all signaling in the DL dedicated signaling channel to the RS. The transparent RS can generate the necessary NAKs and combine with all other signaling and send to the RS.

The following figures illustrate the HARQ operation under different scenarios. Figure 1 illustrates the operation using the dedicated resources for retransmission for both centralized and distributed allocation. Figure 2 illustrates the operation using the additional resource for retransmission for distributed allocation with RS DL_DCH signaling channel allocated. Figure 3 illustrates the operation using additional resource for retransmission when NAK bitmap is not used. Lastly, Figure 4 illustrates the operation without RS_DL_DCH for centralized allocation.

Fundamentally, retransmission is done through resource from the same HARQ channel (same ACID). For retransmission using the dedicated resource, the next resource of the same ACID/HARQ channel is used for retransmission.  For retransmission using the additional resource, one-time additional resource is allocated to the same ACID/HARQ channel for retransmission.

For the retransmission described in Figure 1, the next occurrence of transmission resource for ACID 0 is used to retransmit burst A after the synchronous NAK bitmap.  For CTC IR HARQ, the SPID is also provided together with the NAK bitmap.

In Figure 2, the additional retransmission resource is allocated to ACID 0 for retransmission of burst A after the synchronous NAK that was sent on RS DL_DCH. The additional allocation is done through RS_UL_DCH HARQ RETX IE.  Figure 3 describes the same retransmission but synchronous NAK bitmap is not used because of no RS DL_DCH signaling channel. The RS_UL_DCH HARQ RETX IE provides the implicit NAK when it allocates the additional retransmission resource.

In Figure 4, for centralized allocation, the request for asynchronous retransmission is sent from the superordinate RS to the MR-BS using RS UL_DCH HARQ retransmission request header. The MR-BS then constructs the required RS_UL_DCH HARQ RETX IE and forward to the superordinate RS for the one time retransmission resource allocation.

Figure 1: Retransmission using the dedicated resource for both centralized and distributed allocation.

Figure 2: Retransmission using the additional resource for distributed allocation with allocated RS DL_DCH.

Figure 3: Retransmission using the additional resource for distributed allocation without RS_DL_DCH.

Figure 4: Retransmission for centralized allocation.

# 3. Text Proposals

+++++++++++++++ Start Text Proposal +++++++++++++++++++++++++++++++++++++++++++++

[Add the following Subclause]

6.3.17.6 HARQ support on RS_UL_DCH

The HARQ on RS_UL_DCH is a NAK-based HARQ and a hop-by-hop HARQ. The RS_UL_DCH assignment IE configures and updates HARQ operation on RS_UL_DCH. The HARQ control field indicates HARQ NAK feedback type on RS_UL_DCH, which is either synchronous HARQ NACK bitmap or asynchronous HARQ NAK. The HARQ configuration remains same and HARQ NAK feedbacks shall be as given in the format of the selected HARQ NAK feedback type.

Packets from multiple MSs/RSs are multiplexed and transmitted through the UL DCH. Each DCH resource block can be used for transmitting a single HARQ burst at a time. For allocation of new UL DCH resource block a number of ACID is also assigned. Implicit sequential cycling of ACID is used for each occurrence of the periodically assigned resource. The first transmission after enabling HARQ is always HARQ channel 0 (ACID is 0). The ACID is incremented by 1 for each periodic transmission and reset to 0 when the maximum ACID number is reached.

The synchronous NAK bitmap and retransmission control signaling are sent by the RS. The RS that receives HARQ UL burst at i-th frame may transmit NAK signal at (i+j)-th frame. The NAK bitmap is sent only when one or more bursts that are received in error. The frame offset “j” is defined by the “HARQ ACK Delay for UL Burst” field in the DCD message. The asynchronous NAKs are identified by a set of DCH resource ID and ACID, which identify bursts on RS_UL_DCH. The asynchronous NAK may be sent if the feedback delay is variable.

The resources for retransmission are allocated at either dedicated resources on RS_UL_DCH or at additional resources. For using the dedicated resource, the retransmission is sent using the same HARQ channel and it is sent on the next occurrence of the same HARQ channel after the NAK signal. Thus, the dedicated resource allocation minimizes signaling overhead for the retransmissions. For using the additional resource, a one-time additional resource is allocated to the HARQ channel that requires retransmission. This allocation is assigned by using RS_UL_DCH HARQ RETX IE and it also includes the retransmission parameters. In this case, the number of ACID shall be large enough to allow for the maximum number of retransmission attempts before the ACID wrap around. When the bandwidth on RS_UL_DCH is required to be guaranteed, the additional resources shall be allocated.

RS_UL_DCH HARQ RETX IE and UL DCH HARQ control header may be sent for carrying HARQ NAK feedbacks and requesting retransmission for corrupted bursts. The RS that sends the NAK may use one of two formats for feedback. If RS_DL_DCH is established and its resource is available for HARQ NAK feedback, the RS could send UL DCH HARQ control header on RS_DL_DCH. RS_UL_DCH HARQ RETX IE can be used under both centralized and distributed scheduling for requesting the retransmissions while UL DCH HARQ control header with synchronous HARQ NACK bitmap can be transmitted optionally.

UL DCH HARQ control header can be used for allocating retransmission resource only in UL dedicated resource on RS_UL_DCH. After receiving a UL DCH HARQ control header, the RS shall retransmit the corrupted bursts at the next available UL DCH resources. RS_UL_DCH HARQ RETX IE can request retransmissions at both dedicated resources on RS_UL_DCH and at other additional resources. In case there is no additional resource allocated in RS_UL_DCH_ HARQ RETX IE, the next available UL DCH resources are used for retransmission.

In case of centralized scheduling, the intermediate RS may also generate UL DCH control header on RS_DL_DCH with the necessary NAKs. If the dedicated bandwidth on RS_UL_DCH requires to be guaranteed, the intermediate RS shall use the RS_UL_DCH header with HARQ retransmission request so that the MR-BS can allocate additional resources for retransmission.

[Change Table 496d in Subclause 8.4.5.9.1.1 on page 192 as indicated]

Table 496d-RS_UL_DCH assignment IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| RS_UL_DCH assignment IE() { |  |  |
| Type | 5 bits | RS UL DCH assignment IE = 0x00 |
| Length | 4 bits | Variable |
| RSCID | 8 bits 4,8,12,16 bits | Reduced basic CID of the RS RS basic CID in RCID_IE format (see 8.4.5.3.20.1) |
| Update type | 2 bits | 00 = Normal 01 = Service flow based 10-11 = Reserved |
| If (Update type == 01) { |  | If service flow based update |
| Throughput size | 24 bits | Amount of throughput update in byte/s |
| Access RSCID | 8 bits 4,8,12,16 bits | Reduced basic CID of t The access RS of the MS that completed the service flow event. RS basic CID in RCID_IE format (see 8.4.5.3.20.1) |
| } |  |  |
| Assignment type | 2 bits | 00 = Incremental (Add the specified resource to UL DCH) 01 = Aggregate (An aggregate assignment with no resource indicates all UL DCH removal) 10 = Remove Removal (Remove the specified resource from UL DCH) 11 = Remove all Tx profile and settings update |
| OFDMA symbol offset | 8 bits |  |
| DCH resource ID | 3 bits | ID of the DCH resource being assigned or managed |
| HARQ Control | 1 bit | 0: Synchronous HARQ NAK bitmap 1: Asynchronous HARQ NAK |
| HARQ type | 2 bits | 0b00 – Disabled 0b01 – HARQ Chase 0b10 – HARQ CTC IR 0b11 – HARQ CC IR |
| if (HARQ type == 0b10) { |  |  |
| NEP | 4 bits |  |
| NSCH | 4 bits |  |
| } else { |  |  |
| UIUC | 4 bits |  |
| Repetition coding indication | 2 bits | 0b00 – no repetition coding 0b01 – repetition coding of 2 0b10 – repetition coding of 4 0b11 – repetition coding of 6 |
| } |  |  |
| if (Assignment type == 01 ¦¦ Assignment == 11) { |  |  |
| Number of ACID | 5 bits | Maximum number of HARQ channels associated with this assignment |
| } |  |  |
| if (Assignment type != 0b11) { |  |  |
| OFDMA symbol offset | 8 bits |  |
| Subchannel offset | 8 bits |  |
| Duration | 10 bits | Resources allocated to DCH (in OFDMA slots) |
| } |  |  |
| if (Assignment type == 0b01) { |  |  |
| Frequency (N) | 4 bits | Allocation repeats once every N frames. |
| } |  |  |
| } |  |  |

[Add the following table to the end of Subclause 6.3.2.1.3]

Table XXX—Type field encodings for DL MAC control header

| Type field | MAC control header Type | Reference figure | Reference table |
| --- | --- | --- | --- |
| 0 | UL DCH HARQ control header |  |  |
| 1-15 | Reserved |  |  |

[Add the following Subclause]

6.3.2.1.3.1 UL DCH HARQ control header

The UL_DCH HARQ control header is used by the RS to send DCH HARQ control signaling. This includes either the synchronous HARQ NAK bitmap or the asynchronous HARQ NAK. The synchronous HARQ NAK bitmap indicates an error in the corresponding received RS UL_DCH HARQ bursts. The RS that receives HARQ UL burst at i-th frame should transmit NAK signal at (i+j)-th frame. The frame offset “j” is defined by the “HARQ ACK Delay for UL Burst” field in the DCD message. The NAK bitmap is sent only when one or more bursts are received in error. The asynchronous HARQ NAK indicates corrupted bursts identified by sets of DCH resource ID and Frame number. After receiving HARQ NAKs in formats of both synchronous and asynchronous NAK, the transmitting RSs shall retransmit the corrupted bursts in the following dedicated resources. For a DCH resource ID in the RS, the burst indicated by k-th NAK should be retransmitted at k-th dedicated burst from the frame in which the RS_UL_DCH HARQ RETX IE is received.

The format of the header is shown in Figure XXX.

Table XXX - UL DCH HARQ control header format

| Name | Length (bits) | Description |
| --- | --- | --- |
| HT | 1 | Header type. Shall be set to 1 |
| EC | 1 | Encryption control. Shall be set to 0 |
| Type | 4 | Shall be set to 0000 for UL DCH HARQ control header |
| HARQ Control | 1 | 0: Synchronous HARQ NAK bitmap 1: Asynchronous HARQ NAK |
| SPID indicator | 1 | CTC IR HARQ retransmission SPID |
| if (HARQ Control == 0) { |  | Synchronous HARQ NAK bitmap |
| NAK bitmap | 8 | HARQ NAK bitmap for the corresponding HARQ bursts sent in each DCH resource block. The LSB bit indicates NAK for resource block allocated with DCH resource ID of “0” and sequentially to the MSB bit. Each bit is set to 1 if the corresponding uplink DCH HARQ burst cannot be successfully received; otherwise, it shall be 0 |
| if (SPID indicator) { |  | For CTC IR, specify SPID for retransmission bursts |
| SPID list | 16 | The least significant two bits specify SPID for resource block with DCH resource ID of “0” and sequentially to the most significant two bits |
| } |  |  |
| } |  |  |
| else if (HARQ Control == 1) { |  | Asynchronous HARQ NAK |
| Number of bursts | 2 |  |
| for (i=0;i<Number of bursts; i++) { |  |  |
| DCH resource ID | 3 bits | ID of the RS_UL_DCH resource |
| ACID | 5 bit | HARQ channel required retransmission |
| SPID indicator | 1 bit | SPID field indication |
| if (SPID indicator) { |  |  |
| SPID | 2 bits | For CTC IR, specify SPID for retransmission bursts |
| } |  |  |
| } |  |  |
| Padding | variable |  |
| HCS | 8 | Header check sequence |

[Change Table 496c in Subclause 8.4.5.9.1 as indicated]

Table 496c—R-link specific IE types

| Type (hexadecimal) | Usage |
| --- | --- |
| 00 | RS_UL_DCH assignment IE |
| 01 | RS_BW-ALLOC IE |
| 02 | RS_DL_DCH assignment IE |
| 03 | RS_UL_DCH HARQ RETX IE |
| 0204-1F | Reserved |

[Add the following Subclause]

8.4.5.9.1.3 RS DL DCH assignment IE

This IE is used for the initial allocation and subsequent updates of the downlink dedicated channel for the RS.  The dedicated channel is used for DL signaling including MAC management messages.

Table XXX. RS_DL_DCH assignment IE format.

| Syntax | Size | Notes |
| --- | --- | --- |
| RS_DL_DCH assignment IE { |  |  |
| Type | 5 bits | RS_DL_DCH assignment IE = 0x02 |
| Length | 4 bits |  |
| RSCID | 4,8,12,16 bits | RS basic CID in RCID_IE format (see 8.4.5.3.20.1) |
| Assignment type | 2 bits | 00 = Aggregate (An aggregate assignment with no resource indicates DL DCH removal) 01 = Update 10-11 = Reserved |
| If (Assignment type == 00) { |  |  |
| DIUC | 4 bits |  |
| Boosting | 3 bits |  |
| Repetition coding indication | 2 bits |  |
| OFDMA Symbol offset | 8 bits |  |
| Subchannel offset | 8 bits |  |
| No. OFDMA Symbols | 7 bits |  |
| No. Subchannels | 6 bits |  |
| Frequency (N) | 4 bits | Allocation repeats once every N frames |
| } |  |  |
| If ((Assignment type == 01) { |  |  |
| DIUC | 4 bits |  |
| Boosting | 3 bits |  |
| Repetition coding indication | 2 bits |  |
| } |  |  |
| } |  |  |

[Add the following Subclause]

8.4.5.9.1.4 RS_UL_DCH HARQ RETX IE

RS_UL_DCH HARQ RETX IE is transmitted to RSs to request the retransmission of HARQ bursts on RS UL_DCH. The corrupted HARQ bursts are indicated by their corresponding DCH resource ID and ACID. It includes the allocation of dedicated UL DCH or additional resources for retransmission. If Retransmission type is 0, a RS that receives HARQ NAK feedbacks should retransmit the corrupted bursts in the dedicated resources on RS_UL_DCH. For a DCH resource ID in the RS, the burst indicated by k-th NAK should be retransmitted at k-th dedicated burst from the frame in which the RS_UL_DCH HARQ RETX IE is received. If Retransmission type is 1, the corrupted burst is retransmitted in additional resource.

The format of the RS_UL_DCH HARQ RETX IE is shown in Table XXX.

Table XXX. RS_UL_DCH HARQ RETX IE format.

| Syntax | Size | Notes |
| --- | --- | --- |
| RS_UL_DCH HARQ ASYNC RETX IE () { |  |  |
| Type | 5 bits | RS_UL_DCH HARQ ASYNC RETX IE = 0x03 |
| Length | 4 bits | Variable |
| Number of bursts | 3 bits |  |
| for (i=0;i<Number of bursts; i++) { |  |  |
| RSCID | 4,8,12,16 bits | RS basic CID in RCID_IE format (see 8.4.5.3.20.1) |
| DCH resource ID | 3 bits | ID of the RS_UL_DCH resource has transmission failure |
| ACID | 5 bit | HARQ channel required retransmission |
| Retransmission Type | 1 bit | 0: HARQ retransmission to occur in allocated dedicated UL_DCH 1: HARQ retransmission to occur in resource assigned in this IE |
| if (Retransmission Type == 1) { |  | HARQ retransmission to occur in resource assigned in this IE |
| OFDMA Symbol offset | 8 bits |  |
| Subchannel offset | 8 bits |  |
| Duration | 10 bits | Resources allocated for HARQ retransmission (in OFDMA slots) |
| } |  |  |
| SPID indicator | 1 bit | SPID field indication |
| if (SPID indicator == 1) { |  |  |
| SPID | 2 bits |  |
| } |  |  |
| } |  |  |
| } |  |  |

[Change the text in Subclause 6.3.2.1.2.2.2.2 on page 11]

6.3.2.1.2.2.2 RS UL DCH request header (RS_UL_DCH)

The RS requests a dedicated uplink resource through the RS_UL_DCH request header by specifying DCH Request. Under centralized allocation, it is also used for intermediate RS to request HARQ retransmission resource from the MR-BS by specifying HARQ retransmission request. The MR-BS responses with RS_UL_DCH HARQ RETX IE. The format of this header is illustrated in Figure 35c and described in Table 19c.

Figure 35c-RS_UL_DCH request header format

Table 19c-Description of fields in RS UL_DCH request header

| Syntax | Size | Notes |
| --- | --- | --- |
| MAC Header() { |  |  |
| HT | 1 bit | Shall be set to 1 |
| EC | 1 bit | Shall be set to 1 |
| Type | 1 bit | Shall be set to 1 |
| Extended TYPE | 3 bits | Shall be set to 001 for RS_UL_DCH request header |
| TYPE | 4 bits | 0000 = DCH Request 0001 = Reserved 0010 = HARQ retransmission request 0001 0011 – 1111 = Reserved |
| if (TYPE==0000) { |  | DCH Request |
| DCHTYPE | 2 bits | 00 = DCH Request Incremental 01 = DCH Request Aggregate 10 = DCH Request Rate Based 11 = Reserved |
| if (DCHTYPE == 00) { |  | DCH Request Incremental |
| Bandwidth request | 16 bits | Number of bytes requested by the RS. Zero in this field indicated DCH release request. |
| N | 4 bits | Allocation repeats once every N frames. |
| } else if (DCHTYPE == 01) { |  | DCH Request Aggregate |
| Bandwidth request | 16 bits | Number of bytes requested by the RS. Zero in this field indicated DCH release request. |
| N | 4 bits | Allocation repeats once every N frames. |
| } else if (DCHTYPE == 10) { |  | DCH Request Rate Based |
| Average rate | 20 bits | Average data rate in units of bytes per second 18 MSB bits: magnitude 2 LSB bits: base-10 exponent |
| } |  |  |
| RS CID | 8 bits | 8 LSB of Reduced Basic CID of RS |
| } |  |  |
| if (TYPE == 0010) { |  | HARQ retransmission request |
| RS CID | 8 bits | 8 LSB of Basic CID of the RS requiring HARQ retransmission |
| Number of bursts | 2 bits |  |
| for (i=0;i< Number of bursts; i++) { |  |  |
| DCH resource ID | 3 bits | ID of the DCH resource that needs HARQ retransmission |
| ACID | 5 bits | HARQ channel require retransmission |
| } |  |  |
| Reserved | variable |  |
| } |  |  |
| HCS | 8 bits | Header check sequence |
| } |  |  |