| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Corrections to HARQ ACK/NAK Scheduling |  |
| Date Submitted | 2007-09-19 |  |
| Source(s) | Rakesh Taori Samsung Advanced Institute of Technology Youngbin Chang, Jung Je Son Samsung Electronics Eugene Visotsky Motorola | Voice: +82 31 280 9635 E-mail: rakesh.taori@samsung.com E-mail: yb.chang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | In response to the call for comments on the IEEE 802.16j D1 |  |
| Abstract | In this document we propose some corrections to the HARQ ACK/NAK scheduling mechanism. We propose a mechanism that is simpler, is based on more reasonable assumptions and fits the design philosophy of centralized scheduling |  |
| Purpose | Discussion and Adoption in the IEEE 802.16j Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Corrections to HARQ ACK/NAK scheduling

Rakesh Taori, Youngbin Chang, Jung Je Son

# Problem

The current TGj draft (P802.16j D1) specifies an Equation which is used to compute “i + n”, where n is the number of frames after which an RS sends encoded ACK/NAK for an HARQ burst received in frame i:

n = (H-1)*p + H*j

….Equation (1)

where H is equal to “hop_depth” that the represents number of hops that MR-BS/RS is away from the MS, while p represents the DL burst processing delay at the RS and j the "HARQ_ACK_Delay for DL Burst" for RS and MS RS and MS.

There are some problems with the application of this Equation:

(1) The Use of j in the above equation is basedon one of the following assumptions:

(A) The assumption that the delay incurred in forwarding an ACK/NAK (let us call it the ACK forwarding delay) is the same as the delay incurred for an MS to process the DL burst and generate an ACK/NAK.

OR

(B) ACK/NAK forwarding delay in RS is the same as RS’s processing capability p in the above equation.

Either of these assumptions are not reasonable, however. Arguably, the ACK/NAK can even be forwarded in the same frame that it was received. In any case, forwarding delay is likely to be faster than DL data burst processing delay of RS and MS, respectively, p and j, because RS does not need to perform data burst decoding or CRC verification.

(2) ACK/NAK forwarding delay is also dependent on the frame structure. Contrast, for instance, the cases when UL frame has one access zone and one relay zone with the case that UL frame has one access zone and two relay zones. In such cases, ACK/NAK can be forwarded within one frame or be forwarded in the next frame.  Furthermore, if we consider odd-hop and even-hop frames in a multi-hop situation, then it is quite different as to how the delay should be calculated.

In short the above equation is applicable under specific assumptions that may not always be the case.

(3) Further more it introduces complexity at the RS that can be avoided. Centralized scheduling is mainly in the specification for facilitating a simple design of the RS

Suggested Remedy

First, let us clarify some definitions. If h represents the number of hops that an RS is away from the MS, we denote the delay incurred at the hth RS in processing a DL burst and preparing the data for forwarding as ph, while we denote the delay incurred in forwarding the ACK/NAK information as qh. The HARQ ACK delay incurred at the MS is denoted by j (same as the current 802.16e specification).

Figure 1: The delay incurred in processing a DL burst is (p).  The delay incurred in forwarding the ACK/NACK information (q) can be different from the HARQ ACK delay at the MS (j) which is already defined in the 802.16e specification.

In general, the delays ph and qh will be different and p and q for different RSs will be different depending on the capability of each RS. Therefore, according to the Figure, it is clear that the ACK/NAK scheduling delay at any RS can be given by the following Equation:

n=h=1Hph+qh+j

…. Equation (2)

The next thing to consider is whether we need to transmit the hop depth. Even if the hop depth is transmitted, an RS (e.g. RS2) needs to know the values p and q for all subordinate RSs (e.g. p1 and q1). This means that an RS will not be able to calculate n based on hop depth alone.

We argue in this contribution that instead of transmitting the hop depth, it is beneficial for the BS to transmit the frame number (offset) in which the RS should transmit the encoded ACK/NAK. This offset only need to be transmitted once per HARQ DL MAP because one HARQ DL MAP shall consist of bursts that are same number of hops away (See page 127, line 16/17).

Moreover, if the BS transmits the frame number at which the encoded ACK/NAK needs to be transmitted, the RS need not calculate this by itself. This simplifies the RS implementation which is in line with the idea of centralized scheduling.

The BS shall calculate n according to Equation 2 using p, q and j. The values for p and q may be exchanged during capability negotiation.

.

# Proposed Text

Change lines 42-45 on Page 126 as follows:

The number of prescheduled retransmissions for a HARQ flow may be provided to the access RS from the MR-BS in the “hop_depth” field of the RS_HARQ_DL_MAP_IE. The UL ACK channel from the access RS to the MR-BS shall be allocated after the prescheduled retransmission attempts have been acknowledged by the MS. This ACK/NACK delay shall be specified in the ACK_frame_delay field of the RS_HARQ_DL_MAP_IE.

Change lines 16-23 on Page 127 as follows:

MR-BS shall allocate the HARQ region which contain bursts destined to MSs which have same number of hops away in common the same ACK_frame_delay by using RS HARQ DL MAP IE. Similarly MR-BS shall allocate ACKCH by using HARQ_ACKCH region allocation for Relay Data IE.

MR-BS shall indicate the hop_depth ACK_frame_delay in RS HARQ DL MAP IE as well as in HARQ_ACKCH region allocation for Relay data IE so that RS can map the HARQ burst and corresponding HARQ ACK/NAK accordingly to indicate to the RS the offset from the current frame for transmitting the encoded ACK/NAK.

Change the row regarding hop_ depth in Table 411, page 178, as follows:

| hop_depth ACK_frame_delay | 3 bits 4bits | B000 and b001 are invalid. When MR-BS/RS transmits HARQ burst for the nth hop away MSs, it shall set hop_depth = n. hop_depth represents the value of H defined in subcluase 8.4.5.4.25. The RS shall transmit encoded ACK/NAK after ACK_frame_delay frames from receiving the DL HARQ MAP. |
| --- | --- | --- |
| Reserved | 1 bit |  |

Change Table 484a on Page 186 as follows:

Table 484a—HARQ_ACKCH region allocation for UL Relay Data IE

| Syntax | Size | Notes |
| --- | --- | --- |
| HARQ_ACKCH_Region_for Relay Data IE() { |  |  |
| Extended 2 UIUC | 4 bits | HARQ ACKCH Region for Relay Data IE = 0x09 |
| Length | 8 bits | Length in bytes |
| Direction 1 bit | 1 bit | 0 = IE is related to UL HARQ Data IE 1 = IE is related to DL HARQ Data IE |
| If (direction == 1) { |  |  |
| N_hop | 4bits |  |
| for (i = 0; i < N_hop; i++) { |  |  |
| hop_depth | 3 bits | B000 and b001 are invalid. When MR-BS/RS transmits HARQ burst for the nth hop away MSs, it shall set hop_depth =n. |
| Reserved | 1 bit |  |
| ACKCH_offset | 8 bits | ACKCH_offset indicates the starting point in the ACKCH region for sending HARQ ACK/NAK for corresponding hop_depth. |
| } |  |  |
| } else { |  |  |
| Reserved | 3 bits |  |
| } |  |  |
| OFDMA Symbol | 8 bits |  |
| Subchannel offset | 7 bits |  |
| No. OFDMA symbols | 5 bits |  |
| No. subchannels | 4 bits |  |
| } |  |  |

Change lines 47 (Page 186) – Line 4( page 187) as follows:

When RS receives HARQ DL sub-burst for relaying to MS at frame i, it shall transmit the encoded ACK/ NAK signal through ACK Channel in the ACKCH region at frame (i + n) where n is given by the ACK_frame_delay in HARQ DL MAP IE which is transmitted by the MR-BS to the RS. calculated at each RS according to the following equation.

n = (H-1)*p + H*j

H is equal to “hop_depth” transmitted in RS HARQ DL MAP IE and HARQ_ACKCH region allocation for relay Data IE. It represents number of hops MR-BS/RS is away from the MS. p is defined by the "HARQ_burst_Delay for DL Burst" field in the DCD messages.

j is defined by the "HARQ_ACK_Delay for DL Burst" field in the DCD messages. It is applicable to both RS and MS.

In 2-hop case, there is only one RS and n=p + 2*j.

ACK/NAKs in response to sub-bursts belonging to HARQ DL MAP IEs received in different frames, shall be transmitted in the order in which the MAPs were received.

Delete lines 6 -16 on page 187:

If pre-scheduling of retransmissions on the access link on the DL is enabled, only HARQ flows with the same number of pre-scheduled retransmission attempts shall be scheduled into the same HARQ region. An UL HARQ feedback region is allocated in the frame i + n at an RS for a HARQ region received in frame i, where

n= (H˜ – 1)× p +H˜×j

and and k denotes the number of pre-scheduled attempts. For pre-scheduled bursts, H˜ is specified in the “hop_depth” field of the RS_HARQ_DL_MAP_IE. At an access RS, H = 1, and hence the number of pre-scheduled attempts for a HARQ burst can be computed at the access RS as . k=H˜ – 1.