| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on SS ranging and automatic adjustments for the RS group |  |
| Date Submitted | 2007-11-14 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan Israfil Bahceci, Senarath Gamini, Peiying Zhu, Wen Tong, Mo Han Fong, Derek Yu, David Steer, Mark Naden Nortel Networks 3500 Carling Avenue Ottawa, Ontario K2H 8E9 Adrian Boariu Nokia Siemens Networks [add other co-authors here] | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw Voice: +613-763-1315 E-mail: wentong@nortel.com E-mail: pyzhu@nortel.com E-mail: adrian.boariu@nsn.com |
| Re: | IEEE 802.16j-07/043: “IEEE 802.16 Working Group Working Group Letter Ballot #28” |  |
| Abstract | This contribution proposes MS ranging and automatic adjustments for the RS group |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on SS ranging and automatic adjustments for the RS group

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry (III)

Introduction

The accepted contribution (IEEE C802.16j-07/459r6 Comments on SS ranging and network entry in transparent and non-transparent mode) did not include MS ranging and automatic adjustments for the RS group. This contribution proposes remedy for MS ranging and automatic adjustments for the RS group and covers the following comments:

| Comment# | subclauses | Authors |  | Type |
| --- | --- | --- | --- | --- |
| 673 | 6.3.10.3.4.2.1 | Gamini | Senarath | Technical |
| 683 | 6.3.10.3.4.4.1 | Gamini | Senarath | Technical |

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D1 are listed below.

Specification Changes

[Note: The text accepted in the previous meeting (comment #0565: document IEEE C802.16j-07/459r6) are marked in “Gray”]

[Delete 6.3.9.16.3.1.1 as following indicated:]

6.3.9.16.3.1.1 MS network entry procedures

Each RS group member shall monitor the CDMA ranging codes from subordinate nodes. If the group parent is not a member of the RS group, then RS group members shall follow the procedures in 6.3.9.16.1. If the group parent is a member of the RS group, then the RS group members other than the parent shall follow the procedure in 6.3.9.16.1, and the parent (if not MR-BS) shall follow the procedures in 6.3.9.16.2.

6.3.10.3.1.1 MR-BS and RS behavior during contention-based initial ranging

[Modified the following paragraphs as indicated:]

- If adjustments are required, the MR-BS shall transmit the RNG-RSP to the SS and the process shall repeat. When the ranging code requires no further adjustment, the MR-BS shall provide an allocation in the access uplink for the SS to forward a RNG-REQ with its MAC address by inserting a CDMA_Allocation_IE in the UL-MAP. If management messages are relayed on the uplink, the MR-BS shall precede the CDMA_Allocation_IE with an UL_Burst_Receive_IE containing the access RS’s basic CID or multicast management CID.  A transparent RS, whose CID matches the RS basic CID or the multicast management CID of the UL_Burst_Receive_IE, shall receive the RNG-REQ on a burst specified by the CDMA_Allocation_IE and relay it to the MR-BS on the RS basic CID.

[Insert the following paragraphs as indicated:]

When an SS performs initial ranging in systems with non-transparent RSs using shared BSIDs with other access stations, MR-BSs and those non-transparent RSs with shared BSIDs shall perform the same steps as transparent RSs with the following modifications:

- If management messages are relayed on the uplink, the MR-BS shall precede the CDMA_Allocation_IE with an UL_Burst_Receive_IE, containing the access RS’s basic CID or multicast management CID, in the RS_Access-MAP message sent to the RS. The RS shall remove the UL_Burst_Receive_IE when compose the (compress) UL-MAP which the RS broadcasted. A non-transparent RS, whose CID matches the RS basic CID or the multicast management CID of the UL_Burst_Receive_IE, shall receive the RNG-REQ on a burst specified by the CDMA_Allocation_IE and relay it to the MR-BS on the RS basic CID.

- If adjustments are required, the MR-BS shall transmit the RNG-RSP to the SS directly or to a non-transparent RSs with the RS basic CID or the multicast management CID, which shall relay the received RNG-RSP to the MS with the initial ranging CID.

When an SS performs initial ranging in systems using non-transparent RSs with unique BSIDs and centralized scheduling, MR-BSs and non-transparent RSs shall perform the following tasks: