| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Direct Relaying Zone in Frame Structure for Transparent Mode |  |
| Date Submitted | 2007-09-09 |  |
| Source(s) | Su Chang Chae, Young-il Kim, Sunggeun Jin, Hyunjae Kim ETRI Kyu Ha Lee, Changyun Kim, Yong Wook Lee Samsung Thales Jai H. Eu, Jcast Networks Korea, Inc Hyung-joon Jeon POSDATA Jun Bae Ahn SOLiD technologies | E-mail:schae@etri.re.kr E-mail:kyuha.lee@samsung.com E-mail:jeu@jcastnet.com E-mail:hjjun@posdata.co.kr jbahn@st.co.kr:kyuha.lee@samsung.com |
| Re: | IEEE 802.16j-07/043:”IEEE 802.16 Working Group Letter Ballet #28” |  |
| Abstract | This contribution proposes direct relaying zone in frame structure for transparent mode |  |
| Purpose | Test proposal for C802 16j-D1 (August 2007),”Draft Standard for Local and Metropolitan Area Networks” |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Direct Relaying Zone in Frame Structure for Transparent Mode

# 1. Introduction

Transparent mode is not enough to provide fast traffic relaying. In order to reduce the latency to relay the traffic via transparent RS, we define the direct relaying zone in the transparent frame structure. In the direct relaying zone, intermediate RS relays traffic to its subordinates without latency within a single frame. For this purpose, intermediate RS demodulates traffic in an access region and forwards it through optional transparent zone by applying appropriate modulation.

# 2. Relaying Scheme

Figure 1 show direct relaying zone partitioned into two parts. The direct relaying zone in an access zone is used for intermediate RS to receive traffic from MR-BS. The other part in optional transparent zone is used to forward traffic to its subordinates. In this case, intermediate RS manipulates the traffic by adopting demodulation and Forward (M&F) scheme.

Figure 1 Direct Relaying Zone

M&F is a traffic transmission scheme to relay data without decoding and re-encoding. RS receives data by demodulating them, and then forwards the received data by modulating them. The intermediate RS requires only one OFDM symbol for the M&F scheme, and therefore, it can relay data successfully within a single frame.

This proposal provides a new scheme by using the direct relaying zone, where an intermediate RS relays data within the same frame by adopting M&F. For our direct relaying zone, MR-BS should notify information about direct relaying zone to RS.

# 3. M&F Scheme for Direct Relaying Zone

Figure-2 shows M&F scheme for direct relaying zone. In order to provide M&F, RS should demodulate data in the access zone, and then modulate them sequentially. For proper modulation, (1) RS may change modulation order of the demodulated data in the pilot and data modulation block. After that, (2) it generates PRBS by considering OFDM symbol offset value of the data. Finally, (3) it performs IFFT. The proposed M&F scheme needs at least one OFDM symbol so that RS can relay data directly within a single frame.

Figure 2 M&F procedure

# 3.1. Sub-carrier Randomization

When M&F is used, it is possible that the sub-carrier randomization block generates PRBS outputs according to symbol offset of the data, which RS will relay as shown by Figure-3.

Figure 3 PRBS outputs of M&F

# 3.2. Modulation Order Change

In order to change the modulation order for access link into that for relay link, it is well-known fact that QPSK, 16QAM and 64QAM require 2, 4 and 6 bits for a point in its constellation mapping, respectively. For this reason, if an RS tries to change the modulation order of data, which was modulated with QPSK, into 16QAM, the number of required slots shrinks to a half. Figure-4 shows this case.

Figure 4 example of changing the QPSK to 16QAM

FEC block size is determined by the modulation order and the number of allocated slots for a burst, and hence, it could change due to the modulation order change for direct relaying. In this case, there exists mismatching pair as to FEC block sizes employed in both access link and relay link. In order to avoid the mismatched pair during modulation order change, MR-BS has to prepare a MAP, which indicates the modulation order change for direct relaying zone, by considering the case. Table- shows every modulation order change, which can be employed for direct relaying zone. In this table-1, X indicates not-allowed modulation order change.

Slot boundary conditions	: nA·mA = nR·mR

Here,

nA : the number of slots allocated in the access zone,

nR  : the number of slots allocated in the relay zone,

mA : modulation order in the access zone,

mR : modulation order in the relay zone.

Table1 Encoding slot concatenation for different rates in CTC

| MR-BS |  | RS | MS |  |
| --- | --- | --- | --- | --- |
| Modulation and rate | j | feasible MCS for M&F | Modulation and rate | j |
| QPSK 1/2 | 10 | O | 16QAM 1/2 | 5 |
| QPSK 1/2 | 10 | O | 64QAM 1/2 | 3 |
| QPSK 3/4 | 6 | O | 16QAM 3/4 | 3 |
| QPSK 3/4 | 6 | O | 64QAM 3/4 | 2 |
| 16QAM 1/2 | 5 | O | QPSK 1/2 | 10 |
| 16QAM 1/2 | 5 | O | 64QAM 1/2 | 3 |
| 16QAM 3/4 | 3 | O | QPSK 3/4 | 6 |
| 16QAM 3/4 | 3 | O | 64QAM 3/4 | 2 |
| 64QAM 1/2 | 3 | X | QPSK 1/2 | 10 |
| 64QAM 1/2 | 3 | X | 16QAM 1/2 | 5 |
| 64QAM 3/4 | 2 | O | QPSK 3/4 | 6 |
| 64QAM 3/4 | 2 | O | 64QAM 3/4 | 2 |

# 3.3. Direct Relaying Zone IE

MR-BS indicates the information about direct relaying zone to its subordinate RSs by transmitting Direct Relaying Zone IE in order to reduce MAP overhead since the assigned direct relaying zone is configured rarely. We summarize the parameters for a configuration of direct relaying zone as follows:

a) Symbol offset for direct relaying zone for DL

The OFDMA symbol offset in which direct relaying zone starts for downlinks

b) Symbol offset for direct relaying zone for UL

The OFDMA symbol offset in which direct relaying zone starts for uplinks

c) No. OFDMA symbols for DL

The number of OFDMA symbols by which RS should relay signal without decoding and re-encoding for downlinks.

d) No. OFDMA symbols for UL

The number of OFDMA symbols by which RS should relay signal without decoding and re-encoding for uplinks.

Figure 5 Parameters of Direct Relaying Zone

Upon reception of these parameters, RS shall relay data in access link to its subordinates through relay link. Direct relaying zone is feasible without addition of new MAP IE. However, in order to support direct relaying zone, MR-BS has to construct MAP IE with the considerations as follows:

- RS utilizes CID-based data relaying.

- RS forwards data by employing burst-based relaying.

Figure 6 Example of relaying burst #1 to burst #2

# 4. Specific text changes

### 8.4.4.7.1 Frame Structure for Transparent Mode

[Insert new subclause 8.4.4.7.1.3:]

### 8.4.4.7.1.3 Direct Relaying Zone(Optional)

In the frame structure for transparent mode, direct relaying zone may be optionally indicated by MR-BS. In the direct relying zone, RS can relay the data of access zone through the relay zone within one frame by using demodulation-and-forward (M&F) scheme for downlink and uplink.

When supporting M&F, nA·mA allocated in the MR-BS shall be equal to nR·mR allocated in the MS, here nA is the number of slots allocated in the access zone, nR  is the number of slots allocated in the relay zone, and mA, mR is the modulation order (2 for QPSK, 4 for 16-QAM and 6 for 64-QAM).

[Change Table 385 as indicated:]

Table 385—Extended-2 DIUC code assignment for DIUC=14

| Extended-2 DIUC (hexadecimal) | Usage |
| --- | --- |
| 00 | MBS_MAP_IE |
| 01 | HO_Anchor_Active_DL_MAP_IE |
| 02 | HO_Active_Anchor_DL_MAP_IE |
| 03 | HO_CID_Translation_MAP_IE |
| 04 | MIMO_in_another_BS_IE |
| 05 | Macro-MIMO_DL_Basic_IE |
| 06 | Skip_IE |
| 07 | HARQ DL MAP IE |
| 08 | HARQ ACK IE |
| 09 | Enhanced DL MAP IE |
| 0A | Closed-loop MIMO DL Enhanced IE |
| 0C | RS-RNG_RSP_ALLOC_IE |
| 0D | DL Burst transmit IE |
| 0E | AAS_SDMA_DL_IE |
| 0F | Reserved Direct Relaying DL Zone IE |

[Insert a new subclause 8.4.5.3.30]

### 8.4.5.3.30 Direct Relaying DL Zone_IE format

Table xxx allows transparent RS to use M&F with direct relaying zone for downlink.

Table xxx - Direct Relaying DL Zone IE

| Syntax | Size | Notes |
| --- | --- | --- |
| Direct Relaying DL_Zone IE() { |  |  |
| Extended-2 DIUC= | 4 bits | 0x0C |
| Length | 8 bits | 0x03 |
| RSCID | 16bit | Basic CID of RS |
| Symbol offset for direct relaying zone for DL | 6 bits |  |
| No. OFDMA symbols for RS relaying for DL | 6 bits |  |

Symbol offset for direct relaying zone

The OFDMA symbol offset in which direct relaying zone starts for downlink

No. OFDMA symbols for RS receiving

The number of OFDMA symbols which RS should receive for relaying without decoding and encoding

No. OFDMA symbols of gap

The number of OFDMA symbols between the receiving area and the forwarding area

No. OFDMA symbols for RS relaying

The number of OFDMA symbols by which RS should relay signal without decoding and encoding for downlink

[Change Table 427 as indicated.]

Table 427—Extended UIUC Code Assignment for UIUC=15

| Extended UIUC (hexadecimal) | Usage |
| --- | --- |
| 00 | Power_control_IE |
| 01 | Mini-subchannel_allocation_IE |
| 02 | AAS_UL_IE |
| 03 | CQICH_Alloc_IE |
| 04 | UL Zone IE |
| 05 | PHYMOD_UL_IE |
| 06 | MIMO_UL_Basic_IE |
| 07 | UL-MAP_Fast_Tracking_IE |
| 08 | UL_PUSC_Burst_Allocation_in_Other_Segment_IE |
| 09 | Fast_Ranging_IE |
| 0A | UL Allocation Start IE |
| 0B | UL_Burst_Receive_IE |
| 0C | Direct Relaying UL Zone IE |
| 0D … 0F | Reserved |

[Insert a new subclause 8.4.5.4.42]

### 8.4.5.4.42 Direct Relaying Zone UL_IE format

Table yyy allows transparent RS to use M&F with direct relaying zone for uplink.

Table yyy - Direct Relaying UL Zone IE

| Syntax | Size | Notes |
| --- | --- | --- |
| Direct Relaying UL Zone IE() { |  |  |
| Extended UIUC= | 4 bits | 0x0B |
| Length | 8 bits | 0x03 |
| RSCID | 16bits | Basic CID of RS |
| Symbol offset for direct relaying UL zone for UL | 6 bits |  |
| No. OFDMA symbols for RS relaying for UL | 6 bits |  |

Symbol offset for direct relaying zone

The OFDMA symbol offset in which direct relaying zone starts for uplink

No. OFDMA symbols for RS receiving

The number of OFDMA symbols which RS should receive for relaying without decoding and encoding

No. OFDMA symbols of gap

The number of OFDMA symbols between the receiving area and the forwarding area

No. OFDMA symbols for RS relaying

The number of OFDMA symbols by which RS should relay signal without decoding and encoding for uplink

# References

C802 16j-06_294r14, “Direct Relaying Zone in Transparent RS system”

C802 16j-D1 (August 2007),”Draft Standard for Local and Metropolitan Area Networks”