| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On enhancing MCS for the relay link |  |
| Date Submitted | 2007-09-02 |  |
| Source(s) | Su Chang Chae, Young-il Kim, Hyunjae Kim ETRI Kyu Ha Lee, Changyun Kim, Yong Wook Lee Samsung Thales Aik Chindapol Siemens Adrian Boariu Nokia Jai H. Eu, Jcast Networks Korea, Inc Hyung-joon Jeon POSDATA Jun Bae Ahn SOLiD technologies | E-mail:schae@etri.re.kr E-mail:kyuha.lee@samsung.com E-mail:Aik.Chindalpol@siemens.com E-mail:Adrian.Boariu@nokia.com E-mail:jeu@jcastnet.com E-mail:hjjun@posdata.co.kr jbahn@st.co.kr |
| Re: | This is a response to the call for technical contributions IEEE 802.16j-07/019 |  |
| Abstract | Propose Enhanced MCS for Demodulation and Forward in Transparent RS |  |
| Purpose | To incorporate the proposed text into C802 16j-D1 (August 2007),”Draft Standard for Local and Metropolitan Area Networks” |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

On enhancing MCS for the relay link

# 1. Introduction

There is a need for expanding the number of MCSs that currently exists in the standard in order to offer finer granularity for transmissions over the relay link. The contribution targets CTC operation because it offers greater flexibility and easy implementation.

# 2. Proposed Text Change

### 8.4.4.7.1 Frame Structure for Transparent Mode

[Insert new subclause 8.4.4.7.1.3:]

### 8.4.4.7.1.3 Direct Relaying Zone(Optional)

In the frame structure for transparent mode, Direct Relaying Zone may be optionally indicated by MR-BS. In the direct relying zone, RS can relay the data of access zone through the relay zone within one frame by using demodulation-and-forward (M&F) scheme for downlink and uplink. When using Direct Relaying Zone, RS shall be negotiated by including TLV(11.8.3.7.29) of the Direct Relaying Zone for RS in the SBC-REQ/RSP messages.

[Insert new subclause11.9.3.7.29:]

### 11.8.3.7.29 Direct Relaying Zone for RS

In order to reduce the latency to relay the traffic via transparent RS, intermediate RS relays traffic to its subordinates without latency within a single frame. For this purpose, intermediate RS demodulates traffic in an access region and forwards it through relay zone by applying appropriate modulation.

Figure-xxx shows Direct Relaying Zone partitioned into two parts. The Direct Relaying Zone in an access zone is used for intermediate RS to receive traffic from MR-BS. The other part in optional transparent zone is used to forward traffic to its subordinates. In this case, intermediate RS manipulates the traffic by adopting demodulation and Forward (M&F) scheme.

M&F is a traffic transmission scheme to relay data without decoding and re-encoding. RS receives data by demodulating them, and then forwards the received data by modulating them. The intermediate RS requires only one OFDM symbol for the M&F scheme, and therefore, it can relay data successfully within a single frame.

In order to change the modulation order for access link into that for relay link, it is well-known fact that QPSK, 16QAM and 64QAM require 2, 4 and 6 bits for a point in its constellation mapping, respectively. For example, if an RS tries to change the modulation order of data, which was modulated with QPSK, into 16QAM, the number of required slots shrinks to a half.

FEC block size is determined by the modulation order and the number of allocated slots for a burst, and hence, it could change due to the modulation order change for direct relaying. In this case, there exists mismatching pair as to FEC block sizes employed in both access link and relay link. In order to avoid the mismatched pair during modulation order change, MR-BS has to prepare a MAP, which indicates the modulation order change for Direct Relaying Zone, by considering the case. An MR-BS shall provide Direct Relaying Zone according to following equation:

nA·mA = nR·mR

where,

nA : the number of slots allocated in the access zone,

nR  : the number of slots allocated in the relay zone,

mA : modulation order in the access zone(2 for QPSK, 4 for 16-QAM and 6 for 64-QAM),

mR : modulation order in the relay zone(2 for QPSK, 4 for 16-QAM and 6 for 64-QAM).

Note the parameters and configuration of Direct Relaying Zone as follows:

a) Symbol offset for Direct Relaying Zone for DL

The OFDMA symbol offset in which Direct Relaying Zone starts for downlinks

b) Symbol offset for Direct Relaying Zone for UL

The OFDMA symbol offset in which Direct Relaying Zone starts for uplinks

c) No. OFDMA symbols for DL

The number of OFDMA symbols by which RS should relay signal without decoding and re-encoding for downlinks.

d) No. OFDMA symbols for UL

The number of OFDMA symbols by which RS should relay signal without decoding and re-encoding for uplinks.

Figure-xxx Parameters of Direct Relaying Zone

This TLV shall be used to indicate Direct Relaying Zone in transparent RS which transmit signals to be relayed through relay zone within one frame.

| Type (1byte) | Length | Value | scope |
| --- | --- | --- | --- |
| TBD | 4 | Bit#0~5, Symbol offset for Direct Relaying Zone for DL: The OFDMA symbol offset in which Direct Relaying Zone starts for downlinks. Bit#6~11, No. OFDMA symbols for DL: The number of OFDMA symbols by which RS should relay signal without decoding and re-encoding for downlinks. Bit#12~17, Symbol offset for Direct Relaying Zone for UL: The OFDMA symbol offset in which Direct Relaying Zone starts for uplinks. Bit#18~23, No. OFDMA symbols for UL: The number of OFDMA symbols by which RS should relay signal without decoding and re-encoding for uplinks. | SBC-REQ SBC-RSP |

[Add the following text at the end of section 8.4.9.2.3.1]

On the RS link when CTC is used the data bursts may be encoded using the Table xxx.

Table xxx Encoding slot concatenation for different rates in CTC

| Modulation and rate | j |
| --- | --- |
| QPSK 1/2 | 10 |
| QPSK 2/3 | 6 |
| QPSK 3/4 | 6 |
| QPSK 5/6 | 6 |
| 16QAM 1/2 | 5 |
| 16QAM 2/3 | 3 |
| 16QAM 3/4 | 3 |
| 16QAM 5/6 | 3 |
| 64QAM 1/2 | 3 |
| 64QAM 2/3 | 2 |
| 64QAM 3/4 | 2 |
| 64QAM 5/6 | 2 |

## 11.3.1.1 Uplink burst profile encodings

[Insert sub-clause and new Table 612-xx in 11.3.1.1 “Uplink burst profile encodings” as shown]

Table 612-xx shall be used only in the RS link

Table 612-xx-UCD burst profile encodings-Wireless MAN-OFDMA

| Name | Type | Length | Value |  |
| --- | --- | --- | --- | --- |
| FEC Code Type and modulation type | 150 | 1 | 0=QPSK(CC) 1/2 1=QPSK(CC)3/4 2=16-QAM(CC)1/2 3=16-QAM(CC)3/4 4=64-QAM(CC)1/2 5=64-QAM(CC)2/3 6=64-QAM(CC)3/4 7=QPSK(BTC)1/2 8=QPSK(BTC)3/4 9=16-QAM(BTC)3/5 10=16-QAM(BTC)4/5 11=64-QAM(BTC)5/8 12=64-QAM(BTC)4/5 13=QPSK(CTC)1/2 14=reserved 15=QPSK(CTC)3/4 16=16-QAM(CTC)1/2 17=16-QAM(CTC)3/4 18=64-QAM(CTC)1/2 19=64-QAM(CTC)2/3 20=64-QAM(CTC)3/4 21=64-QAM(CTC)5/6 22=QPSK(ZT CC) 1/2 23=QPSK(ZT CC)3/4 24=16-QAM(ZT CC)1/2 25=16-QAM(CC)3/4 26=64-QAM(ZT CC)1/2 | 27=64-QAM(ZT CC)2/3 28=64-QAM(ZT CC)3/4 29=QPSK(LDPC)1/2 30= QPSK(LDPC)2/3 A code 31=QPSK(LDPC)3/4 A code 32=16-QAM(LDPC)1/2 33=16-QAM(LDPC)2/3 A code 34=16-QAM((LDPC)3/4 A code 35=64-QAM(LDPC)1/2 36=64-QAM(LDPC)2/3 A code 37=64QAM(LDPC)3/4 A code 38=QPSK(LDPC)2/3 B code 39= QPSK(LDPC)3/4 B code 40=16-QAM((LDPC)2/3 B code 41=16-QAM((LDPC)3/4 B code 42=64-QAM(LDPC)2/3 B code 43=64QAM(LDPC)3/4 B code 44=QPSK(CC with optional inter leaver)1/2 45=QPSK(CC with optional inter leaver)3/4 46=16-QAM(CC with optional inter leaver)1/2 47=16-QAM(CC with optional inter leaver)3/4 48=64-QAM(CC with optional inter leaver)1/2 49=64-QAM(CC with optional inter leaver)3/4 50=QPSK(LDPC)5/6 51=16-QAM(LDPC)5/6 52=64-QAM(LDPC)5/6 53=QPSK(CTC)2/3 54=QPSK(CTC)5/6 55=16-QAM(CTC)2/3 56=16-QAM(CTC)5/6 5357..255=Reserved |

## 11.4.2 Downlink burst profile encodings

[Insert sub-clause and new Table 619-xx in 12.4.2 “Uplink burst profile encodings” as shown]

Table 619-xx shall be used only on the RS link.

Table 619-xx-UCD burst profile encodings-Wireless MAN-OFDMA

| Name | Type | Length | Value |  |
| --- | --- | --- | --- | --- |
| FEC Code Type and modulation type | 150 | 1 | 0=QPSK(CC) 1/2 1=QPSK(CC)3/4 2=16-QAM(CC)1/2 3=16-QAM(CC)3/4 4=64-QAM(CC)1/2 5=64-QAM(CC)2/3 6=64-QAM(CC)3/4 7=QPSK(BTC)1/2 8=QPSK(BTC)3/4 9=16-QAM(BTC)3/5 10=16-QAM(BTC)4/5 11=64-QAM(BTC)5/8 12=64-QAM(BTC)4/5 13=QPSK(CTC)1/2 14=reserved 15=QPSK(CTC)3/4 16=16-QAM(CTC)1/2 17=16-QAM(CTC)3/4 18=64-QAM(CTC)1/2 19=64-QAM(CTC)2/3 20=64-QAM(CTC)3/4 21=64-QAM(CTC)5/6 22=QPSK(ZT CC) 1/2 23=QPSK(ZT CC)3/4 24=16-QAM(ZT CC)1/2 25=16-QAM(CC)3/4 26=64-QAM(ZT CC)1/2 | 27=64-QAM(ZT CC)2/3 28=64-QAM(ZT CC)3/4 29=QPSK(LDPC)1/2 30= QPSK(LDPC)2/3 A code 31=QPSK(LDPC)3/4 A code 32=16-QAM(LDPC)1/2 33=16-QAM(LDPC)2/3 A code 34=16-QAM((LDPC)3/4 A code 35=64-QAM(LDPC)1/2 36=64-QAM(LDPC)2/3 A code 37=64QAM(LDPC)3/4 A code 38=QPSK(LDPC)2/3 B code 39= QPSK(LDPC)3/4 B code 40=16-QAM((LDPC)2/3 B code 41=16-QAM((LDPC)3/4 B code 42=64-QAM(LDPC)2/3 B code 43=64QAM(LDPC)3/4 B code 44=QPSK(CC with optional inter leaver)1/2 45=QPSK(CC with optional inter leaver)3/4 46=16-QAM(CC with optional inter leaver)1/2 47=16-QAM(CC with optional inter leaver)3/4 48=64-QAM(CC with optional inter leaver)1/2 49=64-QAM(CC with optional inter leaver)3/4 50=QPSK(LDPC)5/6 51=16-QAM(LDPC)5/6 52=64-QAM(LDPC)5/6 53=QPSK(CTC)2/3 54=QPSK(CTC)5/6 55=16-QAM(CTC)2/3 56=16-QAM(CTC)5/6 5357..255=Reserved |

## 8.4.9.2.3.2 CTC interleaver

[Insert the sub-clause and table 566-xx below the Table 566 as indicated:]

Table 566-xx shall be used for the RS link only.

Table 566-xx CTC Channel coding per modulation for RS link

| Modulation | Data block size (bytes) | Encoded data block size (bytes) | Code rate | N | P0 | P1 | P2 | P3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 6 | 12 | 1/2 | 24 | 5 | 0 | 0 | 0 |
| QPSK | 12 | 24 | 1/2 | 48 | 13 | 24 | 0 | 24 |
| QPSK | 18 | 36 | 1/2 | 72 | 11 | 6 | 0 | 6 |
| QPSK | 24 | 48 | 1/2 | 96 | 7 | 48 | 24 | 72 |
| QPSK | 30 | 60 | 1/2 | 120 | 13 | 60 | 0 | 60 |
| QPSK | 36 | 72 | 1/2 | 144 | 17 | 74 | 72 | 2 |
| QPSK | 48 | 96 | 1/2 | 192 | 11 | 96 | 48 | 144 |
| QPSK | 54 | 108 | 1/2 | 216 | 13 | 108 | 0 | 108 |
| QPSK | 60 | 120 | 1/2 | 240 | 13 | 120 | 60 | 180 |
| QPSK | 24 | 36 | 2/3 | 96 | 7 | 48 | 24 | 72 |
| QPSK | 48 | 72 | 2/3 | 192 | 11 | 96 | 48 | 144 |
| QPSK | 9 | 12 | 3/4 | 36 | 11 | 18 | 0 | 18 |
| QPSK | 18 | 24 | 3/4 | 72 | 11 | 6 | 0 | 6 |
| QPSK | 27 | 36 | 3/4 | 108 | 11 | 54 | 56 | 2 |
| QPSK | 36 | 48 | 3/4 | 144 | 17 | 74 | 72 | 2 |
| QPSK | 45 | 60 | 3/4 | 180 | 11 | 90 | 0 | 90 |
| QPSK | 54 | 72 | 3/4 | 216 | 13 | 108 | 0 | 108 |
| QPSK | 30 | 36 | 5/6 | 120 | 13 | 60 | 0 | 60 |
| QPSK | 60 | 72 | 5/6 | 240 | 13 | 120 | 60 | 180 |
| 16-QAM | 12 | 24 | 1/2 | 48 | 13 | 24 | 0 | 24 |
| 16-QAM | 24 | 48 | 1/2 | 96 | 7 | 48 | 24 | 72 |
| 16-QAM | 36 | 72 | 1/2 | 144 | 17 | 74 | 72 | 2 |
| 16-QAM | 48 | 96 | 1/2 | 192 | 11 | 96 | 48 | 144 |
| 16-QAM | 60 | 120 | 1/2 | 240 | 13 | 120 | 60 | 180 |
| 16-QAM | 48 | 72 | 2/3 | 192 | 11 | 96 | 48 | 144 |
| 16-QAM | 18 | 24 | 3/4 | 72 | 11 | 6 | 0 | 6 |
| 16-QAM | 36 | 48 | 3/4 | 144 | 17 | 74 | 72 | 2 |
| 16-QAM | 54 | 72 | 3/4 | 216 | 13 | 108 | 0 | 108 |
| 16-QAM | 60 | 72 | 5/6 | 240 | 13 | 120 | 60 | 180 |
| 64-QAM | 18 | 36 | 1/2 | 72 | 11 | 6 | 0 | 6 |
| 64-QAM | 36 | 72 | 1/2 | 144 | 17 | 74 | 72 | 2 |
| 64-QAM | 54 | 108 | 1/2 | 216 | 13 | 108 | 0 | 108 |
| 64-QAM | 24 | 36 | 2/3 | 96 | 7 | 48 | 24 | 72 |
| 64-QAM | 48 | 72 | 2/3 | 192 | 11 | 96 | 48 | 144 |
| 64-QAM | 27 | 36 | 3/4 | 108 | 11 | 54 | 56 | 2 |
| 64-QAM | 54 | 72 | 3/4 | 216 | 13 | 108 | 0 | 108 |
| 64-QAM | 30 | 36 | 5/6 | 120 | 13 | 60 | 0 | 60 |
| 64-QAM | 60 | 72 | 5/6 | 240 | 13 | 120 | 60 | 180 |