| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Direct MAP Relay support for centralized scheduling |  |
| Date Submitted | 2007-09-18 |  |
| Source(s) | Young-jae Kim, Kyu Ha Lee, Jae Hyung Eom , Changkyoon Kim Samsung Thales San 14, Nongseo-Dong, Giheung-Gu, Yongin , Gyeonggi-Do, Korea 449-712 Byung-Jae Kwak, Young-il Kim ETRI 161, Gajeong-Dong, Yuseong-Gu, Daejeon, Korea 205-350 Aik Chindapol Siemens Corporate Research 755 College Road East Princeton, NJ, USA Yousuf Saifullah Nokia 6000 Connection Drive, Irving, TX Jun Bae Ahn SOLiD Technologies 10th Fl., IT Venture Tower East Wing, 78 Garak-Dong, Songpa-Gu, Seoul, Korea 138-803 Jai H. Eu Jcast Networks Korea, Inc. Anyang K-Center #1004 Kwanyang-Dong 1591-9, Dongan-Gu, Anyang-Si, Gyeonggi-Do, Korea 431-815 | Voice: +82-31-280-9975 Fax : +82-31-280-1562 youngjae2.kim@samsung.com kyuha.lee@samsung.com Voice: +82-42-860-6618 Fax: +82-42-861-1966 bjkwak@etri.re.kr yikim@etri.re.kr Voice: +1 609 734 3364 Fax: +1 609 734 6565 aik.chindapol@siemens.com Voice: +1 972 894 5000 Yousuf.Saifullah@nokia.com Voice: +82 2 2142 3861 Fax: +82 2 2142 3999 jbahn@st.co.kr Voice : +82 31 476 6989 Fax : +82 31 476 6930 jeu@jcastnet.com |
| Re: | IEEE 802.16j-07/043:”IEEE 802.16 Working Group Letter Ballet #28” |  |
| Abstract | The document contains technical proposals for IEEE P802.16j that provides TLV for simultaneous reception and transmission support |  |
| Purpose | Text proposal for C802 16j-D1 (August 2007),”Draft Standard for Local and Metropolitan Area Networks |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Direct MAP Relay Support for centralized scheduling

1. Introduction.

The RSs with centralized scheduling such as virtual group has been introduced to support to share preamble, FCH and MAP. When the virtual RS group includes MR-BS, each member RS of the group shall either transmit the same preamble, FCH and MAP as the MR-BS or they all do not transmit any preamble, FCH or MAP. When the virtual RS group doesn’t include MR-BS, one of the RSs in virtual group is non-transparent RS and all the others shall either transmit the preamble, FCH and MAP of the said non-transparent RS or they all do not transmit preamble, FCH and MAP. If a member RS of group wants to transmit preamble, FCH and MAP, those information should be received in relay zone of previous frame. This causes a little overhead in radio resource in relay zone of previous frame. In this contribution, we propose a method to reduce the overhead by relaying FCH and MAP to subordinate terminals directly

2. Direct MAP Relay

Direct MAP Relay function works like a repeater for MAP period.

There are many types of IEs in MAP. Some are about burst allocation and others are about control. When RS relay full MAP through relay zone, all those information is delivered to subordinate RS. This scheme is shown in Figure 1.

Figure 1 Example of FCH, MAP relaying without Direct Relaying

But RS doesn’t need all the information in MAP to function properly. Some information is necessary only for MS. Some information may not necessary for RS. For those information, RS just need to deliver them to subordinate terminal. With Direct Relaying, RS may receive only necessary MAP data. Because member RS repeats the whole FCH and MAP from MR-BS or super-ordinate RS with Direct Relaying function, subordinate terminal can receive the original FCH and MAP. Thus we can save some radio resource which could be allocated to relay some MAP information unnecessary for RS without Direct MAP Relay function. This scheme is shown in Figure 2.

Figure 2 Example of FCH, MAP relaying with Direct Relaying

3. Proposed Text change.

#### Insert followings at the end of section 6.3.2.3.23

Direct MAP Relay

- This parameter is sent by RS to indicate its capability of Direct MAP Relay .

In Direct MAP Relay, RS performs repeater function during MAP period. While RS is repeating FCH and MAP, RS may receive the MAP. If MAP does not occupy all the subchannels in a symbol pair, rest subchannels shall not be allocated to RS.

##### Add followings after the third characteristics in 6.3.9.16.3.1

If a RS member of virtual group is to transmit preamble, FCH and MAP, in Direct MAP Relay , only burst allocation information of MAP shall be received in previous frame and other IEs are relayed directly by RS.

#### Insert the followings at section 11.8.3.7.xx

This field indicates the availability of RS for Direct MAP Relay

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| TBA | 1 | Bit #0: Direct MAP Relay support Bit #1-7 : reserved | SBC-REQ SBC-RSP |

During the network entry process, RS sends SBC-REQ with Direct MAP Relay support capability(set bit #0 to 1, if capable) to MR-BS or super-ordinate RS and MR-BS or super-ordinate RS responds with SBC-RSP to notify whether to use Direct MAP Relay function or not.

Change Table 496c of section 8.4.5.9.1 as indicated:

Table 496c-R-link specific IE format

| Extended DIUC (hexadecimal) | # Usage |
| --- | --- |
| 00 | RS_UL_DCH assignment IE |
| 01 | Map_End_Location_IE |
| 0x012-1F | ## Reserved |

Insert section 8.4.5.3.29 as indicated:

8.4.5.9.2 Map_End_Location_IE

This IE is issued by the MR-BS or super-ordinate RS to member RS for Direct Relay of MAP. To relay them directly, RS should know the end point of MAP in unit of OFDMA symbol

Table xxx – Map End Location IE

| Syntax | # Size (bits) | Notes |
| --- | --- | --- |
| Map_End_Location_IE(){ |  |  |
| Type | 5 bits | Map_End_Location_IE=01 |
| Length | 4 bits | Length = 0x2 |
| Map End Indication | 8 bits | This field indicates the last OFDMA symbol index of MAP of next frame |
| Reserved | 8bits | Shall be set to zero |
| } |  |  |