| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on AK Transfer |  |
| Date Submitted | 2007-09-14 |  |
| Source(s) | Sergey Seleznev, Hyoung Kyu Lim, Hyunjeong Kang, Jungje Son Samsung Electronics Rep. of Korea, Gyonggi-do, Suwon | Voice: +82312795968 E-mail: s.sergey@samsung.com |
| Re: | IEEE 802.16j-07/045 |  |
| Abstract | This contribution proposes text changes to clarify security context distribution in a distributed security model. |  |
| Purpose | Discuss and adopt proposed text changes in reply to comments. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarification on AK Transfer

Sergey Seleznev et. al.

Samsung Electronics

# Introduction

This contribution attempt is to harmonize comments #239, 241, 242

On comment 239: Table 50 is updated; descriptions are added; AK-Parameters are defined.

On comment 241: Descriptions are added, table titles are added.

On comment 242: GSA is deleted. However, along with the AK AR-RS shall receive a set of SA-descriptors from the MR-BS, since RS is not aware of the services MS authorized to access. SA-descriptors are added.

# Proposed text changes

[Change subclause 6.3.2.3.9.29 as follows]

In MR system with distributed security control, the MR-BS shall send PKMv2 AK Transfer message to MS’s access AR-RS, when AK is delivered to the MR-BS.

Table Xx – PKMv2 AK Transfer message

| Attribute | Contents |
| --- | --- |
| Key Sequence Number | AR RS AK sequence number |
| SAID | AR RS primary SAID |
| SAID | MS/RS’s primary SAID |
| AK | MS/RS’s authorization key |
| Key Sequence Number | MS/RS’s AK sequence number |
| Key Lifetime | MS/RS’s AK lifetime |
| Group SA Descriptor | TLV that specifies GSAID and additional properties of that SA |
| AK-Parameters | AK related parameters defined in 11.9.42 |
| Frame Number | An absolute frame number in which the old PMK and all its associate AKs should be discarded. |
| (one or more) SA-Descriptor(s) | Each compound SA-Descriptor attribute specifies SAID and additional properties of the SA. |
| PKMv2 configuration settings | PKMv2 configuration defined in 11.9.36 |
| Nonce | A random number generated in an MR-BS |
| HMAC/CMAC Digest | Message authentication digest |

[Change subclause 6.3.2.3.9.30 as follows]

AR-RS shall send PKMv2 AK Transfer Ack message to the MR-BS in reply to PKMv2 AK Tranfer message in order to securely acknowledge key reception.

Table Xx – PKMv2 AK Transfer Ack

| Attribute | Contents |
| --- | --- |
| Key Sequence Number | AR RS AK sequence number |
| SAID | AR RS primary SAID |
| Key Sequence Number | MS/RS’s AK sequence number |
| SAID | MS/RS’s primary SAID |
| Nonce | A same random number included in the PKMv2 AK Transfer message |
| HMAC/CMAC Digest | Message authentication digest |

[Insert the following rows in Table 50]

| Code | PKM message type | MAC management message name |
| --- | --- | --- |
| 31 | PKMv2 AK Transfer | PKM-RSP |
| 32 | PKMv2 AK Tranfer Ack | PKM-RSP |
| 33-255 | Reserved | - |

[Insert the following subclause 11.9.42]

11.9.42 AK-Parameters

This attribute is a compound attribute, consisting of a collection of sub-attributes. These sub-attributes represent all the security parameters relevant to a particular Authorization key of MS or RS.

| Type | Length | Value |
| --- | --- | --- |
| TBD | variable | The compound field contains the sub-attributes as defined in the Table Xx. |

Table Xx—AK-Parameters definition

| AK | 160 | MS/RS’s authorization key |
| --- | --- | --- |
| AK Sequence Number | 8 | MS/RS’s AK sequence number |
| AK Lifetime | - | MS/RS’s AK remaining lifetime |