| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of Tunnel QoS |  |
| Date Submitted | 2007-11-08 |  |
| Source(s) | Haihong Zheng, Yousuf Saifullah, Shashikant Maheshwari Nokia Siemens Networks 6000 Connection Drive, Irving, TX 75019 USA | Voice: 972-894-5000 Email: Haihong.Zheng@nsn.com |
| Re: | IEEE 802.16j-06/027: “Call for Technical Proposals regarding IEEE Project P802.16j” |  |
| Abstract | This proposal clarifies the DSx messages used over relay links. |  |
| Purpose | Discuss and adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein.. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Tunnel Service Flows

# 1. Introduction

There are some comments (comment #717 and #696) on how to management tunnel service flow in 80216-07_045. This contribution proposes text change to address these related comments.

# 2. Specific Text Change

[Replace section 6.3.14.10 with following text:]

6.3.14.10 QoS for Tunnel

6.3.14.10.1  Tunnel Service Flows

In multihop relay system, one or more tunnel connection may be established between the MR-BS and an RS. The tunnel connection is unidirectional (in either direction) and is used to carry MPDUs from multiple individual service flows for SSs. Each tunnel is associated with a tunnel service flow and shall be assigned a connection identifier (T-CID or MT-CID) and optionally a set of service flow parameters. No Service Flow Identifier (SFID) is needed for a tunnel. The service flow parameters specify the QoS requirement of the tunnel. The tunnel CIDs are drawn from the same space as CIDs assigned to individual SS, however, a specific range of CIDs is assigned to support tunnels.

6.3.14.10.1.1 Tunnel Service Flow in Centralized Scheduling Mode

With centralized scheduling mode, if use of tunnel is specified, only one tunnel is established between the MR-BS and a RS. No service flow parameters are assigned to this tunnel. The T-CID and MT-CID are assigned or removed in the same way as transport CIDs are assigned or removed for SS using DSx procedures.

6.3.14.10.1.2 Tunnel Service Flow in Distributed Scheduling Mode

With distributed scheduling mode, if use of tunnel is specified, one or more tunnel may be established between the MR-BS and an access RS. Each tunnel shall be assigned with a set of service flow parameters. Individual connections for SSs that have the similar QoS requirement may be mapped to the same tunnel. The service flow parameter associated with a tunnel connection is determined by the MR-BS. MR-BS shall ensure that the service flow parameters defined for the tunnel meets the QoS requirement of all the individual connections traversing the tunnel. When new individual connections for SS are added to or removed from a tunnel or the QoS requirement of the individual connection for SS is changed, and this leads to a change of QoS requirement for the tunnel, the MR-BS also shall modify the service flow parameter for the tunnel. The service flow management procedure (i.e., DSA, DSC, DSD procedures) for the tunnel connection are described in 6.3.14.9.3, 6.3.14.9.4 and 6.3.14.9.5.

The service flow parameters associated with a tunnel are listed below. Depending on the type of data delivery services, a subset of the following parameters is included.

- Type of Data Delivery Services (11.13.25)

- Maximum Sustained Traffic Rate (11.13.6)

- Maximum Traffic Burst (11.13.7)

- Maximum Reserved Traffic Rate (11.13.8)

- Vendor-specific QoS parameters (11.13.10)

- Tolerated Jitter (11.13.13)

- Maximum Latency (11.13.14)

- Unsolicited Grant Interval (11.13.20)

- Unsolicited Polling Interval (11.13.21)

- Traffic Priority (11.13.5)