| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | PLI Count Remedy |  |
| Date Submitted | 2007-09-17 |  |
| Source(s) | Shang Zheng Peng Yan | Voice: +86-21-50993165 E-mail: shang_zheng@huawei.com Voice : +86-21-50993162 Email: peng.yan@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Technical Proposals regarding IEEE Project P802.16j (IEEE 802.16j-07/007r2) |  |
| Abstract | This contribution is used to clarify the calculation of the PLI count |  |
| Purpose | Add proposed spec changes in P802.16j Baseline Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

PLI Count Remedy

Shang Zheng, Peng Yan

Huawei Technologies Co. Ltd

# 1. Introduction

When there is no frame number offset between the MR-BS and each subordinate RSs. The PLI count calculation is shown below.

Considering the frame number offset, the PLI count calculation is shown below.

# 2. Proposed Text Changes

---------------------------------------------------------Start of the Text----------------------------------------------

[Adopt the following text modification to the P802.16j Draft document]

Modify the Section 6.3.2.3.56 (Table 174) in page 30

Table 174—MOB_PAG-ADV message format

| Syntax | Size | Notes |
| --- | --- | --- |
| MOB_PAG-ADV_Message_format() { | — | — |
| Management Message Type=62 | 8 bits | — |
| Num_Paging_Group_IDs | 16 bits | Number of Paging Group IDs in this message |
| For (i=0; i<Num_Paging_Group_IDs; i++) { | — | — |
| Paging Group ID | 16 bits | — |
| } | — | — |
| Num_MACs | 8 bits | Number of MS MAC addresses |
| For (j=0; j<Num_MACs; j++){ | — | — |
| MS MAC Address hash | 24 bits | The hash is obtained by computing a CRC24 on the MS 48-bit MAC address. The polynomial for the calculation is 0x1864CFB |
| Action Code | 2 bits | Paging action instruction to MS 0b00=No Action Required 0b01=Perform Ranging to establish location and acknowledge message 0b10=Enter Network 0b11=Reserved |
| Stop Paging | 1 bit | 0b0= paging start command 0b1= paging stop command |
| PLI Count | 32 bits | The number that PLI has been elapsed since the first time MR-BS sending out the MOB_PAG-ADV message Number of the PLI of its superordinate station has been elapsed at the current receiving frame. The maximum value of PLI Count is ‘0b10’. 0b11: The PLI Count filed shall be ignored by RS. |
| Reserved | 23 bits | — |
| } | — | — |
| padding | variable | Padding bits to ensure octet aligned |
| TLV Encoded Information | variable | TLV specific |
| } | — | — |

Modify the Section 6.3.2.3.56 (Table 174) in page 30 Line 35 as indicated

PLI Count

This field indicates the number MOB_PAG-ADV messages have already been sent over the access link during the current PLI. This field is used for indicating the RS how many PLI of its superordinate station has been elapsed. The RS will determine shall calculate its own paging retry count according to by subtract the “PLI Count” and from the “Paging Retry Count” of MR-BS. When a RS relay the MOB_PAG-ADV message to its subordinate RSs, the “PLI Count” value shall be increased by one if the receiving RS will miss one more MOB_PAG-ADV message transmission over the access link there is one PLI has been elapsed at the current sending frame. The maximum value of PLI Count is ‘0b10’. When the PLI Count has reached the maximum value, it shall not be increased any more.

When this filed is set to ‘0b11’ by MR-BS, the RS shall ignore this field and using the same “Paging Retry Count” as the MR-BS.

Replace the fifth paragraph of the section 6.3.24.6.1 in Page 144 Line 21 as indicated

In order to let each RS’s paging retry count decrease to zero at the same time with MR-BS, a “PLI Count” indication field may be included in the MOB_PAG-ADV message transmitted in the relay link, which is described in 6.3.2.3.56. This field is used for indicating the RS how many MOB_PAG-ADV messages have already been sent over the access link during the current PLI. The RS will determine its own paging retry count according to the “PLI Count” and the “Paging Retry Count” of MR-BS defined in Table 342.

In order to let MR-BS wait reasonable time for paging response, a “PLI Count” field may be included in the MOB_PAG-ADV message transmitted in the relay link, which is described in 6.3.2.3.56. When this “PLI Count” filed is included, each RS shall calculate its own paging retry count by subtract the “PLI Count” from the “Paging Retry Count” of MR-BS. When this filed is set to ‘0b11’ by MR-BS, the RS shall ignore this field and using the same “Paging Retry Count” as the MR-BS.

Modify the Section 6.3.24.6.1 in Page 144 Line 27 as indicated

When a RS relay the MOB_PAG-ADV message to its subordinate RSs with “PLI Count” field embedded, the “PLI Count” value may shall be increased by one if the receiving RS will miss one more MOB_PAG-ADV message transmission over the access link there is one PLI has been elapsed at the current sending frame. The maximum value of PLI Count is ‘0b10’. When the PLI Count has reached the maximum value, it shall not be increased any more. The MR-BS shall guarantee that the value of “Paging Retry Count” of MR-BS is greater than the maximum value of “PLI Count”, so that the RS in the last hop broadcast paging message in the access link at least once.

---------------------------------------------------------End of the Text-----------------------------------------------

# 3. Reference

[1]	P802.16j/D1, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems Multihop Relay Specification”