| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Cooperative Relaying |  |
| Date Submitted | 2007-09-17 |  |
| Source(s) | Aik Chindapol, Jimmy Chui, Adrian Boariu, Shashikant Maheshwari, Haihong Zheng, Yousuf Saifullah, Peter Wang Nokia Siemens Networks Kyu Ha Lee, Young-Jae Kim, Changkyoon Kim Samsung Thales San 14, Nongseo-Dong, Giheung-Gu, Yongin, Gyeonggi-Do, Korea 449-712 Byung-Jae Kwak, Sungcheol Chang, D. H. Ahn, Young-il Kim ETRI 161, Gajeong-Dong, Yuseong-Gu, Daejeon, Korea 305-350 | aik.chindapol@siemens.com Voice: +82-31-280-9917 Fax: +82-31-280-1562 kyuha.lee@samsung.com Voice: +82-42-860-6618 Fax: +82-42-861-1966 bjkwak@etri.re.kr |
| Re: | This is in response for call for comments P802.16j/D1 |  |
| Abstract | Clarification on Cooperative Relaying |  |
| Purpose | Review and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarification on Cooperative Relaying

NSN, Samsung Thales, ETRI

# Introduction

This contribution is a response to Comment 401 and merges RS_CDC-REQ into the RS_CD zone configuration IE to reduce overhead and to allow per-zone configuration.

# Specification changes

[Remove Section 6.3.2.3.62 and Table 183a]

[Remove the following line from Table 38]

[Change Table 183zzz in Section 6.3.2.3.73 as follows]

Table 183zzz Zone Configuration IE

| Zone Configuration IE _Format() { | - | - |
| --- | --- | --- |
| Zone Configuration bitmap | 8 bits | b0=1, permutation based included b1=1, range of subchannels included b2=1, STC mode b3=1, Cooperative diversity configuration b4=1, AMC mode b5=1, Safety/PAR reduction mode b6-b7: reserved |
| if (b0 of Zone Configuration bitmap ==1) { |  |  |
| Permutation based | 6 bits | DL_PermBase to be used in this zone |
| Reserved | 2 bits |  |
| } | - | - |
| if (b1 of Zone Configuration bitmap ==1) { | - | - |
| Min Subchannel index | 8 bits | The index of subchannel from which the allocation starts. |
| Max Subchannel index | 8 bits | The index of subchannel at which the allocation ends. |
| } | - | - |
| if (b2 of Zone Configuration bitmap ==1) { | - | - |
| STC mode | TBD |  |
| } | - | - |
| if (b3 of Zone Configuration bitmap ==1) { | - |  |
| Num_Relay_Antennas | 2 bits | Number of antennas to be used at the relay. If this value is zero, cooperative diversity is not enabled. |
| for (i=0; i<Num_Relay_Antennas; i++) { |  |  |
| Antenna assignment for relay antenna i | 2 bits | Relay antenna i will transmit as antenna 0, 1, 2, or 3, according to the 2-bit value in this field, MSB first. (Section 8.4.8.10) |
| } |  |  |
| Padding | variable | Padding to ensure byte aligned. |
| } | - | - |
| if (b4 of Zone Configuration bitmap ==1) { | - | - |
| AMC mode | TBD |  |
| } | - | - |
| if (b5 of Zone Configuration bitmap ==1) { | - | - |
| Safety/PAR reduction mode | TBD |  |
| } | - | - |
| } | - | - |

[Change Section 8.4.8.10 as follows]

8.4.8.10 Cooperative Relaying

Cooperative relaying can be achieved within an MR-cell using either an MR-BS and one or more RSs or multiple RSs transmitting in cooperation. These RSs are either transparent or non-transparent transmitting the same frame start preamble, FCH and MAPs. It is possible to achieve diversity by sending appropriately coded signals across different MR-BS and RS transmit antennas during the transmission of a burst to subordinate stations. The three modes of operation for cooperative relaying are cooperative source diversity, cooperative transmit diversity, and cooperative hybrid diversity.

For cooperative source diversity, the transmitting antennas simultaneously transmit the same signal using the same time-frequency resource. In cooperative transmit diversity mode, STC-encoded signals are transmitted across the transmitting antennas using the same time-frequency resource (refer to Section 8.4.8 for a list of valid STCs). Cooperative hybrid diversity is identical to cooperative transmit diversity, except that at least one antenna assignment is given to multiple antennas.

Cooperative transmit diversity and cooperative hybrid diversity are achieved by the cooperative diversity configuration parameter in the Zone Configuration IE of Table 183o.  This parameter defines an antenna assignment for each RS antenna for the specified zone.  The antenna assignment identifies how the RS shall encode the data, and how the RS shall transmit pilots, for the RS antenna.  The RS shall perform local STC encoding by using the STC Matrix as defined by STC_DL_Zone_IE (or MIMO DL Basic IE or MIMO DL Enhanced IE) as indicated by the assigned antenna number(s) in the Zone Configuration IE, and the RS shall not forward an incorrectly decoded burst to its subordinate stations.  Each RS antenna shall transmit pilots based on the permutation, the number of antennas as indicated in STC_DL_Zone IE, and its antenna assignment.  The antenna assignment will be effective until the next zone configuration message.

Figure 323a is an example of local STC encoding at the RS.  The STC Encoder is identical to the encoder in Figure 244[Figure 301 in Rev2] of 8.4.8.1.

In cooperative relaying, the frames sent by MR-BS and RS at a given frame time must arrive at the MS within the prefix interval, similar to MDHO.

Figure 323a–A logical block example of local STC Encoding at RS.