| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On the issue of frame alignment and gaps |  |
| Date Submitted | 2007-11-14 |  |
| Source(s) | Dorin Viorel Fujitsu Microelectronics Canada Inc. Michiharu Nakamura Fujitsu Laboratories Ltd Chenxi Zhu Fujitsu Laboratories of America Inc Youngbin Chang Samsung Electronics Erik Colban Nextwave Broadband Inc | Voice: +1 403 207 6311 dviorel@fmci.fujitsu.com Voice: +81 468 39 5371 michi@labs.fujitsu.com Voice: +1 301 486 0671 chenxi.zhu@us.fujitsu.com Voice : +82 31 279 5519 yb.chang@samsung.com Voice: +1 858 480 3240 ecolban@nextwave.com |
| Re: | In response to the LB#28 reply comments |  |
| Abstract | This document provides proposed text for IEEEP802.16j on frame structure requirements and the definition of RSTTG and RSRTG |  |
| Purpose | Discussion and adoption in the P802.16j draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

On the issue of frame alignment and gaps for multihop relay transmissions

Dorin Viorel, Chenxi Zhu, Michiharu Nakamura, Youngbin Chang and Erik Colban

Fujitsu Microelectrionics Canada Inc., Fujitsu Laboratories of America Inc., Fujitsu Laboratories Ltd..,

Samsung Electronics, Nextwave Broadband Inc.

# Introduction

In order to provide support for the one radio RS operation as defined by 802.16j, new gaps (R-TTG and its related RSTTG, R-RTG and its related RSRTG) have been introduced, in order to avoid any data drops due to the Tx/Rx and Rx/Tx switching times.

This contribution provides some information on the specifications of these gaps based on the arrangement of the DL and UL subframe start time arrangement between the MR-BS and RSs.

This contribution is trying to supersede the following IEEE 802.16-07/045r3 comments

- #879, #880 (text in 8.4.4.2)

# Gaps in the relay frames

Figure 1 depicts MR-BS and RS frame structure with highlights of various gaps for information.

.

Figure 1 Frame structure (for informaion)

# Specific text changes

Change the text in sub-clause #8.4.4.2 as follows:

In multihop relay MR systems where relay links and access links on consecutive hops are time separated, relay station RS allowances shall be made for an RSRTG and for an RSTTG. The relay station shall not transmit downlink information to a subordinate station later than RSTTG-RTD/2 before the beginning of a received ]mode DL relay zone . The relay station shall not receive uplink information from a subordinate station later than RSRTG+RTD/2 before the beginning of a transmit mode UL relay zone. The parameters of RSRTG and RSTTG for a RS are capabilities provided by the RS to MR-BS upon request during RS network entry (see 11.8.3.1).

All DL transmissions shall be symbol aligned with the corresponding symbols at the MR-BS. All UL transmissions shall be time advanced such that they are symbol aligned at the receiving station with the corresponding symbols at the MR-BS.

An R-RTG shall be inserted between the zones where RS is required to switch from receive to transmit mode. Transmission to this RS shall not be scheduled during the R-RTG symbols.

An R-TTG shall be inserted between the zones where RS is required to switch from transmit to receive mode. Transmission to this RS shall not be scheduled during the R-TTG symbols.

The R-RTG and R-TTG are defined in 3.99 and 3.100, respectively.

Delete the text in 8.4.4.7.1.2 (Relay frame structure for transparent mode), as follows:

If the RS switches from transmission to reception mode, an R-TTG shall be inserted. If the RS switches from

reception to transmission mode, an R-RTG shall be inserted.

Change the text in 8.4.4.7.2.2 (Relay frame structure for non-transparent mode), as follows:

An example of an RS frame structure is shown in Figure 270b .

The RS transmits its frame start preamble time aligned with its superordinate station’s frame start preamble.

The UL sub-frame of the RS is aligned to the UL sub-frame of the MR-BS.

The DL sub-frame shall include at least one DL access zone and may include one or more relay zones. An

R-TTG may be placed between a DL access zone and a DL relay zone and an R-TTG or R-RTG may be

placed between two adjacent DL relay zones.

The UL sub-frame may include one or more UL access zones and one or more relay zones. An R-RTG may

be placed between a UL access zone and a UL relay zone and an R-TTG or R-RTG may be inserted between

two adjacent UL relay zones.

A relay zone may be utilized for either transmission, reception, or idle but the RS shall not be required to

support both modes of operation within the same zone.

If the relay station switches from transmission to reception mode, an R-TTG may be required. If the relay

station switches from reception to transmission mode, an R-RTG may be required. There may be more than

one R-TTG and more than one R-RTG inserted in the RS frame. In each frame, the TTG shall be inserted

between the DL sub-frame and the UL sub-frame. The RTG shall be inserted at the end of each frame.

The contents of the FCH, DL-MAP and UL-MAP in the Relay Frame may be different from those in the

MR-BS frame.

Each RS frame begins with a preamble followed by an FCH and the DL-MAP and possibly a UL-MAP. In

the DL access zone, the subchannel allocation, the FCH transmission, and the FCH shall be as defined in

Section 8.4.4.2.

The R-FCH and the R-DL-MAP shall be transmitted in the first DL Relay zone that is in Tx mode.

The MR-BS or RS shall transmit the RS-CD message in the access zone for the subordinate RSs to configure

the multihop relay frame structure through initial network entry or shall transmit the RS-CD message in

the DL relay zone, when the frame configuration is changed through normal relay operation.

For synchronization purpose, the relay amble, when present, shall be located either at the end of the last DL

relay zone in which MR-BS/RS is in transmit mode or at the end of the DL subframe. For monitoring purpose,

the relay link amble, when present, shall be located at the end of the DL subframe. An R-TTG or RRTG

may be inserted before relay amble.