| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification and fix on DL HARQ retransmission and ACK/NAK relaying in centralized scheduling |  |
| Date Submitted | 2007-11-01 |  |
| Source(s) | Shashikant Maheshwari, Adrian Boariu, Yousuf Saifullah, Haihong Zheng, Peter Wang, Aik Chindapol NSN | E-mail: shashi.maheshwari@nsn.com |
| Re: | This is in response for reply comment #942 on P80216j/D1 |  |
| Abstract | Provide fix for the DL HARQ retransmission and HARQ ACK/NAK relaying in centralized scheduling |  |
| Purpose | Review and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# DL HARQ retransmission and ACK/NAK relaying in Centralized scheduling

NSN

# Introduction

DL HARQ procedure for Non-transparent RS in centralized scheduling is defined in section 6.3.17.4.1 of IEEE 802.16j/D1. However, it is not clear how the HARQ ACK/NAK of retransmitted HARQ bursts are relayed by the intermediate RSs. This contribution provides the clarification and fix for the issue.

# Specification changes

[Insert following paragraph after 3rd paragraph of sub clause 6.3.17.4.1]

When a transmission of an HARQ burst fails, MR-BS shall schedule retransmission of HARQ burst only on the failed links while allocating the UL ACK channel along the path from MS to MR-BS such that encoded ACK/NAK report is received by the MR-BS.

MR-BS shall provide the offset to each RS in the “HARQ ACKCH region allocation for data IE” for relaying the ACK/NAK reports for the retransmitted HARQ burst. The RS has assigned for each “hop_depth” a specific ACKCH sub-region given by the “Retx_ACHCH_offset” field in the “HARQ ACKCH region allocation for data IE” where RS reports the ACK/NAK signals of the corresponding retransmitted HARQ bursts.

When an RS is performing retransmission of HARQ burst, all the sub-ordinate RSs consider this retransmission as new transmission and follow the procedure as described in section 6.3.17.4.1. However, the RS that performed the retransmission of HARQ burst shall report the ACK/NAK signals to the super-ordinate RS in the ACKCH sub-region indicated by the “Retx_ACKCH_offset” field. Before relaying the ACK/NAK signals of the retransmitted HARQ bursts to the superordinate station, the RS shall append in the corresponding “hop_depth” ACKCH sub-region, its own ACK/NAK signals of the retransmitted bursts that failed at its own link; AI_SN field in the (RS-)HARQ-MAP IE identifies the retransmission. The order in which the RS appends the ACK/NAK reports is the same as the order in which the CIDs of the retransmitted bursts have been appeared in the (RS-)HARQ_DL-MAP IE.

RS shall perform the Cx operation when relaying the ACK/NAK signals to super-ordinate RS/MR-BS.

If retransmitted HARQ burst are related to hop_depth ‘x’, RS waits for the ACK/NAK signal from the sub-ordinate RSs according to equation defined in section 8.4.5.4.25 and then RS forwards the ACK/NAK signals to super-ordinate RS in the ACKCH sub-region related to hop_depth ‘x’+1.

[Change Table 484a as follows]

Table 484a- HARQ_ACKCH region allocation for Data IE format

| hop_depth | 3 bits | B000 and b001 are invalid. When MR-BS/RS transmits HARQ burst for the nth hop away MSs, it shall set hop_depth =n. |
| --- | --- | --- |
| Reserved retx_region_present | 1 bit |  |
| ACKCH_offset | 8 bits | ACKCH_offset indicates the starting point in the ACKCH region for sending HARQ ACK/NAK for corresponding hop_depth. |
| If (retx_region_present) { |  |  |
| N_RS | 3 bits | Number_of_RS |
| For(j=0; j< N_RS; j++){ |  |  |
| RCID_IE() | Variable | RS basic CID |
| Retx_ACKCH_offset | 8 bits | Retx_ACKCH_offset indicates the starting point in the ACKCH region for sending ACK/NAK signals for the retransmitted burst. RS uses this region to forward the ACK/NAK signals of retransmitted burst and relaying the ACK/NAK signals of retransmitted bursts received from its sub-ordinate RSs. If retransmitted HARQ burst are related to hop_depth x then it uses ACKCH region in hop_depth x+1 for relaying the ACK/NAK signals. |
| } |  |  |
| } |  |  |