| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification for Burst-based forwarding on transparent mode |  |
| Date Submitted | 2007-11-15 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan [add other co-authors here] | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw |
| Re: | IEEE 802.16j-07/043: “IEEE 802.16 Working Group Working Group Letter Ballot #28” |  |
| Abstract | This contribution clarify the burst-based forwarding scheme on transparent mode |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification for Burst-based forwarding on transparent mode

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry (III)

Problem Statement

In WG Letter Ballot # 28 on initial draft IEEE P802.16j/D1, there are comments commenting on the burst-based forwarding scheme on transparent mode. This contribution is proposed to clarify the burst-based forwarding scheme defined in IEEE P802.16j/D1 and proposed the associated modifications to accommodate the following comments.

| Comment# | Subclauses | Authors |  | Type |
| --- | --- | --- | --- | --- |
|  |  | Last | First |  |
| 499 | 6.3.3.8.2 | Visotsky | Eugene | Technical |
| 501 | 6.3.3.8.2 | Loa | Kanchei | Editorial |
| 503 | 6.3.3.8.2 | Jin | Sunggeun | Editorial |
| 504 | 6.3.3.8.2 | Nakamura | Michiharu | Editorial |
| 923 | 8.4.5.3.2.2 | Nakamura | Michiharu | Technical |
| 928 | 8.4.5.3.28 | Kim | Olszewski | Technical |
| 929 | 8.4.5.3.28 | Hart | Mke | Technical |

Introduction

In order to elaborate the proposed burst-based data forwarding scheme, a DL-MAP IE with “DL_Burst_Transmit_IE” is first given in Table 1 for an RS to transmit data to its subordinated stations. Moreover, an UL-MAP IE with “UL_Burst_Receive_IE” is given in Tables 2 for an RS to receive data from its subordinated stations. Examples of using the proposed MAP IE in DL-MAP and UL-MAP for data relaying with transparent RSs are given in Tables 3, 4, and 5. Examples of 4-hop data relaying is also given in Figures 1 & 2 to illustrate the burst-based data forwarding scheme. Finally, in order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the baseline working document IEEE P802.16j/D1 are listed below.

Table 1 DL-MAP IE with “DL_Burst_Transmit_IE”

| Syntax | Size | Notes |
| --- | --- | --- |
| DL-MAP_IE(RS, Nr) | variable | RS is assign to relay Nr IE |
| { |  |  |
| DIUC | 4 bits | 15 (Extended DIUC dependent IE) |
| DL_Burst_Transmit_IE() { |  |  |
| Extended UIUC | 4 bits | DL_Burst_Transmit_IE |
| Length | 4 bits | Length = 2+2Nr or 3 + 2Nr |
| If(Length is even){ |  |  |
| RCID_IE | 8 bits | Reduced RS basic CID; |
| }else{ |  |  |
| CID | 16 bits | RS basic CID |
| } |  |  |
| Nr | 8 bits | Number of bursts forwarding by RS |
| for (n = 0; n < Nr; n++) { | - | - |
| Relay burst length | 16 bits | Relay burst length (in unit of byte) |
| } |  |  |
| } |  |  |
| } |  |  |

Table 2 UL-MAP IE with “UL_Burst_Receive_IE”

| Syntax | Size | Notes |
| --- | --- | --- |
| UL-MAP_IE(RS, Nr) | 36 bits |  |
| { |  |  |
| CID | 16 bits | RS basic CID |
| UIUC | 4 bits | 15 (Extended UIUC dependent IE) |
| UL_Burst_Receive_IE () { |  |  |
| Extended UIUC | 4 bits | UL_Burst_Receive_IE |
| Length | 4 bits | Length = 1 |
| Nr | 8 bits | Number of UL-MAP IE following current IE for RS to receive data bursts from subordinated station(s) |
| } |  |  |
| } |  |  |

Table 3: Format of DL-MAP

| Syntax | Size | Notes |
| --- | --- | --- |
| DL-MAP_Message_Format() { |  |  |
| Management Message Type = 3 | 8 bits |  |
| PHY Synchronization Field | variable |  |
| DCD Count | 8 bits |  |
| Base Station ID | 48bits |  |
| Begin PHY Specific Section { |  |  |
| if (WirelessMAN-OFDMA) { |  |  |
| No. OFDMA symbols | 8 bits | Number of OFDMA symbols in the DL subframe including all AAS/permutation zone. |
| } |  |  |
| for (n = 0; n < N; n++) { | - | For each DL-MAP element 1 to N. |
| DL-MAP_IE() | Variable | (Table 5a as an example) |
| } |  |  |
| } |  |  |

Table 4 Format of UL-MAP

| Syntax | Size | Notes |
| --- | --- | --- |
| UL-MAP_Message_Format() { |  |  |
| Management Message Type = 3 | 8 bits |  |
| Reserved | 8 bits | Shall be set to zero. |
| UCD Count | 8 bits |  |
| Allocation Start Time | 32 bits |  |
| Begin PHY Specific Section { |  |  |
| if (WirelessMAN-OFDMA) { |  |  |
| No. OFDMA symbols | 8 bits | Number of OFDMA symbols in the UL subframe |
| } |  |  |
| for (n = 0; n < N; n++) { | - | For each UL-MAP element 1 to N. |
| UL-MAP_IE() | Variable | (Table 5b as an example) |
| } |  |  |
| } |  |  |

Table 5a: Example of proposed DL-MAP IEs

| MR Network Topology | Zone | MAP-IEs used to describe the zone(s) | Notes |
| --- | --- | --- | --- |
|  | DL Access Zone (BS:Tx, MS :Rx, RS1:Rx, RS2:Rx) | DL-MAP_IE1() | MAP IEs for MS receiving from MR-BS |
|  |  |  |  |
|  |  | DL-MAP_IEi() |  |
|  |  | DL-MAP_IEi+1 () | Data burst for RS1 itself (with RS1 basic CID) |
|  |  | DL-MAP_IEi+2() | Data burst for RS1 relaying (with RS1 primary CID) |
|  |  | STC_Zone_IE | Indicate zone switch |
|  | 1st DL Trans-parent Zone (RS1:Tx, MS:Rx, RS2:Rx) | DL-MAP_IE with DL_Burst_Transmit_IE (RS1, Nr = m+2) | RS1 is assigned to transmit the following m legacy DL-MAP IEs for MSs and 2 DL-MAP IEs for RS2 |
|  |  | DL-MAP_IEj-1() | MAP IEs for MS receiving from RS1 |
|  |  |  |  |
|  |  | DL-MAP_IEj+m() |  |
|  |  | DL-MAP_IEj+m+1() | Data burst for RS2 itself (with RS2 basic CID) |
|  |  | DL-MAP_IEj+m+2() | Data burst for RS2 relaying (with RS2 primary CID) |
|  |  | STC_Zone_IE | Indicate zone switch |
|  | 2nd DL Transparent Zone (RS2:Tx, MS: Rx) | DL-MAP_IE with DL_Burst_Transmit_IE (RS2, Nr = n) | RS2 is assigned to transmit the following n legacy DL-MAP IEs |
|  |  | DL-MAP_IEk-1() | MAP IEs for MS receiving from RS2 |
|  |  |  |  |
|  |  | DL-MAP_IEk+n () |  |

Table 5b: Example of proposed UL-MAP IEs

| MR Network Topology | Zone | MAP-IEs used to describe the zone(s) | Notes |
| --- | --- | --- | --- |
|  | UL Access Zone (MS:Tx, RS1:Rx, RS2:Rx) | UL-MAP_IE1() | MAP IEs for MS transmitting to MR-BS |
|  |  |  |  |
|  |  | UL-MAP_IEi() |  |
|  |  | UL-MAP_IE with UL Burst Receive IE (RS1, m) | RS1 is assigned to receive following m IEs |
|  |  | UL-MAP_IEj-1() | MAP IEs for MS transmitting to RS1 |
|  |  |  |  |
|  |  | UL-MAP_IEj+m() |  |
|  |  | UL-MAP_IEj with UL Burst Receive IE (RS2, n) | RS2 is assigned to receive following n IEs |
|  |  | UL-MAP_IEk-1() | MAP IEs for MS transmitting to RS2 |
|  |  |  |  |
|  |  | UL-MAP_IEk+n () |  |
|  |  | UL_Zone_IE() | Indicate zone switch |
|  | 1st UL Relay Zone (RS1:Rx, RS2:Tx) | UL-MAP_IE with UL Burst Receive IE (RS1, 1) | RS1 is assigned to receive following IE |
|  |  | UL-MAP_IE() | MAP IE for RS2 transmitting to RS1 |
|  |  | UL_Zone_IE() | Indicate zone switch |
|  | 2nd UL Relay Zone (RS1:Tx BS :Rx) | UL-MAP_IE() | MAP IE for RS1 transmitting to MR-BS |

Figure 1a Example of proposed MAP-based scheme for unicast data relaying (one hop in one frame)

Figure 1b Example of proposed MAP-based scheme for unicast data relaying (all hops in one frame)

Figure 2a Example of proposed MAP-based scheme for multicast data relaying (one hop in one frame)

Figure 2b Example of proposed MAP-based scheme for multicast data relaying (all hops in one frame)

Text Proposal

6.3.3.8.2 Transmission using station CID

[Change the following text as indicated]

There are two schemes for RS to forward received data without using tunnels. The first oOne is the MPDU-called CID based forwarding and the other is burst-based forwarding. In MPDU-the CID based forwarding scheme, the forwarding of MPDUs by each RS is performed based on the CID contained in the MPDU header. An RS is informed about the next hop station during the setup of the service flow. When forwarding using this scheme, tThe inclusion of the CID in the DL-MAP is optional.

Optionally, under centralized scheduling, forwarding of MPDUs by each RS is performed based on burst described in MAP IEs, namely burst-based forwarding. The burst-based forwarding scheme works, which is used with centralized, performs forwarding based on the burst described in the MAP IEs and utilizesing forwarding rules encoded in MAPs. Data bursts that are scheduled to be relayed by the receiving RS, but are not destined for the same RS must rely on MAP IEs shall be sent with the RS primary management CID. For transparent RS, Tthe DL_Burst_Transmit_IE and UL_Burst_Receive_IE, as described in 8.4.5.3.2928 and 8.4.5.4.29, (respectively,) are the IEs that shall be used. The DL_Burst_Transmit_IE is used to describes the DL data relaying information and the UL_Burst_Receive_IE is used to describes UL data relaying information. For DL MAP IEs following the DL_Burst_Transmit_IE, the RS shall forward the data in allocations defined by these IEs, where the forwarded data is received in the DL burst with the RS primary management CID. For UL MAP IEs following the UL_Burst_Receive_IE, the RS shall receive the data in allocations defined by these IEs and forward to its superordinated station in the next available allocation, defined by legacy UL-MAP IE, in UL relay zone.

8.4.5.3.2 DL-MAP extended IE format

8.4.5.3.2.2 DL-MAP extended-2 IE format

[Change Table 277c(.16e)/Table 385(Rev2) as indicated:]

Table 277c—Extended-2 DIUC code assignment for DIUC=14

| Extended-2 DIUC | (hexadecimal) Usage |
| --- | --- |
| 0B | DL_Burst_Transmit_IE MIMO_DL_Basic_IE |
| 0F | Reserved DL_Burst_Transmit_IE |