| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On enhancing MCS for the relay link |  |
| Date Submitted | 2007-11-05 |  |
| Source(s) | Su Chang Chae, Young-il Kim, Hyunjae Kim ETRI Kyu Ha Lee, Changyun Kim, Yong Wook Lee Samsung Thales Aik Chindapol NSN Adrian Boariu NSN Jai H. Eu, Jcast Networks Korea, Inc Hyung-joon Jeon POSDATA Jun Bae Ahn SOLiD technologies | E-mail:schae@etri.re.kr E-mail:kyuha.lee@samsung.com E-mail:Aik.chindapol@siemens.com E-mail:Adrian.Boariu@nokia.com E-mail:jeu@jcastnet.com E-mail:hjjun@posdata.co.kr jbahn@st.co.kr |
| Re: | This is a response to the call for reply comments on IEEE 802.16j-07/045r3 |  |
| Abstract | Propose On enhancing MCS for the relay link |  |
| Purpose | To incorporate the proposed text into C802 16j-D1 (August 2007),”Draft Standard for Local and Metropolitan Area Networks” |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

On enhancing MCS for the relay link

# 1. Introduction

There is a need for expanding the number of MCSs that currently exists in the standard in order to offer finer granularity for transmissions over the relay link. The contribution targets CTC operation because it offers greater flexibility and easy implementation.

# 2. Proposed Text Change

[Add the following text and table at the end of section 8.4.9.2.3.1(16Rev2)]

On the relay link when CTC is used and demodulation and modulation scheme is employed,  the data bursts shall  be encoded using the Table xxx.

Table xxx Encoding slot concatenation for different rates in CTC

| Modulation and rate | j |
| --- | --- |
| QPSK 1/2 | 10 |
| QPSK 2/3 | 6 |
| QPSK 3/4 | 6 |
| QPSK 5/6 | 6 |
| 16QAM 1/2 | 5 |
| 16QAM 2/3 | 3 |
| 16QAM 3/4 | 3 |
| 16QAM 5/6 | 3 |
| 64QAM 1/2 | 3 |
| 64QAM 2/3 | 2 |
| 64QAM 3/4 | 2 |
| 64QAM 5/6 | 2 |

## 11.3.1.1 Uplink burst profile encodings

[Insert paragraph and new Table 612-xx in 11.3.1.1(16Rev2) “Uplink burst profile encodings” as shown]

Table 612-xx shall be used in the relay link when demodulation and modulation scheme is employed.

Table 612-xx-UCD burst profile encodings-Wireless MAN-OFDMA

| Name | Type | Length | Value |  |
| --- | --- | --- | --- | --- |
| FEC Code Type and modulation type | 150 | 1 | 0=QPSK(CC) 1/2 1=QPSK(CC)3/4 2=16-QAM(CC)1/2 3=16-QAM(CC)3/4 4=64-QAM(CC)1/2 5=64-QAM(CC)2/3 6=64-QAM(CC)3/4 7=QPSK(BTC)1/2 8=QPSK(BTC)3/4 9=16-QAM(BTC)3/5 10=16-QAM(BTC)4/5 11=64-QAM(BTC)5/8 12=64-QAM(BTC)4/5 13=QPSK(CTC)1/2 14=reserved 15=QPSK(CTC)3/4 16=16-QAM(CTC)1/2 17=16-QAM(CTC)3/4 18=64-QAM(CTC)1/2 19=64-QAM(CTC)2/3 20=64-QAM(CTC)3/4 21=64-QAM(CTC)5/6 22=QPSK(ZT CC) 1/2 23=QPSK(ZT CC)3/4 24=16-QAM(ZT CC)1/2 25=16-QAM(CC)3/4 26=64-QAM(ZT CC)1/2 | 27=64-QAM(ZT CC)2/3 28=64-QAM(ZT CC)3/4 29=QPSK(LDPC)1/2 30= QPSK(LDPC)2/3 A code 31=QPSK(LDPC)3/4 A code 32=16-QAM(LDPC)1/2 33=16-QAM(LDPC)2/3 A code 34=16-QAM((LDPC)3/4 A code 35=64-QAM(LDPC)1/2 36=64-QAM(LDPC)2/3 A code 37=64QAM(LDPC)3/4 A code 38=QPSK(LDPC)2/3 B code 39= QPSK(LDPC)3/4 B code 40=16-QAM((LDPC)2/3 B code 41=16-QAM((LDPC)3/4 B code 42=64-QAM(LDPC)2/3 B code 43=64QAM(LDPC)3/4 B code 44=QPSK(CC with optional inter leaver)1/2 45=QPSK(CC with optional inter leaver)3/4 46=16-QAM(CC with optional inter leaver)1/2 47=16-QAM(CC with optional inter leaver)3/4 48=64-QAM(CC with optional inter leaver)1/2 49=64-QAM(CC with optional inter leaver)3/4 50=QPSK(LDPC)5/6 51=16-QAM(LDPC)5/6 52=64-QAM(LDPC)5/6 53=QPSK(CTC)2/3 54=QPSK(CTC)5/6 55=16-QAM(CTC)2/3 56=16-QAM(CTC)5/6 5357..255=Reserved |

## 11.4.2 Downlink burst profile encodings

[Insert  paragraph and new Table 619-xx in 12.4.2(16Rev2) “Uplink burst profile encodings” as shown]

Table 619-xx shall be used on the relay link when demodulation and modulation scheme is employed.

..

Table 619-xx-DCD burst profile encodings-Wireless MAN-OFDMA

| Name | Type | Length | Value |  |
| --- | --- | --- | --- | --- |
| FEC Code Type and modulation type | 150 | 1 | 0=QPSK(CC) 1/2 1=QPSK(CC)3/4 2=16-QAM(CC)1/2 3=16-QAM(CC)3/4 4=64-QAM(CC)1/2 5=64-QAM(CC)2/3 6=64-QAM(CC)3/4 7=QPSK(BTC)1/2 8=QPSK(BTC)3/4 9=16-QAM(BTC)3/5 10=16-QAM(BTC)4/5 11=64-QAM(BTC)5/8 12=64-QAM(BTC)4/5 13=QPSK(CTC)1/2 14=reserved 15=QPSK(CTC)3/4 16=16-QAM(CTC)1/2 17=16-QAM(CTC)3/4 18=64-QAM(CTC)1/2 19=64-QAM(CTC)2/3 20=64-QAM(CTC)3/4 21=64-QAM(CTC)5/6 22=QPSK(ZT CC) 1/2 23=QPSK(ZT CC)3/4 24=16-QAM(ZT CC)1/2 25=16-QAM(CC)3/4 26=64-QAM(ZT CC)1/2 | 27=64-QAM(ZT CC)2/3 28=64-QAM(ZT CC)3/4 29=QPSK(LDPC)1/2 30= QPSK(LDPC)2/3 A code 31=QPSK(LDPC)3/4 A code 32=16-QAM(LDPC)1/2 33=16-QAM(LDPC)2/3 A code 34=16-QAM((LDPC)3/4 A code 35=64-QAM(LDPC)1/2 36=64-QAM(LDPC)2/3 A code 37=64QAM(LDPC)3/4 A code 38=QPSK(LDPC)2/3 B code 39= QPSK(LDPC)3/4 B code 40=16-QAM((LDPC)2/3 B code 41=16-QAM((LDPC)3/4 B code 42=64-QAM(LDPC)2/3 B code 43=64QAM(LDPC)3/4 B code 44=QPSK(CC with optional inter leaver)1/2 45=QPSK(CC with optional inter leaver)3/4 46=16-QAM(CC with optional inter leaver)1/2 47=16-QAM(CC with optional inter leaver)3/4 48=64-QAM(CC with optional inter leaver)1/2 49=64-QAM(CC with optional inter leaver)3/4 50=QPSK(LDPC)5/6 51=16-QAM(LDPC)5/6 52=64-QAM(LDPC)5/6 53=QPSK(CTC)2/3 54=QPSK(CTC)5/6 55=16-QAM(CTC)2/3 56=16-QAM(CTC)5/6 5357..255=Reserved |

## 8.4.9.2.3.2 CTC interleaver

[Insert the  paragraph and table 566-xx below the Table 566(16Rev2) as indicated:]

Table 566-xx may be be used for the relay link when demodulation and modulation scheme is employed.

.

Table 566-xx CTC Channel coding per modulation for relay link

| Modulation | Data block size (bytes) | Encoded data block size (bytes) | Code rate | N | P0 | P1 | P2 | P3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 6 | 12 | 1/2 | 24 | 5 | 0 | 0 | 0 |
| QPSK | 12 | 24 | 1/2 | 48 | 13 | 24 | 0 | 24 |
| QPSK | 18 | 36 | 1/2 | 72 | 11 | 6 | 0 | 6 |
| QPSK | 24 | 48 | 1/2 | 96 | 7 | 48 | 24 | 72 |
| QPSK | 30 | 60 | 1/2 | 120 | 13 | 60 | 0 | 60 |
| QPSK | 36 | 72 | 1/2 | 144 | 17 | 74 | 72 | 2 |
| QPSK | 48 | 96 | 1/2 | 192 | 11 | 96 | 48 | 144 |
| QPSK | 54 | 108 | 1/2 | 216 | 13 | 108 | 0 | 108 |
| QPSK | 60 | 120 | 1/2 | 240 | 13 | 120 | 60 | 180 |
| QPSK | 24 | 36 | 2/3 | 96 | 7 | 48 | 24 | 72 |
| QPSK | 48 | 72 | 2/3 | 192 | 11 | 96 | 48 | 144 |
| QPSK | 9 | 12 | 3/4 | 36 | 11 | 18 | 0 | 18 |
| QPSK | 18 | 24 | 3/4 | 72 | 11 | 6 | 0 | 6 |
| QPSK | 27 | 36 | 3/4 | 108 | 11 | 54 | 56 | 2 |
| QPSK | 36 | 48 | 3/4 | 144 | 17 | 74 | 72 | 2 |
| QPSK | 45 | 60 | 3/4 | 180 | 11 | 90 | 0 | 90 |
| QPSK | 54 | 72 | 3/4 | 216 | 13 | 108 | 0 | 108 |
| QPSK | 30 | 36 | 5/6 | 120 | 13 | 60 | 0 | 60 |
| QPSK | 60 | 72 | 5/6 | 240 | 13 | 120 | 60 | 180 |
| 16-QAM | 12 | 24 | 1/2 | 48 | 13 | 24 | 0 | 24 |
| 16-QAM | 24 | 48 | 1/2 | 96 | 7 | 48 | 24 | 72 |
| 16-QAM | 36 | 72 | 1/2 | 144 | 17 | 74 | 72 | 2 |
| 16-QAM | 48 | 96 | 1/2 | 192 | 11 | 96 | 48 | 144 |
| 16-QAM | 60 | 120 | 1/2 | 240 | 13 | 120 | 60 | 180 |
| 16-QAM | 48 | 72 | 2/3 | 192 | 11 | 96 | 48 | 144 |
| 16-QAM | 18 | 24 | 3/4 | 72 | 11 | 6 | 0 | 6 |
| 16-QAM | 36 | 48 | 3/4 | 144 | 17 | 74 | 72 | 2 |
| 16-QAM | 54 | 72 | 3/4 | 216 | 13 | 108 | 0 | 108 |
| 16-QAM | 60 | 72 | 5/6 | 240 | 13 | 120 | 60 | 180 |
| 64-QAM | 18 | 36 | 1/2 | 72 | 11 | 6 | 0 | 6 |
| 64-QAM | 36 | 72 | 1/2 | 144 | 17 | 74 | 72 | 2 |
| 64-QAM | 54 | 108 | 1/2 | 216 | 13 | 108 | 0 | 108 |
| 64-QAM | 24 | 36 | 2/3 | 96 | 7 | 48 | 24 | 72 |
| 64-QAM | 48 | 72 | 2/3 | 192 | 11 | 96 | 48 | 144 |
| 64-QAM | 27 | 36 | 3/4 | 108 | 11 | 54 | 56 | 2 |
| 64-QAM | 54 | 72 | 3/4 | 216 | 13 | 108 | 0 | 108 |
| 64-QAM | 30 | 36 | 5/6 | 120 | 13 | 60 | 0 | 60 |
| 64-QAM | 60 | 72 | 5/6 | 240 | 13 | 120 | 60 | 180 |