| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on association procedure in an MR network |  |
| Date Submitted | 2008-01-14 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee, Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw |
| Re: | IEEE 802.16j-07/059: “IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement” |  |
| Abstract | This contribution proposes modification on MS network topology acquisition in an MR network |  |
| Purpose | Text proposal for 802.16j Baseline Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on association procedure in an MR network

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry (III)

Introduction

In P802.16j/D2, the scheme of “association procedure in an MR network” for the RS operated in centralized scheduling mode with association level 2 is not defined. Under centralized scheduling, the neighbor station, following regular MS ranging procedures, sends RNG-RSP message for association level 0/1. But if the association level is 2, then the neighbor station have to report the ranging result to the MR-BS instead (see Table 1).

Addtionaly, in subclauses 6.3.22.1.4.2 and 6.3.22.1.4.3, there are many redundant text described contention-based initial ranging that have been defined in 6.3.10.3.1.1.

Therefore, we proposed this contribution to support association procedures for the RS operated in centralized scheduling mode and eliminate redundant text that described contention-based initial ranging by merging text in subclauses. There is no change in the technical context for association procedures for the RS in distributed scheduling mode.

Table 1 Coordinated association

| Purpose Item | Coordinated association |  |  |
| --- | --- | --- | --- |
|  | Level 0 | level 1 | level 2 |
| Sending RNG-RSP for MS | Yes | Yes | No |
| Sending MOB_ASC_REP | No | No | Yes |

*1 BS may use regular ranging region instead of dedicated ranging region

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft document P802.16j/D2 are listed below.

Text Proposal

[Modified the following subclause as indicated:]

6.3.22.1.4 6.3.22.1.3.4 MR-BSs and RSs behavior during Association procedure in an MR network

6.3.22.1.4.1 Association parameter acquisition

In a MR system under centralized scheduling, when the serving MR-BS decides to recommend that the MS to scan neighbor stations with association level 2, the MR-BS shall notify the neighbor stations the association parameters (i.e., Rendezvous time, unique CDMA code, Transmission opportunity offset) after sending the MOB_SCN-RSP message.

- If the neighbor station is the serving MR-BS itself, it has already owned the association parameters.

- If the neighbor station is a RS inside the MR-cell the MS attached to, the serving MR-BS shall notify the neighbor station via a MR_ASC-RSP message.

- If the neighbor station is outside the MR-cell the MS attached to, the serving MR-BS shall notify the neighbor MR-BS via the network backbone. If the neighbor station is a RS, the neighbor MR-BS shall then notify the neighbor station via a MR_ASC-RSP message.

In a MR system with under distributed scheduling, when the serving MR-BS decides to recommend that the MS scan neighbor stations with association level 1 or 2, it should obtain association parameters available from the MS’s neighbor stations before sending the MOB_SCN-RSP message.

In a MR system with RSs operating in distributed scheduling mode, when the serving MR-BS decides to recommend that the MS scan neighbor stations with association level 1 or 2, it should obtain association parameters available from the MS’s neighbor stations before sending the MOB_SCN-RSP message.

- If a neighbor station is in different MR-cells from the MS, the MS’s serving MR-BS shall forward a message via the network backbone to the neighbor station's serving MR-BS requesting the neighbor station’s association parameters.

- If the neighbor station is an MR-BS, the association parameters are readily available. However, if the neighbor station is an RS, its serving MR-BS must forward an MR_ASC-REQ message to the RS requesting that it forward its association parameters via an MR_ASC-RSP message.

- If a neighbor station is an RS in the same cell as the MS, the serving MR-BS shall request the association parameters directly from the RSs via an MR_ASC-REQ message.

- When an RS receives an MR_ASC-REQ message from its serving MR-BS, it shall respond with an association response (MR_ASC-RSP) message to indicate the association level allocated to the MS. If the allocated association level is 1 or 2, the MR_ASC-RSP should include the association parameters (i.e. Rendezvous time, CDMA code, and Transmission opportunity offset).

- Upon receiving these association parameters, the serving MR-BS shall determine whether the association parameters satisfy the MS' association requirements or not. If they do, the serving MR-BS shall include those association parameters in the MOB_SCN-RSP message.

For MS neighbor scanning with association level 0 and 1, the access station shall perform the same tasks as contention-based initial ranging described in 6.3.10.3.1.1.

For MS neighbor scanning with association level 2, instead of sending a RNG-RSP message to the MS, the neighbor RS shall send an MR_Code-REP message containing corresponding ranging information to the MR-BS.

[Delete the following subclauses:]

6.3.22.1.4.2 Association level 0 and 1

6.3.22.1.4.3 Association level 2