| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on MS Network Entry in the MR System (including the Distributed Security) |  |
| Date Submitted | 2007-11-15 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan [add other co-authors here] | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw |
| Re: | IEEE 802.16j-07/043: “IEEE 802.16 Working Group Working Group Letter Ballot #28” |  |
| Abstract | This contribution proposes MS ranging and automatic adjustments in transparent and non-transparent |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on MS Network Entry in the MR System
(including the Distributed Security)

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry (III)

Introduction

We propose this contribution to resolve the comments #667 (C80216j-07/498r2). Unlike centralized security scheme, a non-transparent RS (NT-RS) with distributed security and non-centralized control needs to accommodate messages exchanging between MR-BS and ASN-GW during MS network entry procedures. The MR-BS, which is no longer holding the AK on behalf of the RS, cannot proxy RS interactions with the ASN-GW without relaying ASN-GW messages to the NT-RS (AR-RS) that stores AK locally. The following figure illustrates messages exchanging among all entities and shows that we need new MAC messages between MR-BS and the RS in order to allow a MS complete its network entry procedures.

Figure 1 MS network entry with distributed security and non-centralized control

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D1 are listed below.

Specification Changes

6.3.9.7 Negotiate basic capabilities

[Insert new subclause 6.3.9.7.1]

6.3.9.7.1 MR-BS and RS behavior during basic capabilities negotiations

When an SS performs basic capabilities negotiation in systems with transparent RSs, MR-BSs and transparent RSs shall perform the following tasks:

- Upon receiving an SBC-REQ from an SS, the RS shall relay it to the MR-BS.

- The MR-BS shall respond an SBC-RSP to the SS directly.

When an SS performs basic capabilities negotiation in systems using non-transparent RSs with unique BSIDs and centralized scheduling, MR-BSs and non-transparent RSs shall perform the following tasks:

- Upon receiving an SBC-REQ from an MS, the RS shall relay it to the MR-BS.

- The MR-BS shall respond an SBC-RSP to the SS via the RS.

When an SS performs basic capabilities negotiation in systems using non-transparent RSs with unique BSIDs and distributed scheduling, MR-BSs and non-transparent RSs shall perform the following tasks:

- The RS shall send SBC-RSP to the SS directly in response to the received SBC-REQ without Authorization Policy Support

- The RS shall forward the SBC-REQ with Authorization Policy Support to the MR-BS after removing the TLVs it manages. The MR-BS shall determine the SS capabilities in the remaining TLVs and send them in a SBC-RSP to the RS. The RS shall then fill in the TLVs it manages and forward the SBC-RSP to the SS.

When an RS performs basic capabilities negotiation, MR-BSs and RSs shall follow the steps indicated by the type of system in previous sections of this subclause.

Figure 88a— Handling SBC-REQ at a transparent RS

Figure 88b— Handling SBC-REQ at a non-transparent RS

6.3.9.8 SS authorization and key exchange

[Insert the following texts at end of subclause 6.3.9.8]

6.3.9.9.1 MR-BS and RS behavior during authorization and key exchange

When an SS performs registration in systems using RSs with centralized security, MR-BSs and RSs shall perform perform authorization and key exchange as described in 7.1.6.

When an SS performs registration in systems using RSs with distributed security, MR-BSs and RSs shall perform perform authorization and key exchange as described in 7.1.7.

6.3.9.9 Registration

[Insert new subclause 6.3.9.7.1]

6.3.9.9.2 MR-BS and RS behavior during registration

When an SS performs registration in systems with transparent RSs, MR-BSs and transparent RSs shall perform the following tasks:

- Upon receiving an REG-REQ from an SS, the RS shall relay it to the MR-BS.

- The MR-BS shall either respond an REG-RSP to the SS directly or via the RS.

When an SS performs registration in systems using non-transparent RSs with unique BSIDs and centralized security, MR-BSs and non-transparent RSs shall perform the following tasks:

- Upon receiving an REG-REQ from an MS, the RS shall relay it to the MR-BS.

- The MR-BS shall respond an REG-RSP to the SS via the RS.

When an SS performs registration in systems using non-transparent RSs with unique BSIDs and distributed security, MR-BSs and non-transparent RSs shall perform the following tasks:

- The RS shall send REG-RSP to the SS directly in response to the received REG-REQ without Authorization Policy Support

- The RS shall forward the REG-REQ with Authorization Policy Support on RS basic CID to the MR-BS after removing the TLVs it manage and filling in the SS basic CID TLV s. The MR-BS shall response a REG-RSP containing the SS basic CID TLV with RS basic CID to the RS. The RS shall then remove SS basic CID TLV and fill in the TLVs it manages and forward the REG-RSP on SS primary management CID.

When an RS performs registration, MR-BSs and RSs shall follow the steps indicated by the type of system in previous sections of this subclause.

Figure 94a— Handling REG-REQ at a transparent RS under uplink-only mode

Figure 94b— Handling REG-REQ at a RS except uplink-only mode and local CID allocation mode

Figure 94c— Handle REG-REQ first reception at a AR-RS

[Modify 6.3.2.3.8 as indicated:]

In response to a REG-REQ from an RS, the REG-RSP shall contain the following TLV:

MS basic CID (see 11.7.x)

[Insert the following subclause as indicated:]

11.7.x MS basic CID

The MS basic CID TLV indicates that this message is relay by the RS on behalf of the corresponding MS whose basic CID is equal to the value of the TLV

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| TBA | 1 | MS basic CID | REG-REQ REG-RSP |