| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on SS Network Entry via AR-RS |  |
| Date Submitted | 2008-01-15 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan G.Q Wang Nortel [add other co-authors here] | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw guoqiang@nortel.com |
| Re: | IEEE 802.16j-07/059: “IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement” |  |
| Abstract | This contribution proposes clarification on SS network entry via AR-RS |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on SS Network Entry via AR-RS

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry (III)

Introduction

The SS network entry via AR-RS in the distributed security mode requires the HMAC/CMAC re-calculations at the AR-RS, which are defined in the subclause 7.1.7.1 of P802.16j/D2 “After completing MS authentication and establishing security association between MS and the access RS (see detail in 7.1), MAC management messages on the access link shall contain CMAC/HMAC calculated by a key shared between MS and its access RS, while MAC management messages on relay link shall contain CMAC/HMAC calculated by a key shared between RS and the MR-BS. The access RS shall recalculate CMAC/HMAC accordingly when it relays MAC management messages.”

However, during the registration phase, instead of forwarding the REG-REQ to the MR-BS, if the value of CMAC/HMAC is invalid for the REG-REQ message sent by the SS, the AR-RS must locally send the REG-RSP with Response = 1 (Message Auth. Failure) to the SS. Because of the HMAC/CMAC re-calculation, the MR-BS in the distributed security mode, which doesn't share the same HMAC/CMAC, cannot verify the legitimacy of the REG-REQ sent from the SS and response accordingly.

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D2 are listed below.

Specification Changes

[Insert the following text at line 12 of page 97 (subclause 6.3.9.9.2) as indicated:]

The AR-RS operated in local CID allocation mode shall perform the operations shown in Figure 94u and 94v, and the MR-BS shall perform the operations shown in Figure 94w.

Figure 94x— Handling REG-REQ first reception at an AR-RS operated in local CID allocation mode

Figure 94z— Handling REG-REQ at a MR-BS under local CID allocation mode