| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MR_Traffic-REP message |  |
| Date Submitted | 2007-11-15 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan [add other co-authors here] | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw |
| Re: | IEEE 802.16j-07/043: “IEEE 802.16 Working Group Working Group Letter Ballot #28” |  |
| Abstract | This contribution proposes MS ranging and automatic adjustments in transparent and non-transparent |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MR_Traffic-REP message

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry (III)

Introduction

In current draft document, when the offsets of frequency, power, and timing for the data transmission from a MS are beyond the tolerance defined in the specification, a UL-only transparent RS, which cannot generate DL traffic, must transmit a RNG-REQ message with the RS basic CID containing the MS basic CID to the serving MR-BS through the relay path. Such that the MR-BS can send the RNG-RSP to the MS on behalf of the RS.

Since the uplink environment change rapidly for a mobile MS, the access RS have to adjust uplink data transmission frequently that result in large overhead on the relay path. We propose a lower overhead MR_Traffic-REP message to replace RNG-REQ message. Comparisons of overheads between RNG-REQ message and proposed MR_Traffic-REP message are shown in Table 1. Table 2 gives an example of the exact saving for multiple MSs from 1 to 8.

Table 1 RNG-REQ and proposed MR_Traffic-REP

| Message Size |  | Original RNG-REQ | Proposed MR_Traffic-REP |
| --- | --- | --- | --- |
| Generic MAC header |  | 6 byte | 6 byte |
| Message body | Fix part | 2 | 2 |
|  | Variable part | 24N | 11.5N |
| CRC |  | 4 | 4 |
| Total |  | 12+24N | 12+11.5N |

N: Number of MS in the message Table 2 Message size (bytes)

Table 2 Message size (bytes)

| N | Original RNG-REQ | Proposed MR_Traffic-REP |
| --- | --- | --- |
| 1 | 36 | 25 |
| 2 | 60 | 35 |
| 3 | 84 | 47 |
| 4 | 108 | 58 |
| 5 | 132 | 70 |
| 6 | 156 | 81 |
| 7 | 180 | 93 |
| 8 | 204 | 104 |

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D1 are listed below.

Specification Changes

[Note: Texts has been accepted in the previous meeting (such as comment #0565: document IEEE C802.16j-07/459r6) are marked by Gray Color]

6.3.2.3.6 Ranging response (RNG-RSP) message

The following parameter may be included in the RNG-RSP message with abort status during RS initial ranging phase for RS neighbor access stations reselection:

Preamble Indexs

8 bit index of preamble indexes of candidate neighbor access stations

[Reporting Procedure]

[Modified 6.3.10.3.2.1 in IEEE C802.16j-07/459r6 as following indicated:]

In some cases, the superordinate station of an SS/RS may want to initiate ranging based on the channel measurements from data traffic or a CDMA-based bandwidth request ranging code received from the SS/RS.  To initiate the periodic ranging process, the superordinate station shall send an unsolicited RNG-RSP to the SS/RS. If superordinate station is a transparent RS receiving a CDMA-based bandwidth request, the MR-BS and the RS shall follow the procedure defined in 6.3.6.7.2.1.1. If the superordinate station is a transparent RS receiving data traffic and relaying on the uplink only, it shall transmit an RNG-REQ MR_Code-REP to the MR-BS on the RS basic CID to request that the MR-BS send an unsolicited RNG-RSP with the necessary adjustments to the SS. If the superordinate station is a non-transparent RS in centralized scheduling mode or a transparent RS relaying on both uplink and downlink, it shall request bandwidth from the MR-BS on which to send the unsolicited RNG-RSP (see 6.3.6.7.2.1). If the superordinate station is a non-transparent RS in distributed scheduling mode, it shall send the RNG-RSP directly to the SS without interaction with the MR-BS.

[Replace Figure 118b in comment #0565 (C802.16j-07/459r6) by following figure as indicated:]

Figure 118b—SS upstream transmission adjustment at a transparent RS performing uplink relay only

[Replace Figure 118d in comment #0565 (C802.16j-07/459r6) by following figure as indicated:]

Figure 118d—RNG-REQ handling MR_Code-REP with Report Type = 1 at an MR-BS

[MR_Code-REP message]

[Change the following Table 183c as indicated]

Table 183c—MR Code report (MR_Code-REP) message format

| Syntax | Size | Notes |
| --- | --- | --- |
| MR_Code-REP_Message_Format() { | - | - |
| Management Message Type = 69 | 8 bits | - |
| Report Type | 1 bit | 0 = report CDMA code, 1 = report UL traffic measruement |
| Size of Remaining CDMA codes | 87 bits | Amount of uplink bandwidth in bytes for CDMA codes CDMA codes remaining at the RS ready to be forwarded |
| Frame Number Index | 8 bits | LSBs of relevant frame number |
| while(data remain) { | - | - |
| If (Report Type == 0) | - | - |
| Ranging Code | 8 bits | Indicates the CDMA Code sent by the MS. |
| Ranging Symbol | 8 bits | Indicates the OFDMA symbol used by the MS. |
| Ranging subchannel | 7 bits | Identifies the Ranging subchannel used by the MS. |
| Channel Measurement | 6 bits | The mean CINR as measured on CDMA ranging code. |
| } else { | - | - |
| MS basic CID | 16 bits | MS basic CID |
| } | - | - |
| Include_TA | 1 bit | Include timing adjust (0 = absent, 1 = present) |
| Include_PLA | 1 bit | Include power level adjust (0 = absent, 1 = present) |
| Include_OFA | 1 bit | Include offset frequency adjust (0 = absent, 1 = present) |
| If(Include_TA == 1) { | - | - |
| Timing Adjust | 32 bits | Tx Timing offset adjustment (signed 32-bit). |
| } | - | - |
| If(Include_PLA == 1) { | - | - |
| Power Level Adjust | 8 bits | Tx Power level adjustment (signed 8-bit, 0.25 dB units). |
| } | - | - |
| If(Include_OFA == 1) { | - | - |
| Offset Frequency Adjust | 32 bits | Tx Frequency offset adjustment (signed 32-bit, Hz units). |
| } | - | - |
| } | - | - |
| } | - | - |

[Remove redundant TLVs in the RNG-REQ message]

11.5 RNG-REQ message encodings

[Remove TLVs in the following table as indicated:]

Table 613—RNG-REQ message encodings

| Name | Type (1 byte) | Length | Value (variable-length) | PHY Scope |
| --- | --- | --- | --- | --- |
| Ranging Purpose Indication | 6 | 1 | Bit #0: HO indication (when this bit is set to 1 in combination with other included information elements indicates the MS is currently attempting to HO or Network Re-entry from Idle Mode to the BS) Bit #1: Location Update Request (when this bit is set to 1, it indicates MS action of Idle Mode Location Update Process) Bit #2: MRS Location Update Request Bits 23-7: Reserved |  |
| New MS Indication ID | TBA | 1 | Unique identifier assigned by RS for each MS under ranging process |  |
| Received Ranging Codes | TBA | Variable | Received Ranging Codes is a compound TLV value that indicates received code information. | OFDMA |
| Timing Adjust | TBA.1 | 4 | Tx timing offset adjustment (signed 32-bit). The amount of time required to adjust SS transmission so the bursts will arrive at the expected time instance at the BS. Units are PHY specific (see 10.3). | OFDMA |
| Power Level Adjust | TBA.2 | 1 | Tx Power offset adjustment (signed 8-bit, 0.25 dB units) Specifies the relative change in transmission power level that the SS is to make in order that transmissions arrive at the BS at the desired power. When subchannelization is employed, the subscriber shall interpret the power offset adjustment as a required change to the transmitted power density. | OFDMA |
| Offset Frequency Adjust | TBA.3 | 4 | Tx frequency offset adjustment (signed 32-bit, Hz units) Specifies the relative change in transmission frequency that the SS is to make in order to better match the BS. (This is fine-frequency adjustment within a channel, not reassignment to a different channel.) | OFDMA |
| Ranging Status | TBA.4 | 1 | Used to indicate whether uplink messages are received within acceptable limits by BS. 1 = continue, 2 = abort, 3 = success | OFDMA |
| Ranging code attributes | TBA.5 | 4 | Bits 31:22 – Used to indicate the OFDM time symbol reference that was used to transmit the ranging code. Bits 21:16 – Used to indicate the OFDMA subchannel reference that was used to transmit the ranging code. Bits 15:8 – Used to indicate the ranging code index that was sent by the SS. Bits 7:0 – The 8 least significant bits of the frame number of the OFDMA frame where the SS sent the ranging code. | OFDMA |
| Channel Measurement Information | TBA.6 | TBA | TBD |  |
| MS Basic CID | TBA.7 | 2 | MS Basic CID | OFDMA |

11.6 RNG-RSP message encodings

[Remove TLVs in the following table as indicated:]

Table 616—RNG-RSP message encodings

| Name | Type (1 byte) | Length | Value (variable-length) | PHY Scope |
| --- | --- | --- | --- | --- |
| New MS Indication ID | TBA | 1 | Unique identifier assigned by RS for each MS under ranging process |  |