| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on local RS |  |
| Date Submitted | 2007-11-11 |  |
| Source(s) | Aik Chindapol Siemens Corporate Research Jung Je Son Samsung Electronics Co., Ltd. Yuan-Ying Hsu Telcordia Applied Research Center Kenji Saito KDDI Corporation | Voice: +1 609 734 3364 Email: aik.chindapol@siemens.com Voice: +82 31 279 5098 Email: jungje.son@samsung.com Voice: +886 2 37895177#4558 Email: yyhsu@tarc-tw.research.telcordia.com Voice: +81 3 6678 2980 Email: kenji@kddi.com |
| Re: | IEEE 802.16j-07/043: “IEEE 802.16 Working Group Working Group Letter Ballot #28” |  |
| Abstract | This contribution clarifies on local RS. |  |
| Purpose | Text proposal for 802.16j Draft Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on local RS

# Introduction

Localized RS and systematical CID assignment scheme has been unified to local CID allocation mode.

The some paragraph has been modified to implement the above change and clarify.

This contribution implemented the whole proposed remedy of comment 664, 665 and 1047 and last proposed remedy of #339.

# Proposed Remedy :

Change section 6.3.1.3.1 as follows :

6.3.1.3.1 Addressing scheme for relaying

In the procedure of network entry and initialization for a new RS, the MR-BS may pre-allocate a range of basic and primary management CIDs to an RS. The operation for pre-allocation of these CIDs is described in 6.3.9.16.4 .

Change section 6.3.2.3.65 as follows :

6.3.2.3.65 RS CID Allocation Request (CID_ALLOC-REQ) message

The CID_ALLOC-REQ message shall be used to assign a range of CIDS to the RS in the embedded path management scheme (Sec 6.3.25.1). The range of CID shall be allocated according to the procedure given in Sec 6.3.25.1. The CID_ALLOC-REQ message may be transmitted by the MR-BS / superordinate RS to an RS during network entry/re-entry processes. When the network topology is changed or CID (re-)allocation is required, the MR-BS may also transmit this message to related RSs to update CIDs. Upon receiving CID_ALLOC-REQ, the RS shall (re-)configure CID allocation accordingly and use the generic ACK message as defined in 6.3.2.3.86 to acknowledge that it received information about the CID allocation. The message format is shown in Table 183d .

| Syntax | Size | Note |
| --- | --- | --- |
| CID_ALLOC-REQ_Message_Format() { |  |  |
| Management Message Type = 70 | 8 bits |  |
| Transaction ID | 16 bit |  |
| CID_Alloc_method | 3 bits | 0= contiguous method 1= bit partition method 2~7=reserved |
| CID_type | 3 bits | 0 : basic CID 1 : primary management CID 2 : Others 3-7 : reserved |
| If (CID_Alloc_method = =0) { |  |  |
| Start number of CID | 16 bits |  |
| End number of CID | 16 bits |  |
| } |  |  |
| if (CID_Alloc_method ==1) { |  |  |
| New CID for the RS | 16 bits |  |
| Hop count | 8 bits |  |
| K_Code | 8 bits | The maximum number of direct subordinate RSs is 2 to the power of K_Code (2^K_Code). |
| } |  |  |
| } |  |  |

The following parameter shall be included in the message:

Transaction ID

Unique identifier set by the sender for identifying this transaction.

The CID_ALLOC-REQ shall contain the following TLVs:

HMAC/CMAC Tuple (see 11.1.2)

The HMAC/CMAC Tuple attribute contains a keyed message digest (to authenticate the sender).

The HMAC Tuple attribute shall be the final attribute in the CID_ALLOC-REQ messages attribute list.

Change section 6.3.9.16.4 as follows :

6.3.9.16.4 Optional network entry procedure for RS operation in the local CID allocation mode

Insert new subclause 6.3.9.16.4.1:

6.3.9.16.4.1 Local CID pre-allocation

The MR-BS may allocate the management CID range to a subordinate RS operating in the local CID allocation mode, which allows an RS to assign the pre-allocated management CIDs to its subordinate stations.

The CID assignments shall be sent to the subordinate RS by using RS-Config-CMD messages.

When local CID pre-allocation with embedded path management scheme is used, the pre-allocated management CIDs shall be decided based on contiguous integer block or bit partitioning methods as shown in sec 6.3.25.1. and the CID assignment shall be sent to the subordinate RS by using CID_ALLOC-REQ.

Insert new subclause 6.3.9.16.4.2:

6.3.9.16.4.2 MS network entry procedure for RS operation

This MS network entry process is the same as described in 6.3.9.16.2.1, except that RS operating in local CID allocation mode is assigned range of management CIDs by its super-ordinate node in advance. This section states that the RS assigns the management CIDs to its subordinate nodes (MS or RS) during initial ranging process.

When the time and power correction is finished between RS and MS, and the RS receives the RNG-REQ containing MS MAC address, the RS may reply the RNG-RSP containing the management CID that is assigned by the RS. In addition, the RS may inform the MR-BS that a new station (MS or RS) is ready to enter to the network using STA-INFO message.

After assigning the basic and primary management CID to an MS, the MS and MR-BS continue network entry process as described in the 6.3.9.7 through 6.3.9.13 using MS’s management CIDs. The RS shall relay management messages between them.

Change section 6.3.25.1 as follows :

6.3.25.1 Embedded path management for relay

When the embedded path management is used, the MR-BS or RS shall systematically assign CIDs to its subordinate stations such that the CIDs allocated to all descendant RSs of any given parent node is a subset of the allocated CIDs for that parent node. The network topology is embedded into a systematic CID structure to help RSs to find routing paths without storing all CIDs of subordinate RSs in the routing table.

CIDs are assigned systematically using either contiguous integer block or bit partitioning methods. For the contiguous integer block assignment, the CIDs assigned to subordinate stations are a contiguous range of CIDs, and are defined by the lower and upper values of this range. In the bit partition assignment, the MR-BS sets the lowest k bits in ascending order to RSs for RSs associated to the MR-BS directly where the maximum number of RSs the MR-BS or a RS could serve is 2 to the power k. For other level-n RSs, which need n hops to reach the MR-BS, the MR-BS left shifts k bits of its parent CID and sets the lowest k bits according to the arriving sequence of the RS. When the network topology changes, the MR-BS can inform related RSs of updated value of k and n in the RS-Config-CMD message. Figure 157 shows an example of both methods for systematic CID assignment.

Figure 157a

The MR-BS shall be responsible for managing the entire CID allocations within the MR-cell. By informing the range of systematic CID allocation to the RSs, the MR-BS already specifies the relay routing path of the connection based on assigned CIDs and is not required to provide end-to-end signaling. With CID information contained in MAP-IE or MAC header, RS can perform data forwarding to its subordinate RS.

To accommodate temporary topology changes due to mobility or path update, CID encapsulation may be required to route a packet that does not correspond to the routing path implied by the systematic CID assignment. If CID encapsulation is not required, then the packet is transmitted and routed via the embedded path information contained in the systematic CID assignment. If CID encapsulation is required, the initial packet is taken as payload, and another header is prefixed (i.e., the R-MAC header ). Packets are relayed depending on the CID in the outermost tunnel header. Once the packet arrives at the egress of the tunnel, the station at the egress removes the tunnel header and relays the payload, which may itself contain another tunnel

header.

Change section 11.7.28 as following :

11.7.28 MR MAC feature support

This TLV indicates the MR features supported by the RS and the MR-BS.

Bit #0: RS scheduling support

Bit #1: NBR-ADV generating

Bit #2: Tunneling packet mode support

Bit #3: Tunneling burst mode support

Bit #4: RS mobility support

Bit #5: Subordinate RS network entry support

Bit #6: local CID pre-allocation mode

Bit #7: embedded path management support