| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC management message summary table update |  |
| Date Submitted | 2007-11-12 |  |
| Source(s) | Mike Hart UK Broadband 1 Furzeground Way Stockley Park East Uxbridge, UK UB11 1BD | Voice: +44 20 8622 3999 E-mail: mike.hart@ukbroadband.com |
| Re: | Letter ballot #28 |  |
| Abstract | This contribution proposes an update to Table 38 in order to satisfy the following comments submitted in LB#28 on P802.16j/D1: #208, #210, #213, #214, #215, #216, #217. |  |
| Purpose | Accept the proposed changes |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC management message summary table update

Mike Hart

UK Broadband

Introduction

This contribution fills out the TBDs in Table 38 and also corrects some connection types for messages.

Corrections

- RS_MOBMEASR-RSP is sent to a group of RSs, so should be sent on multicast management connection.

- CLK-SYNC needs to be sent in the relay zone, so it is better to assign it to multicast management as there is no way to distinguish between the broadcast connection in the access zone and a broadcast connection in the relay zone, as the message will be sent using CID = 0.

Proposed changes

[Modify Table 38 as indicated:]

Table 38—MAC management messages

| Type | Message name | Message description | Connection |
| --- | --- | --- | --- |
| 74 | MR_LOC-REQ | MR location information request | Primary management |
| 75 | MR_LOC-RSP | MR location information response | Primary management |
| 76 | MS_SCN-INF | MS scanning inform | Basic |
| 78 | MS_INFO-DEL | MS context information delete | Basic |
| 80 | CLK-SYNC | RS clock synchronization | Multicast management |
| 81 | MR_ASC-REQ | MR association request | Basic |
| 82 | MR_ASC-RSP | MR association report | Basic |
| 83 | MOB_RSSCN-REP RS_MOBMEASR-REP | RS group measurement report | Basic |
| 84 | MOB_RSSCN-RSP RS_MOBMEASR-RSP | RS group measurement response | BasicMulticast management |
| 85 | HARQ_Chase_ER-REP | Chase HARQ error report | Basic |
| 86 | HARQ_IR_ER-REP | IR HARQ error report | Basic |
| 88 | STA-INFO | Station information | Primary management |
| 91 | RS_Member_List_Update | RS group member list update | Multicast management or Basic |

[Modify 6.3.2.3.74 as indicated:]

6.3.2.3.74 RS clock synchronization (CLK-SYNC) message

In MR network systems that require the MR-BS and non-transparent RSs to transmit frame-start DL preamble synchronously, CLK-SYNC message may be broadcasted on the relay link by the MR-BS using a multicast management connection.