| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Maximum Hop Count for Distributed Scheduling |  |
| Date Submitted | 2007-12-25 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee, Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw |
| Re: | IEEE 802.16j-07/059: “IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement” |  |
| Abstract | This contribution proposes the maximum hop count limitation for non-transparent RS with unique BSID operated in distributed scheduling. |  |
| Purpose | Text proposal for 802.16j Baseline Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Maximum Hop Count for Distributed Scheduling

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry (III)

Introduction

The maximum hop count of the MR system is restricted by the Round-Trip Time (RTT) that must ensure that the SS will receive “response” after sending “request” before the timer expired (see Table 1). In P802.16j/D2, the maximum hop count of the non-transparent RS with unique BSID operated in distributed scheduling is not addresses. This contribution proposes text changes and the table of maximum hop count based on the analysis of the RTT for the non-transparent RS with unique BSID operated in distributed scheduling mode.

The Table 1 illustrated related global timers for various procedures.

Table 1 Global Timer

| System | Name | Procedure | Minimum (ms) | Default (ms) | Maximum (ms) |
| --- | --- | --- | --- | --- | --- |
| SS | T3 | RNG-RSP after CDMA ranging | - | 60 | 200 |
|  |  | RNG-RSP after initial RNG-REQ |  | 60 |  |
|  |  | Negotiated HO |  | 50 |  |
|  |  | Non-negotiated HO |  | 200 |  |
|  |  | Location update |  | 200 |  |
|  |  | Re-entry from idle mode |  | 200 |  |
| SS | - | RNG-RSP Processing Time | - | - | 2.5 ms + 1 frame |
| BS | T9 | SBC-REQ after RNG-RSP | 300 | 300 | - |
| SS | T18 | SBC-RSP after SBC-REQ | - | 50 | << T9 |
| SS | T17 | REG-REQ after SBC-RSP (Time allowed for SS to complete SS Authorization and Key Exchange) | 300,000 (5min) | 300,000 (5min) | - |
| SS | T6 | REG-RSP after REG-REQ | - | - | 3000 (3s) |
| BS | T13 | TFTP-CPLT after REG-RSP | 900,000 (15min) | 900,000 (15min) | - |
| SS | T26 | TFTP-RSP after TFTP-CPLT | 10 | 200 | 200 |
| SS, BS | T7 | DSx-RSP after DSx-REQ (x =A/C/D) | - | - | 1000 (1s) |
| SS, BS | T8 | DSx-ACK after DSx-RSP (x =A/C) | - | - | 300 |
| SS | T14 | DSx-RVD after DSx-REQ | - | - | 200 |

Figure 1, Figure 2, and Figure 3 below illustrate the analysis of the RNG-REQ for negotiated handover using different bandwidth allocation methods, showing message sequences and correspondent latencies.

Figure 1 Using regular bandwidth request method ( is the frame duration)

Figure 2 Using bandwidth pre-request method ( is the frame duration)

Figure 3 Latencies during RNG-REQ/RSP ( is the frame duration)

The analysis summary by using “normal contention-based”, “pre-request contention-based” and “polling” are shown in Table 2, 3 and 4, respectively.

Table 2 Latencies during network entry (normal BW request)

| Hop count Phase | Frame duration  (ms) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 |
| CDMA Ranging (Regular) |  |  |  |  |
| Initial RNG-REQ/RSP (Local CID allocation) |  |  |  |  |
| Negotiated Handover (Non-local CID allocation) | 2 | 1 | 1 | 1 |
| SBC-REQ/RSP | 2 | 1 | 1 | 1 |
| Hop count limitation (Local CID allocation) | 2 | 1 | 1 | 1 |
| Initial RNG-REQ/RSP (Non-local CID allocation) | 2 | 1 | 1 | 1 |
| Hop count limitation (Non-local CID allocation) | 2 | 1 | 1 | 1 |
| CDMA Ranging (Optional) | 3 | 2 | 1 | 1 |
| Hop count limitation (Optional CDMA ranging) | 2 | 1 | 1 | 1 |

Table 3 Latencies during network entry (BW pre-requested)

| Hop count Phase | Frame duration  (ms) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 |
| CDMA Ranging (Regular) |  |  |  |  |
| Initial RNG-REQ/RSP (Local CID allocation) |  |  |  |  |
| Negotiated Handover (Non-local CID allocation) | 4 | 2 | 1 | 1 |
| SBC-REQ/RSP | 5 | 3 | 2 | 1 |
| Hop count limitation (Local CID allocation) | 4 | 2 | 1 | 1 |
| Initial RNG-REQ/RSP (Non-local CID allocation) | 5 | 2 | 1 | 1 |
| Hop count limitation (Non-local CID allocation) | 4 | 2 | 1 | 1 |
| CDMA Ranging (Optional) | 5 | 2 | 1 | 1 |
| Hop count limitation (Optional CDMA ranging) | 4 | 2 | 1 | 1 |

Table 4 Latencies during network entry (polling)

| Hop count Phase | Frame duration  (ms) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 |
| CDMA Ranging (Regular) |  |  |  |  |
| Initial RNG-REQ/RSP (Local CID allocation) |  |  |  |  |
| Negotiated Handover (Non-local CID allocation) | 5 | 3 | 2 | 1 |
| SBC-REQ/RSP | 5 | 3 | 2 | 1 |
| Hop count limitation (Local CID allocation) | 5 | 3 | 2 | 1 |
| Initial RNG-REQ/RSP (Non-local CID allocation) | 6 | 3 | 2 | 1 |
| Hop count limitation (Non-local CID allocation) | 5 | 3 | 2 | 1 |
| CDMA Ranging (Optional) | 6 | 3 | 2 | 1 |
| Hop count limitation (Optional CDMA ranging) | 5 | 3 | 2 | 1 |

In summary, in order to reduce the latency of relaying traffic, the MR-BS must “pre-request” pre-schedule proper UL bandwidth “via polling” in relay link for sending BR header to the MR-BS after allocating Ranging channel in the RS access link.

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft document P802.16j/D2 are listed below.

Text Proposal

6.3.6.7.1.1 Bandwidth request handling and transmission

[Modified the following text in line 57 of page 82 as indicated:]

The RS may should transmit a BW request header soon after it receives a BW request header from one of its subordinate stations (timed to yield an uplink allocation sequential to the arrival of those packets) instead of waiting for the actual packets to arrive in order to reduce delay in relaying traffic (see Figure 60a).

6.3.6.7.1.1.1 Contention-based CDMA bandwidth requests

[Modified the following text in line 47 of page 83 as indicated:]

The RS reduce the latency of relaying traffic by sending a bandwidth request CDMA ranging code as soon as it receives one from a downstream station instead of waiting for the actual packets to arrive (see Figure 60b).

The RS should may reduce the latency of relaying traffic by sending a bandwidth request CDMA ranging code as soon as it receives one from a downstream station instead of waiting for the actual packets to arrive (see Figure 60b).

[Insert the following subclause as indicated:]

Annex N

(informative)

Hop count limitation

N.1 Hop count limitation for the MR system

Table xxx Hop count limitation for the MR system operated in distributed scheduling mode

| Hop count Phase | Frame duration (ms) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 |
| Regular BW request | 2 | 1 | 1 | 1 |
| BW Pre-request | 4 | 2 | 1 | 1 |
| polling | 5 | 3 | 2 | 1 |